3GPP TSG-SA WG6 Meeting #62	S6-243192

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0310 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LMS reuse the stored UE location information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the TR 23.700-72 v1.1.1, the KI#3 for Location QoS improvement has been concluded as follows: Solution #1 and Solution#6 addressing the KI#3 can be considered in the normative work. The related function and procedure will be specified in 3GPP TS 23.434 [9] to support both of solutions, and the detailed APIs and information flows can be discussed in the normative phase. So it’s needed to specify the functions, procedures and information flows for such KI in TR 23.434 for the normative work. This CR focuses on how to speficy the function for Sol#6 in TS 23.434. The Sol#6 proposes the LMS can reuse the stored UE location information to report to the VAL server to reduce the response time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add function of LMS reusing the stored UE location data to report to the VAL server to reduce the response time in clause 9.3.4,9.3.5,9.3.8,9.3.9 respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the solution can’t be performed, the LCS QoS requested via the application server will not be met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.4,9.3.5,9.3.8,9.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.4	On-demand location reporting procedure

The location management server can request UE location information at any time by sending a location information request to the location management client, which may trigger location management client to immediately send the location report.

Figure 9.3.4-1: On-demand location information reporting procedure

- Based on configurations such as periodical location information timer, or location information request from other entities (e.g., another location management client, VAL server), location management server initiates the immediately request location information from the location management client.

NOTE:	If the LM server identified there is stored and valid UE location data for the target UE that requested by the VAL server or LM Client internally, it shall reuse the stored UE location information and report it to the VAL server or LM Client to reduce the response time. The step 2~5 will be skipped.

2.	The location management server sends a location information request to the location management client.

3.	VAL user or VAL UE is notified and asked about the permission to share its location. VAL user can accept or deny the request

4.	The location management client immediately responds to the location management server with a report containing location information identified by the location management server and available to the location management client.

5.	Upon receiving the report, the location management server updates location of the reporting location management client. If the location management server does not have location information of the reporting location management client before, then just stores the reporting location information for that location management client.

* * * Next Change * * * *

9.3.5	Client-triggered or VAL server-triggered location reporting procedure

Figure 9.3.5-1 illustrates the high level procedure of client-triggered or VAL server-triggered location reporting.

Figure 9.3.5-1: Client-triggered location reporting procedure

1.	Location management client 2 (authorized VAL user or VAL UE) or VAL server sends a location reporting trigger to the location management server to activate a location reporting procedure for obtaining the location information of location management client 1. The location reporting event triggers e.g. minimum time between consecutive reports, SAI changes, access RAT changes, or ECGI changes for reporting the location of the VAL UE, are included.

NOTE:	Step 1 can be performed when Location management client 2 or VAL server require to update the location reporting trigger corresponding to location management client 1.

- Location management server checks whether location management client 2 or VAL server is authorized to send a location reporting trigger. Depending on the information specified by the location reporting trigger, location management server initiates an on-demand location reporting procedure or an event-triggered location reporting procedure for the location of location management client 1.

NOTE:	If the LM server identified there is stored and valid UE location data for the target UE that requested by the VAL server or LM Client internally, it shall reuse the stored UE location information and report it to the VAL server or LM Client to reduce the response time. The on-demand location reporting procedure or an event-triggered location reporting procedure will be skipped.

3.	Once the location information of the location management client 1 is available in the location management server by the on-demand location reporting procedure, a location information report is sent to the location management client 2 or VAL server.

* * * Next Change * * * *

9.3.8	Event-trigger location information notification procedure

Figure 9.3.8-1 illustrates the high level procedure of event-trigger usage of location information. The same procedure can be applied for location management client and other entities that would like to subscribe to location information of VAL user or VAL UE. This procedure is also used for obtaining latest UE's location for tracking purpose.

Figure 9.3.8-1: Event-trigger usage of location information procedure

1.	The location management server receives the latest location information of the UE as per the location report procedure described in clause 9.3.3.3.

2.	The location management server may optionally receive the location information of the UE from 3GPP core and/or the 3rd party location management server network. If the indication for supplementary location information is included in the subscription, then UE location information is obtained from the 3GPP core network and/or the 3rd party location management server.

3.	Based on the configurations, e.g., subscription, periodical location information timer, location management server is triggered to report the latest user location information to VAL server. The location management server determines the location information of UE as received in steps 1 and 2, including the supplementary location information (if indicated). The Location management server may report the location to the VAL server considering the location information received via non-3GPP positioning technologies (e.g. GNSS, Bluetooth), for instance, to improve the location accuracy. The location management server may reuse the stored and valid UE location information (if any) to report to the VAL server/LM client to reduce the response time.

4.	The location management server sends the location information report including the latest location information of one or more VAL users or VAL UEs to the VAL server or to the location management client that has previously configured.

5.	VAL server may further share this location information to a group or to another VAL user or VAL UE.

NOTE:	For other entities, the step 5 can be skipped if not needed.

* * * Next Change * * * *

9.3.9	On-demand usage of location information procedure

The VAL server can request UE location information at any time by sending a location information request to the location management server, which may trigger location management server to immediately send the location report.

Figure 9.3.9-1 illustrates the high level procedure of on demand usage of location information. The same procedure can be applied for location management client and other entities that would like to subscribe to location information of VAL user or VAL UE.

Figure 9.3.9-1: On-demand usage of location information procedure

1.	VAL server sends a location information request to the location management server which may include the location QoS that contains the location accuracy, response Time and QoS class as defined in clause 4.1b of TS 23.273 [50].

2.	The location management server will check if there is stored and valid UE location data for the target UE that requested by the VAL server internally. If there is, it shall reuse the stored UE location information and report it to the VAL server to reduce the response time. If there isn’t, it will acquire the latest location of the UEs being requested, by triggering an on-demand location report procedure as described in subclause 9.3.4, or from PLMN operator, and checks if the obtained location of the VAL UE could meet the location QoS requirements (if any).

3.	Then, location management server immediately sends the location information report including the latest location information acquired of one or more VAL users or VAL UEs. The Location management server may report the location to the VAL server considering the location information received via non-3GPP positioning technologies (e.g. GNSS, Bluetooth), for instance, to improve the location accuracy.

4.	VAL server may further share this location information to a group or to another VAL user or VAL UE.

NOTE:	For other entities, the step 4 can be skipped if not needed.

* * * End of Changes * * * *