3GPP TSG-SA WG6 Meeting #62	S6-243198

# Maastricht, NL, 19 - 23 August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0037 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Collision Detection Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Solution#8 is recommended for normative work on support of Collision Detection Analytics for ranging/sidelink positioning information exposure. See more details in 3GPP TR 23.700-23. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added descriptions and procedures for support of Collision Detection Analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support of Collision Detection Analytics is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3, 6.A(new), 8.X(new), 8.X.1(new), 8.X.2(new), 8.X.2.1(new), 8.X.2.2(new), 8.X.3(new), 8.X.3.1(new), 8.X.3.2(new), 8.X.3.3(new), 8.X.3.4(new), 8.X.3.5(new), 8.X.3.6(new), 8.X.3.7(new), 8.X.3.8(new), 8.X.3.9(new), 9.2.2, 9.2.Y(new), 9.2.Y.1(new), 9.2.Y.2(new), 9.2.Y.3(new), 9.2.Y.4(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

### 5.4.3	Application Data Analytics Enablement server

The application data analytics enablement server functional entity provides application layer analytics to support the VAL applications. The application data analytics enablement server acts as CAPIF's API exposing function as specified in 3GPP TS 23.222 [8]. The application data analytics enablement server also supports interactions with the corresponding application data analytics enablement server in distributed SEAL deployments. The ADAE server also interacts with 3GPP core nework over N33 or N6 interface to subscribe to changes in configuration or other application server specific events. The ADAE server also acts as a co-ordinating entity to collect data from different sources and perform necessary actions to provide required analytics.

The ADAE server provides following server side functionalities:

-	monitoring performance of an application (VAL server or EAS, application session) and providing support for application performance analytics;

-	monitoring performance of a given network slice (from a list of subscribed slices for the VAL customer) and also usage pattern, and providing support for slice-specific application performance analytics and slice usage pattern analytics;

-	monitoring performance of an application session among two or more VAL UEs within a service or group, and providing support for UE-to-UE application performance analytics;

-	monitoring accuracy of a location and providing support for location accuracy analytics;

-	monitoring availability and service level for service APIs and providing support for service API analytics;

-	monitoring edge load parameters and providing support for edge load analytics;

-	monitoring ranging/SL positioning data and location information, and providing support for collision detection analytics.

* * * Next change * * * *

## 6.A	Support for collision detection analytics

This feature supports collision detection analytics to allow an analytics consumer (such as VAL Server, LM server, UAE server, UAS application specific server) to be notified on analytics for collision detection between any target VAL UEs, collision detection between any UEs and target VAL UEs, or collision detection between any UE within the Area of Interest.

* * * Next change * * * *

## 8.X 	Procedure for Collision Detection Analytics

### 8.X.1	General

This clause describes two procedures (covering both subscribe-notify and request-response models in 8.X.2.1 and 8.X.2.2 respectively) for supporting collision detection analytics, where the collision detection analytics are performed based on data collected from the Data Producer (e.g. 5GC NFs (e.g. GMLC, NWDAF), ADAE Client, LM server, LM client, 3rd party LM server) and A-ADRF.

### 8.X.2	Procedure

Pre-conditions:

-	Information about environment, e.g. static devices, are available from the 3rd party LM server.

-	The location information about the UEs is available at SEAL LM server and/or LM client.

-	The Ranging/Sidelink Positioning information exposure of the UEs is allowed at 5GC.

#### 8.X.2.1	Subscribe-notify model

Figure 8.X.2.1-1: ADAES support for collision detection analytics

1.	The analytics consumer (e.g. VAL Server, LM server, UAE server, UAS application specific server) sends analytics subscription request for collision detection analytics to ADAE server. The Analytics ID in the request message is set to "Collision detection analytics". For analytics subscription request, the request contains message as defined in table 8.X.3.2-1.

2.	Upon receiving the event subscription request from the consumer, the ADAE server checks for the relevant authorization for the event subscription. If the authorization is successful, the ADAE server stores the request information. The ADAE server sends a service API event subscription response indicating successful subscription.

3.	The ADAE server sends a subscription request to the Data Producer (e.g. 5GC NFs (e.g. GMLC, NWDAF), ADAE Client, LM server, LM client, 3rd party LM server, A-ADRF for historical data) with the respective Data Collection Event ID and the requirement for collection of ranging and/or sidelink positioning related data or analytics of UEs, and location information of UEs. Data collection at the UE(s) reuses mechanism defined in 3GPP TS 26.531 [3].

4.	The Data Producer sends subscription response as a positive or negative acknowledgement to the ADAE server.

5.	The ADAE server based on subscription receive ranging and/or sidelink positioning related data/analytics of Ues, and location information of UEs.

NOTE:	The procedures for UE related data collection need to take user consent into account.

6.	The ADAE server performs analytics relevant operations to generate the analytics for the collision detection between any target VAL UEs, collision detection between any UEs and target VAL UEs, or collision detection between any UE within the Area of Interest and based on the request provided in step 1 and data received from the Data Producer (e.g. 5GC NFs (e.g. GMLC, NWDAF), ADAE Client, LM server, LM client, 3rd party LM server, A-ADRF for historical data).

7.	The ADAE server sends notifications to the consumer with the required collision detection analytics.

#### 8.X.2.2	Request-response model

Pre-conditions:

-	ADAE Server already have the analytics data derived from steps 3-6 in the procedure introduced in clause 8.X.2.1.

Figure 8.X.2.2-1: ADAES support for collision detection analytics

1.	The analytics consumer (e.g. VAL Server, LM Server, UAE server, UAS application specific server) sends a request message to the ADAE server to receive analytics data for collision detection analytics with Analytics ID in the request message setting to "Collision detection analytics". The request contains message as defined in table 8.X.3.8-1.

2.	Upon receiving the request, the ADAE server authenticates and authorizes the analytics consumer.

3.	If the analytics consumer is authorized, the ADAE server sends a response message including the analytics data (statistical and/or predictive) of the collision detection analytics.

### 8.X.3	Information flows

#### 8.X.3.1	General

The following information flows are specified for collision detection analytics based on clause 8.X.2.

#### 8.X.3.2	Collision detection analytics subscription request

Table 8.X.3.2-1 describes the information flow from the consumer (e.g. VAL Server, LM Server, UAE server, UAS application specific server) as a request or update request for the collision detection analytics.

Table 8.X.3.2-1: Collision detection analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M (NOTE) | The identifier of the consumer. |
| Analytics ID | O (NOTE) | The identifier of the analytics event. This ID can be for example "Collision Detection analytics". |
| Analytics type | M | The type of analytics for the event, e.g. statistics or predictions, |
| Analytics filter information | M | Filter information for the analytics event |
| >Target UE(s) | O | The UE(s) for which the analytics subscription applies. |
| > Any UE(s) | O | The list of any UE(s) collision with that shall be analysed. |
| > Collision between any target UE indicator | O | Indicates the request for the collision detection analytics between any UE provided in the Target VAL UE(s) IE. |
| > Collision between any UE(s) and target UE(s) | O | Indicates the request for the collision detection analytics between any any UE and any target UE. |
| > Collision detection between any UE within the Area of Interest | O | Indicates the request for the collision detection analytics between any UEs within the Area of Interest. |
| >Target VAL server ID | O | If consumer is different from the VAL server, this identifier shows the target VAL server for which the analytics subscription applies. |
| >Area of Interest | O | The geographical or service area for which the subscription request applies |
| Attributes for collision detection | O | The parameters for collision detection analytics apply. |
| >Distance | O | The minimum distance allowed for UEs. The collision is detected if the distance between the UEs is less than the provided value. |
| >Velocity | O | The maximum velocity of the UEs. |
| >Direction | O | The expected direction of the UEs moving. |
| >Movement track | O | The parameters for Movement track (e.g., the list of geographical points) of UEs. |
| >Granularity | O | The granularity of the movement track. |
| Reporting requirements | O | It describes the requirements for analytics reporting. This requirement may include e.g. the type and frequency of reporting (periodic or event triggered), the reporting periodicity in case of periodic, and reporting thresholds. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Time validity | O | The time validity of the subscription request. |
| NOTE: This information element shall not be updated. |  |  |

#### 8.X.3.3	Collision detection analytics subscription response

Table 8.X.3.3-1 describes the information elements for the collision detection analytics subscription response from the ADAE server to the consumer.

Table 8.X.3.3-1: Collision detection analytics subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the analytics subscription request (positive or negative acknowledgement). |

#### 8.X.3.4	Collision detection analytics notification

Table 8.X.3.4-1 describes the information flow from the ADAES to the consumer (e.g. VAL Server, LM Server, UAE server, UAS application specific server) as a response for the collision detection analytics.

Table 8.X.3.4-1: Collision detection analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example "Collision Detection analytics". |
| Outputs | O | The reported analytics for the collision detection analytics. |
| >VAL UE IDs | M | The VAL UE identifiers for which the analytics outputs apply. |
| >>Time of potential collisions | M | Time of the potential collisions between the UEs. |
| >>Location of potential collisions | M | Location of the potential collisions between the UEs. |
| >>Direction of moving | O | Direction of the UEs moving. |
| >>Velocity of moving | O | Velocity of the UEs moving. |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |

#### 8.X.3.5	Ranging/SL positioning data and location information collection subscription request

Table 8.X.3.5-1 describes information elements for the Ranging/SL positioning data and location information collection subscription request from the ADAE server to the Data Producer, e.g. 5GC NFs (e.g. GMLC, NWDAF), ADAE Client, LM server, LM client, 3rd party LM server, A-ADRF for historical data.

Table 8.X.3.5-1: Data collection subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the consumer. |
| Data Collection Event ID | M | The identifier of the data collection event |
| Data Collection requirements | M | The requirements for data collection, including the format of data, frequency of reporting, level of abstraction of data, level of accuracy of data. |
| Analytics ID | O | The identifier of the analytics event, for which the data collection is needed. This ID can be for example "Collision Detection analytics". |
| List of Data Producer IDs | O | In case when this request is performed via A-DCCF, then the list of Data Producer IDs is needed. |
| >Target data producer profile criteria | O | Characteristics of the data producers to be used. |
| >VAL UE IDs | O | The VAL UE(s) identifiers for which the data/analytics apply. |
| Area of Interest | O | The geographical or service area for which the requirement request applies. |
| Time validity | O | The time validity of the request. |

#### 8.X.3.6	Ranging/SL positioning data and location information collection subscription response

Table 8.X.3.6-1 describes information elements for the Data collection subscription response from the Data Producer, e.g. 5GC NFs (e.g. GMLC, NWDAF), ADAE Client, LM server, LM client, 3rd party LM server, A-ADRF for historical data.

Table 8.X.3.6-1: Data collection subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the Ranging/SL positioning data and location information collection subscription request (positive or negative acknowledgement). |

#### 8.X.3.7	Data Notification

Table 8.X.3.7-1 describes information elements for the Data Notification from the Data Producer to the ADAE server.

Table 8.X.3.7-1: Data notification

| Information element | Status | Description |
| --- | --- | --- |
| Data Collection Event ID | M | The identifier of the data collection event. |
| Data Producer ID | M | The identity of Data Producer. |
| Analytics ID | O | The identifier of the analytics event. This ID can be for example "Collision Detection analytics". |
| Data Type | M | The type of reported data samples which can be network data, application data, edge data, or different granularities / abstraction of data (e.g. real time, non-real time). This also indicates whether data are offline (from A-ADRF or not). |
| Data Output | M | The reported data, which can be inform of measurements or offline/historical data on the requested parameter based on subscription. Such data can be ranging/sidelink positioning information of UEs, location information of UEs (moving devices like UAS devices, V2X devices, robots, and/or people with UE), and the location information about environment (static devices like infrastructures) for a given time and area of interest, Relative Proximity analytics. |

#### 8.X.3.8	Get analytics data request

Table 8.X.3.8-1 describes information elements for the collision detection analytics request from the analytics consumer to the ADAE server.

Table 8.X.3.8-1: Get analytics data request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the consumer. |
| Analytics ID | O | The identifier of the analytics event. This ID can be for example "Collision Detection analytics". |
| Analytics type | M | The type of analytics for the event, e.g. statistics or predictions, |
| Analytics filter information | M | Filter information for the analytics event |
| >Target VAL UE ID(s) | O | The VAL UE(s) for which the analytics subscription applies |
| >Target VAL server ID | O | If consumer is different from the VAL server, this identifier shows the target VAL server for which the analytics subscription applies. |
| >Area of Interest | O | The geographical or service area for which the subscription request applies |
| Attributes for collision detection | O | The parameters for collision detection analytics apply. |
| >Distance | O | The minimum distance allowed for two UEs. |
| >Velocity | O | The maximum velocity of the UEs. |
| >Direction | O | The expected direction of the UEs moving. |
| >Granularity | O | The granularity of the movement track. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |

#### 8.X.3.9	Get analytics data response

Table 8.X.3.9-1 describes information elements for the Get collision detection analytics response from the ADAE server to the consumer.

Table 8.X.3.9-1: Get analytics response

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | O | The identifier of the analytics event. This ID can be for example "Collision Detection analytics". |
| Outputs | O | The reported analytics for the collision detection analytics. |
| >VAL UE IDs | M | The VAL UE identifiers for which the analytics outputs apply. |
| >>Time of potential collisions | M | Time of the potential collisions between the UEs. |
| >>Location of potential collisions | M | Location of the potential collisions between the UEs. |
| >>Direction of moving | O | Direction of the UEs moving. |
| >>Velocity of moving | O | Velocity of the UEs moving. |
| >>Timestamp | O | Time stamp of the collected analytics data. |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |

* * * Next change * * * *

### 9.2.2	ADAE server APIs

Table 9.2.2-1 illustrates the ADAE server APIs.

Table 9.2.2-1: List of ADAE server APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_ADAE_VAL_performance_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_slice_performance_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_UE-to-UE_performance_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_server-to-server_performance_analytics | Subscribe | VAL server / EES | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_location_accuracy_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_service_API_analytics | Subscribe | VAL server / Subscriber/ API invoker | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_slice_usage_pattern_analytics | Subscribe | VAL server / SEAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_edge_analytics | Subscribe | VAL server / EAS / EES | Subscribe/Notify |
|  | Notify |  |  |
|  | Get | VAL server / EAS / EES | Request / Response |
| SS_ADAES_slice_usage_stats | Get | VAL server | Request / Response |
| SS_ADAES_edge_preparation_analytics | Subscribe | VAL server / ECS / EES | Subscribe/Notify |
|  | Notify |  |  |
|  | Get | VAL server / ECS / EES | Request / Response |
| SS_ADAE_collision_detection_analytics | Subscribe | VAL server / LM server / UAE server, UAS application specific server | Subscribe/Notify |
|  | Notify |  |  |
|  | Get | VAL server / LM server / UAE server, UAS application specific server | Request / Response |

* * * Next change * * * *

### 9.2.Y	SS_ADAE_collision_detection_analytics API

#### 9.2.Y.1	General

API description: This API enables the analytics consumer (e.g. VAL Server, LM Server, UAE server, UAS application specific server) to communicate with the ADAE server for requesting or subscribing to collision detection analytics.

#### 9.2.Y.2	Subscribe

API operation name: collision_detection_analytics_subscribe

Description: The consumer subscribes for collision detection analytics.

Inputs: See clause 8.X.3.2.

Outputs: See clause 8.X.3.3.

See clause 8.X.2.1 for details of usage of this operation.

#### 9.2.Y.3	Notify

API operation name: collision_detection_analytics_notify

Description: The consumer is notified by the ADAE server on collision detection analytics.

Inputs: -

Outputs: See clause 8.X.3.4.

See clause 8.X,2.1 for details of usage of this operation.

#### 9.2.Y.4	Get

API operation name: collision_detection_analytics_get

Description: The consumer requests collision detection analytics.

Inputs: -See clause 8.X.3.8

Outputs: See clause 8.X.3.9.

See clause 8.X.2.2 for details of usage of this operation.

* * * End of changes * * * *