3GPP TSG-SA WG6 Meeting #62	S6-243199

# Maastricht, NL, 19 - 23 August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0038 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Location-related UE Group Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Solution#9 is recommended for normative work on support of location-related UE group analytics, e.g., statistics/predictions on time duration of UE(s) staying at an area, UE group route, UE density, and UE group member deviation, to support location information exposure and value-added location services. See more details in 3GPP TR 23.700-23. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added descriptions and procedures for support of location-related UE group analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support of location-related UE group analytics is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.4.3, 6.A(new), 8.X(new), 8.X.1(new), 8.X.2(new), 8.X.2.1(new), 8.X.2.2(new), 8.X.3(new), 8.X.3.1(new), 8.X.3.2(new), 8.X.3.3(new), 8.X.3.4(new), 8.X.3.5(new), 8.X.3.6(new), 8.X.3.7(new), 8.X.3.8(new), 8.X.3.9(new), 9.2.2, 9.2.Y(new), 9.2.Y.1(new), 9.2.Y.2(new), 9.2.Y.3(new), 9.2.Y.4(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture"

[4]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[5]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[6]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[7]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[8]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[9]	3GPP TS 23.501: “System architecture for the 5G System”.

[10]	GSMA NG.116 - Generic Network Slice Template.

[11]	3GPP TS 22.261: “Service requirements for the 5G system”.

[12]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[13]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[14]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[15]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[16]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[17]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2";

[XX]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

* * * Next change * * * *

### 5.4.3	Application Data Analytics Enablement server

The application data analytics enablement server functional entity provides application layer analytics to support the VAL applications. The application data analytics enablement server acts as CAPIF's API exposing function as specified in 3GPP TS 23.222 [8]. The application data analytics enablement server also supports interactions with the corresponding application data analytics enablement server in distributed SEAL deployments. The ADAE server also interacts with 3GPP core nework over N33 or N6 interface to subscribe to changes in configuration or other application server specific events. The ADAE server also acts as a co-ordinating entity to collect data from different sources and perform necessary actions to provide required analytics.

The ADAE server provides following server side functionalities:

-	monitoring performance of an application (VAL server or EAS, application session) and providing support for application performance analytics;

-	monitoring performance of a given network slice (from a list of subscribed slices for the VAL customer) and also usage pattern, and providing support for slice-specific application performance analytics and slice usage pattern analytics;

-	monitoring performance of an application session among two or more VAL UEs within a service or group, and providing support for UE-to-UE application performance analytics;

-	monitoring accuracy of a location and providing support for location accuracy analytics;

-	monitoring availability and service level for service APIs and providing support for service API analytics;

-	monitoring edge load parameters and providing support for edge load analytics;

-	monitoring location information and providing support for location-related UE group analytics.

* * * Next change * * * *

## 6.A	Support for location-related UE group analytics

This feature supports location-related UE group analytics to allow an analytics consumer (such as LMS) to be notified on analytics for UE(s) staying time duration, UE group route, UE density, or UE group member deviation. Such analytics can be used, e.g. time duration prediction for UE staying in an area can be used for VAL to get estimation on how long a person can stay in the office, UE density prediction in an area can be used for VAL to get hot zone information due to people gathering, UE group route prediction can be used to formulate application group profile with Expected Group Geographical Service Area as described in 3GPP TS 23.558 [14] clause 8.2.11. UE group member deviation prediction can be used for VAL to know which UE group member falls behind other group members or group leader (then VAL can send warning/reminder to the group members).

* * * Next change * * * *

## 8.X 	Procedure for Location-related UE Group Analytics

### 8.X.1	General

This clause describes two procedures (covering both subscribe-notify and request-response models in 8.X.2.1 and 8.X.2.2 respectively) for supporting location-related UE group analytics, where the location-related UE group analytics are performed based on data collected from the Data Producer (e.g. 5GC NFs (e.g. GMLC (via NEF), NWDAF), ADAE Client, LM server) and A-ADRF.

### 8.X.2	Procedure

Pre-conditions:

-	The location information about the UEs is available at SEAL LM server.

-	The location information of the UEs exposure is allowed at 5GC.

#### 8.X.2.1	Subscribe-notify model

Figure 8.X.2.1-1: ADAES support for location-related UE group analytics

1.	The analytics consumer (i.e. LMS) sends analytics subscription request for location-related UE group analytics to ADAE server. The Analytics ID in the request message is set to "UE(s) staying time duration analytics", "UE group route analytics", "UE density analytics", or "UE group member deviation". For analytics subscription request, the request contains message as defined in table 8.X.3.2-1.

2.	Upon receiving the event subscription request from the consumer, the ADAE server checks for the relevant authorization for the event subscription. If the authorization is successful, the ADAE server stores the request information. The ADAE server sends a service API event subscription response indicating successful subscription.

3.	The ADAE server sends a subscription request to the Data Producer (e.g. 5GC NFs (e.g. GMLC (via NEF), NWDAF), ADAE Client, LM server, A-ADRF for historical data) with the respective Data Collection Event ID and the requirement for collection location related data/analytics of UEs. Data collection at the UE(s) reuses the mechanism defined in 3GPP TS 26.531 [3].

4.	The Data Producer sends subscription response as a positive or negative acknowledgement to the ADAE server.

5.	The ADAE server based on subscription receive location related data/analytics of UEs from the Data Producer.

NOTE:	The procedures for UE location information collection need to take user consent into account.

6.	The ADAE server performs analytics relevant operations to generate the analytics based on the data received from the Data Producer (e.g. 5GC NFs (e.g. GMLC, NWDAF), ADAE Client, LM server, A-ADRF for historical data). In detail, the ADAE server may generate analytics in the following ways.

-	For UE(s) staying time duration analytics: Inputs include the location information of UE(s) at an area from LM server (e.g. measurement of UE(s) moving in or moving out of the area of interest in clause 9.3.12 of 3GPP TS 23.434 [2]) and the analytics from NWDAF (e.g. UE mobility analytics and/or Movement behaviour analytics in 3GPP TS 23.288 [4]). For example, measurement of UE(s) moving in or moving out of the area of interest can be used by the ADAES to get the information whether UE(s) is in the AoI, and UE mobility information can be used by the ADAES to obtain potential moving direction of the UE, etc., by combining all the information collected, predictions on UE(s) staying time duration can be generated at ADAES.

-	For UE group route analytics: Inputs include the location information of UE(s) from LM server (e.g. UE's location in clause 9.3.7 of 3GPP TS 23.434 [2]), the analytics from NWDAF (e.g. UE mobility analytics, Movement behaviour analytics, UE communication analytics, and/or Dispersion analytics in 3GPP TS 23.288 [4]). For example, UE's location service from LMS allows ADAES to obtain the UE location, UE mobility analytics provides potential moving direction of the UE(s), Dispersion analytics gives ADAES information on UE data volume dispersion predictions, etc., by combining all the information collected, predictions on UE group route can be generated.

-	For UE density analytics: Inputs include the number of UE(s) in an area from LM server (e.g. obtaining UE(s) information at the area of interest in clause 9.3.10 of 3GPP TS 23.434 [2], measurement of UE(s) moving in or moving out of the area of interest in clause 9.3.12 of 3GPP TS 23.434 [2]), analytics from NWDAF (e.g. UE mobility analytics, and/or Movement behaviour analytics in 3GPP TS 23.288 [4]). For example, ADAES get the UE information via the obtaining UE(s) information at the area of interest service from LMS,  ADAES obtains the moving direction predictions on these UE(s), etc.,  by combining all the information collected, predictions on UE density can be generated.

-	For UE group member deviation analytics: Inputs include the Ranging/Sidelink Positioning location information of the UE group member from GMLC (via NEF) (e.g. Ranging/Sidelink Positioning location results in clause 6.20 of 3GPP TS 23.273 [XX]), location information of the UE group member from LM server (e.g. UE's location in clause 9.3.7 of 3GPP TS 23.434 [2]), and the analytics from NWDAF (e.g. UE mobility analytics, Movement behaviour analytics, Relative Proximity analytics, and/or Abnormal behaviour analytics in 3GPP TS 23.288 [4]) of this UE group member. The ADAE server can aggregate the collected information (e.g. distance between UEs from the Ranging/Sidelink Positioning location information service, predictions on distance between UEs from the Relative Proximity analytics, etc.) and derive which UE group member is deviating (or will deviate) from other group member(s) or group leader (e.g. UE's ranging information with any other UE(s) in a group is larger than a threshold, and such trend shows longer and longer ranging within a time period, or the UE moving direction is different from any other UE(s) in a group or group leader).

7.	The ADAE server sends notifications to the consumer with the required location-related UE group analytics.

#### 8.X.2.2	Request-response model

Pre-conditions:

-	ADAE Server already has the analytics data derived from steps 3-6 in the procedure introduced in clause 8.X.2.1.

Figure 8.X.2.2-1: ADAES support for location-related UE group analytics

1.	The analytics consumer (i.e. LMS) sends a request message to the ADAE server to receive analytics data for location-related UE group analytics with Analytics ID in the request message setting to "UE(s) staying time duration analytics", "UE group route analytics", "UE density analytics", or "UE group member deviation". The request contains message as defined in table 8.X.3.8-1.

2.	Upon receiving the request, the ADAE server authenticates and authorizes the analytics consumer.

3.	If the analytics consumer is authorized, the ADAE server sends a response message including the analytics data (statistical and/or predictive) of the location-related UE group analytics.

### 8.X.3	Information flows

#### 8.X.3.1	General

The following information flows are specified for location-related UE group analytics based on clause 8.X.2.

#### 8.X.3.2	Location-related UE group analytics subscription request

Table 8.X.3.2-1 describes the information flow from the consumer (LMS) as a request or update request for the location-related UE group analytics.

Table 8.X.3.2-1: Location-related UE group analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M (NOTE) | The identifier of the consumer. |
| Analytics ID | O (NOTE) | The identifier of the analytics event. This ID can be for example "UE(s) staying time duration analytics", "UE group route analytics", "UE density analytics", or "UE group member deviation analytics". |
| Analytics type | M | The type of analytics for the event, e.g. statistics or predictions, |
| Analytics filter information | M | Filter information for the analytics event. |
| >Application Group ID | O | Application group identifier for which the analytics subscription applies. |
| >Target VAL UE ID(s) | O | The identifier(s) of the VAL UE(s) for which the analytics subscription applies. |
| >Area of Interest | O | The geographical or service area for which the subscription request applies. |
| >Application Group profile(s) | O | Information about the Application Group(s) with common EAS (as defined in 3GPP TS 23.558 [14] Table 8.X.11-1). |
| >Deviation | O | Deviation information, e.g. different moving direction or moving speed, distance with the group leader UE larger than an threshold. |
| >>Group leader UE ID | O | The identifier of the group leader UE. |
| >>Guideline | O | The guideline used for detecting deviation of group member UE, e.g. distance, moving direction, moving speed. |
| >>Threshold | O | The threshold used for detecting deviation of group member UE from the other group members or group leader with the guideline. |
| Reporting requirements | O | It describes the requirements for analytics reporting. This requirement may include e.g. the type and frequency of reporting (periodic or event triggered), the reporting periodicity in case of periodic, and reporting thresholds. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Time validity | O | The time validity of the subscription request. |
| NOTE: This information element shall not be updated. |  |  |

#### 8.X.3.3	Location-related UE group analytics subscription response

Table 8.X.3.3-1 describes the information elements for the location-related UE group analytics subscription response from the ADAE server to the consumer.

Table 8.X.3.3-1: Location-related UE group analytics subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the analytics subscription request (positive or negative acknowledgement). |

#### 8.X.3.4	Location-related UE group analytics notification

Table 8.X.3.4-1 describes the information flow from the ADAES to the consumer (LM Server) as a response for the location-related UE group analytics.

Table 8.X.3.4-1: Location-related UE group analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example "UE(s) staying time duration analytics", "UE group route analytics", "UE density analytics", or "UE group member deviation analytics". |
| Outputs for UE(s) staying time duration | O (NOTE 1) | The reported analytics for UE(s) staying time duration analytics apply. |
| >List of VAL UE ID(s) | M | List of identifiers of the VAL UE(s). |
| >>VAL UE ID | M | Identifier of VAL UE. |
| >>Location | M | Location information where the UE staying in an area. |
| >>Time duration | M | Time duration of the UE staying in the location. |
| >>Times UE leaves area | O | The number of times UE leaves the area. |
| >>Times UE enters area | O | The number of times UE enters the area. |
| Outputs for UE group route | O (NOTE 1) | The reported analytics for UE group route analytics apply. |
| >List of EAS ID(s) | M | List of identifiers of EAS. |
| >>EAS ID(s) | O | Identifier(s) of EAS(s). |
| >>Route ID | M | The identifier of the common route for the UE(s) to EAS(s). |
| >>VAL UE ID(s) | M | The identifiers of the VAL UE(s) which with the common route to the EAS(s). |
| Outputs for UE density | O (NOTE 1, NOTE 2) | The reported analytics for UE density analytics apply. |
| >Area of Interest | M | The geographical or service area. |
| >Number of UE(s) | O | The number of UE(s) in the AoI. |
| >Density of UE | M | The density of UE in the AoI. |
| Outputs for UE group member deviation | O (NOTE 1, NOTE 3) | The reported analytics for group member deviation analytics apply. |
| >Application Group ID | M | Application group identifier. |
| >>Group leader UE ID | O | The identifier of the group leader UE. |
| >List of VAL UE ID(s) | M | The VAL UE(s) which with different behaviour with the majority group members or the group leader. |
| >>VAL UE ID | O | Identifier of the VAL UE. |
| >>Deviation | O | Detail information of deviation for the VAL UE, e.g., different moving direction or moving speed, distance with the group leader. |
| >>Time duration | O | The time duration of this UE deviation. |
| Validity Time | O | The valid time duration of the analytics. |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |
| NOTE 1: One of these shall be present based on the analytics event. NOTE 2: Statistics on UE density including e.g. distribution, minimum, maximum, and average density during the validity time. NOTE 3: Statistics on UE deviation including e.g. minimum, maximum, and average UEs moving into area, moving out of area, moving speed, and distance from group leader. |  |  |

#### 8.X.3.5	Location information collection subscription request

Table 8.X.3.5-1 describes information elements for the Ranging/SL positioning data and location information collection subscription request from the ADAE server to the Data Producer, e.g. 5GC NFs (e.g. GMLC, NWDAF), ADAE Client, LM server, A-ADRF for historical data.

Table 8.X.3.5-1: Data collection subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the consumer. |
| Data Collection Event ID | M | The identifier of the data collection event |
| Data Collection requirements | M | The requirements for data collection, including the format of data, frequency of reporting, level of abstraction of data, level of accuracy of data. |
| Analytics ID | O | The identifier of the analytics event, for which the data collection is needed. This ID can be for example "UE(s) staying time duration analytics", "UE group route analytics", "UE density analytics", or "UE group member deviation analytics". |
| List of Data Producer IDs | O | In case when this request is performed via A-DCCF, then the list of Data Producer IDs is needed. |
| >Target data producer profile criteria | O | Characteristics of the data producers to be used. |
| >VAL UE IDs | O | The VAL UE(s) identifiers for which the data/analytics apply. |
| Area of Interest | O | The geographical or service area for which the requirement request applies. |
| Time validity | O | The time validity of the request. |

#### 8.X.3.6	Location information collection subscription response

Table 8.X.3.6-1 describes information elements for the Data collection subscription response from the Data Producer, e.g. 5GC NFs (e.g. GMLC, NWDAF), ADAE Client, LM server, A-ADRF for historical data.

Table 8.X.3.6-1: Data collection subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the location information collection subscription request (positive or negative acknowledgement). |

#### 8.X.3.7	Data Notification

Table 8.X.3.7-1 describes information elements for the Data Notification from the Data Producer to the ADAE server.

Table 8.X.3.7-1: Data notification

| Information element | Status | Description |
| --- | --- | --- |
| Data Collection Event ID | M | The identifier of the data collection event. |
| Data Producer ID | M | The identity of Data Producer. |
| Analytics ID | O | The identifier of the analytics event. This ID can be for example "UE(s) staying time duration analytics", "UE group route analytics", "UE density analytics", or "UE group member deviation analytics". |
| Data Type | M | The type of reported data samples which can be network data, application data, edge data, or different granularities / abstraction of data (e.g. real time, non-real time). This also indicates whether data are offline (from A-ADRF or not). |
| Data Output | M | The reported data, which can be inform of measurements or offline/historical data on the requested parameter based on subscription. Such data can be location information of UEs (moving devices like UAS devices, V2X devices, robots, and/or people with UE, static devices like infrastructures) for a given time and area of interest, measurement of UE(s) moving in or moving out of the area of interest, UE mobility analytics, Ranging/Sidelink Positioning location information of UEs, Movement behaviour analytics, UE communication analytics, Dispersion analytics, Relative Proximity analytics, and/or Abnormal behaviour analytics. |

#### 8.X.3.8	Get analytics data request

Table 8.X.3.8-1 describes information elements for the location-related UE group analytics request from the analytics consumer to the ADAE server.

Table 8.X.3.8-1: Get analytics data request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the consumer. |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example "UE(s) staying time duration analytics", "UE group route analytics", "UE density analytics", or "UE group member deviation analytics". |
| Analytics type | M | The type of analytics for the event, e.g. statistics or predictions, |
| Analytics filter information | M | Filter information for the analytics event. |
| >Application Group ID | O | Application group identifier for which the analytics subscription applies. |
| >Target VAL UE ID(s) | O | The identifier(s) of the VAL UE(s) for which the analytics subscription applies. |
| >Area of Interest | O | The geographical or service area for which the subscription request applies. |
| >Application Group profile(s) | O | Information about the Application Group(s) with common EAS (as defined in 3GPP TS 23.558 [14] Table 8.X.11-1). |
| >Deviation | O | Deviation information, e.g. different moving direction or moving speed, distance with the group leader UE larger than a threshold. |
| >>Group leader UE ID | O | The identifier of the group leader UE. |
| >>Guideline | O | The guideline used for detecting deviation of group member UE, e.g. distance, moving direction, moving speed. |
| >>Threshold | O | The threshold used for detecting deviation of group member UE from the other group members or group leader with the guideline. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |

#### 8.X.3.9	Get analytics data response

Table 8.X.3.9-1 describes information elements for the Get location-related UE group analytics response from the ADAE server to the consumer.

Table 8.X.3.9-1: Get analytics response

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example "UE(s) staying time duration analytics", "UE group route analytics", "UE density analytics", or "UE group member deviation analytics". |
| Outputs for UE(s) staying time duration | O (NOTE 1) | The reported analytics for UE(s) staying time duration analytics apply. |
| >List of VAL UE ID(s) | M | List of identifiers of the VAL UE(s). |
| >>VAL UE ID | M | Identifier of VAL UE. |
| >>Location | M | Location information where the UE staying in an area. |
| >>Time duration | M | Time duration of the UE staying in the location. |
| >>Times UE leaves area | O | The number of times UE leaves the area. |
| >>Times UE enters area | O | The number of times UE enters the area. |
| >Timestamp | O | Time stamp of the collected analytics data. |
| Outputs for UE group route | O (NOTE 1) | The reported analytics for UE group route analytics apply. |
| >List of EAS ID(s) | M | List of identifiers of EAS. |
| >>EAS ID(s) | O | Identifier(s) of EAS(s). |
| >>Route ID | M | The identifier of the common route for the UE(s) to EAS(s). |
| >>VAL UE ID(s) | M | The identifiers of the VAL UE(s) which with the common route to the EAS(s). |
| >Timestamp | O | Time stamp of the collected analytics data. |
| Outputs for UE density | O (NOTE 1, NOTE 2) | The reported analytics for UE density analytics apply. |
| >Area of Interest | M | The geographical or service area. |
| >Number of UE(s) | O | The number of UE(s) in the AoI. |
| >Density of UE | M | The density of UE in the AoI. |
| >Timestamp | O | Time stamp of the collected analytics data. |
| Outputs for UE group member deviation | O (NOTE 1, NOTE 3) | The reported analytics for group member deviation analytics apply. |
| >Application Group ID | M | Application group identifier. |
| >>Group leader UE ID | O | The identifier of the group leader UE. |
| >List of VAL UE ID(s) | M | The VAL UE(s) which with different behaviour with the majority group members or the group leader. |
| >>VAL UE ID | O | Identifier of the VAL UE. |
| >>Deviation | O | Detail information of deviation for the VAL UE, e.g., different moving direction or moving speed, distance with the group leader. |
| >>Time duration | O | The time duration of this UE deviation. |
| >Timestamp | O | Time stamp of the collected analytics data. |
| Validity Time | O | The valid time duration of the analytics. |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |
| NOTE 1: One of these shall be present based on the analytics event. NOTE 2: Statistics on UE density including e.g. distribution, minimum, maximum, and average density during the validity time. NOTE 3: Statistics on UE deviation including e.g. minimum, maximum, and average UEs moving into area, moving out of area, moving speed, and distance from group leader. |  |  |

* * * Next change * * * *

### 9.2.2	ADAE server APIs

Table 9.2.2-1 illustrates the ADAE server APIs.

Table 9.2.2-1: List of ADAE server APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_ADAE_VAL_performance_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_slice_performance_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_UE-to-UE_performance_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_server-to-server_performance_analytics | Subscribe | VAL server / EES | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_location_accuracy_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_service_API_analytics | Subscribe | VAL server / Subscriber/ API invoker | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_slice_usage_pattern_analytics | Subscribe | VAL server / SEAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_edge_analytics | Subscribe | VAL server / EAS / EES | Subscribe/Notify |
|  | Notify |  |  |
|  | Get | VAL server / EAS / EES | Request / Response |
| SS_ADAES_slice_usage_stats | Get | VAL server | Request / Response |
| SS_ADAES_edge_preparation_analytics | Subscribe | VAL server / ECS / EES | Subscribe/Notify |
|  | Notify |  |  |
|  | Get | VAL server / ECS / EES | Request / Response |
| SS_ADAE_location-related_UE_group_analytics | Subscribe | LM server | Subscribe/Notify |
|  | Notify |  |  |
|  | Get | LM server | Request / Response |

* * * Next change * * * *

### 9.2.Y	SS_ADAE_location-related_UE_group_analytics API

#### 9.2.Y.1	General

API description: This API enables the analytics consumer (e.g. LMS) to communicate with the ADAE server for requesting or subscribing to location-related UE group analytics.

#### 9.2.Y.2	Subscribe

API operation name: location-related_UE_group_analytics_subscribe

Description: The consumer subscribes for location-related UE group analytics.

Inputs: See clause 8.X.3.2.

Outputs: See clause 8.X.3.3.2.

See clause 8.X.2.1 for details of usage of this operation.

#### 9.2.Y.3	Notify

API operation name: location-related_UE_group_analytics_notify

Description: The consumer is notified by the ADAE server on location-related UE group analytics.

Inputs: -

Outputs: See clause 8.X.3.4.

See clause 8.X.2.1 for details of usage of this operation.

#### 9.2.Y.4	Get

API operation name: location-related_UE_group_analytics_get

Description: The consumer requests location-related UE group analytics.

Inputs: -See clause 8.X.3.8

Outputs: See clause 8.X.3.9.

See clause 8.X.2.2 for details of usage of this operation.

* * * End of changes * * * *