3GPP TSG-SA WG6 Meeting #62	S6-243200

Maastricht, The Netherlands, 19th – 23rd August 2024

Source:	Ericsson

Title:	Support of VFL at Enablement Layer

Spec:	3GPP TS 23.482 V0.1.0

Agenda item:	9.10

Document for:	Approval

Contact:	Jing Yue (jing.yue@ericsson.com)

1. Introduction

Added descriptions and procedure for VFL at enablement layer to TS 23.482.

2. Reason for Change

Solution#18 is recommended for normative work on procedure to support Vertical FL (VFL) at enablement layer. See more details in 3GPP TR 23.700-82. This paper proposes to add descriptions and procedure for VFL at enablement layer to TS 23.482.

3. Conclusions

This paper proposes to add descriptions and procedure for VFL at enablement layer to TS 23.482.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.1.0.

* * * First Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

FL		Federated Learning

VFL	Vertical FL

* * * Next Change * * * *

## 8.x	VFL at Enablement Layer

This clause describes procedure for supporting VFL at Enablement Layer.

### 8.x.1	General

### 8.x.2	Procedure

The following clauses specify procedures, information flows and APIs to support VFL among Application Layer multiple UEs.

Pre-conditions:

-	VFL member (e.g., AIMLE Client which is deployed to an UE) has dataset or can access to data sources or is able to collect data from data producers for the AI/ML operations.

Figure 8.x.2-1: Procedure for supporting VFL in Enablement Layer

0.	VFL members (AIMLE Clients which are deployed on UEs) registered their VFL profile to the AIMLE Server. The VFL members may update their status or information in their VFL profile to the AIMLE Server.

1.	Consumer (e.g. VAL Server) sends request to the AIMLE Server for train ML model. The consumer includes an indication for VFL and other VFL requirements such as ML models, required VFL operation features, data samples requirements, VFL members requirements.

2.	After receiving the request from the consumer, the AIMLE Server determines that VFL training between domains is needed based on request from the consumer (if the request on VFL been provided in step 1) or local configuration (according to the requirements from the consumer on the AI/ML operations).

3.	The AIMLE Server gets VFL members information and sends ML model training capability evaluation request to the VFL members (AIMLE Clients which are deployed on UEs). The request message may include e.g. test task, requirements on available time for support VFL operations, ML model, required features of the dataset from the UE, available data sources, domains of the datasets at the UE.

4.	The VFL members evaluate their capability and availability to join the VFL training process. The VFL members (AIMLE Clients which are deployed on UEs) run the test task contained in the request in step 3 and judge whether it will join the VFL process. As part of the test task, data alignment between the datasets of the different domains are determined. The VAL server may also provide data labels for the data alignment.

NOTE:	The procedures for data collection from UE need to take user consent into account.

5.	The VFL members (AIMLE Clients which are deployed on UEs) send response to the AIMLE Server. The response message includes the decision on whether to join the VFL training process or not, with the reason if not join or with test result if join.

6.	The AIMLE Server determines the VFL members for this VFL training process based on the information received in step 5 and parameters received in step 1. The criteria used by the AIMLE Server include:

-	Available data and minimum number of data samples for the same sample among the VFL members.

-	Feature alignment of the sample/datasets with data labels among the VFL members.

-	Available time of the VFL members for support the VFL training operations.

-	Capability and minimum number of the VFL members for the VFL training operations.

- AIML model information for the VFL members and for the AIMLE Server.

7.	The AIMLE Server coordinates the selected VFL members to perform VFL training operations for the VFL process using the parameters received in step 1. During VFL training, the VFL members send the AIMLE Server intermediate results, and the AIMLE Server sends the VFL members gradients to update the model parameters maintained by each VFL member.

7a.	The AIMLE Server may report to the consumer with the intermediate training status. The consumer may adjust its request on the ML model.

8.	The AIMLE Server sends response to the consumer. The response message may contain trained ML model.

### 8.x.3	Information flows

Editor’s Note:	Details of the information flows is FFS.

* * * Next Change * * * *

### 9.Y.A	AIMLE ML Model Training Capability Evaluation API

Editor’s Note:	This clause provides the corresponding AIMLE ML Model training capability evaluation API.

# *** End of Changes ***