3GPP TSG-SA WG6 Meeting #62	S6-243203

Maastricht, The Netherlands, 19th – 23rd August 2024

Source:	Ericsson

Title:	Support Transfer of Intermediate AIML Operation Information

Spec:	3GPP TS 23.482 V0.1.0

Agenda item:	9.10

Document for:	Approval

Contact:	Jing Yue (jing.yue@ericsson.com)

1. Introduction

Added descriptions and procedures for supportting transfer of intermediate AIML operation information to TS 23.482.

2. Reason for Change

Solution#30 is recommended for normative work normative work to support transfer of intermediate AIML operation information. See more details in 3GPP TR 23.700-82. This paper proposes to add descriptions and procedure for supportting transfer of intermediate AIML operation information to TS 23.482.

3. Conclusions

This paper proposes to add descriptions and procedure for supportting transfer of intermediate AIML operation information to TS 23.482.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.1.0.

* * * First Change * * * *

## 8.x	VFL at Enablement Layer

This clause describes procedure for supportting transfer of intermediate AIML operation information to TS 23.482.

### 8.x.1	General

The following clauses specify procedures to support transfer of intermediate AIML operation information.

Pre-conditions:

-	Due to various reasons (e.g., changes of available resource, changes of available time), an AI/ML member (e.g., AIMLE Client, VAL Client) finds itself cannot finish the assigned AI/ML task during the performing process.

-	To save resource and time, the AI/ML member (source AI/ML member) decides to transfer the intermediate AI/ML information (e.g., the intermediate AI/ML operation status and results) to another AI/ML member (target AI/ML member) for further operations to complete the AI/ML task.

### 8.x.2	Procedures for intermediate AI/ML information transfer

#### 8.x.2.1	AIMLE Server assist intermediate AI/ML information transfer

Pre-conditions:

-	The information of target AI/ML member (e.g. another AIMLE Client or VAL Client different from the source AI/ML member) is unknown at the source AI/ML member. The source AI/ML member decides that assist from the AIMLE Server is needed.

Figure 8.x.2.1-1: Procedure for AIMLE Server assist intermediate AI/ML information transfer

Figure 8.x.2.1-1 illustrates the procedure for AIMLE Server assist intermediate AI/ML information transfer. The corresponding procedure in detail is as follows:

- The source AI/ML member (e.g. an AIMLE Client) sends request to AIMLE Server for assisting intermediate AI/ML information transfer. The request message contains the type of the AI/ML operation (e.g. ML model training), the type of the intermediate AI/ML information (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results), the requirement on the transfer (e.g. data volume, except time for finishing the transfer, requirement on the target AI/ML member on available resource and time). The request message may contain requirement on server-controlled intermediate AI/ML information transfer.

Editor’s Note:	The procedure can also be used for transfer AI/ML services (e.g., ML model performance evaluation, ML model update) between AI/ML members. The details is FFS.

- The AIMLE Server discovers other AI/ML members (e.g., other AIMLE Clients) directly or via ML repository, and selects one or more target AI/ML member(s) based on the request from the source AI/ML member.

- The AIMLE Server sends request to the selected target AI/ML member(s) for intermediate AI/ML operation information transfer. The request message contains the information in the request message from the source AI/ML member in step 1, may contain the information of the source AI/ML member.

- The target AI/ML member(s) authenticates and authorizes the request from the AI/ML Enablement Server. If the request is authorized, the target AI/ML member(s) performs step 5.

- The target AI/ML member(s) responses to the AIMLE Server with information (e.g. available time for continue performing the AI/ML operation).

If the information of the source AI/ML member is contained in the request message in step 3, the target AI/ML member(s) may determine to interact with the source AI/ML member directly based on its local configuration. The response message may contain an indication on direct transfer.

- The AIMLE Server determines the transfer mode, i.e. transfer with server-controlled or transfer directly from the source AI/ML member to the target AI/ML member, according to the information of the source and the target AI/ML members (e.g. the location of the source and the target AI/ML members, requirement on transfer QoS, the requirement on server-controlled transfer in the request in step 1, the indication on direct transfer in the response in step 5).

- If target AI/ML member(s) is discovered based on the steps 2-5, the AIMLE Server generates assistance information for the transfer of intermediate AI/ML operation information from the source AI/ML member to the target AI/ML member(s) (e.g. time window for the transfer).

- The AIMLE Server responses to the source AI/ML member with the information of the target AI/ML member(s), and assistance information generated in step 6. The response message contains the information of the target AI/ML member, may contain an indication of direct transfer if it is decided in step 6.

- The intermediate AI/ML operation information may be transferred directly between the source and target AI/ML members as shown in step 9b, or via the AIMLE Server (e.g. AIMLE Server controlled information transfer between two AI/ML members) as shown in step 9a.

If transfer directly, the source AI/ML member performs information transfer to the target AI/ML member(s) based on assistance information received from the AIMLE Server in step 7. The source AI/ML member may initiate the directly transfer process based on its local configuration. Or the target AI/ML member may initiate the directly transfer process, the source AI/ML member may authorize the direct transfer request from the target AI/ML member if the direct transfer indication is received from step 7.

#### 8.x.2.2	Direct intermediate AI/ML information transfer

Pre-conditions:

-	The information of target AI/ML member (e.g., another AIMLE Client or VAL Client different from the source AI/ML member) is assumed to be known at the source AI/ML member, or the source AI/ML member may get the target AI/ML member information via ML repository. The source AI/ML member decides to transfer the intermediate AI/ML operation information to the target AI/ML member.

Figure 8.x.2.2-1: Procedure for direct intermediate AI/ML information transfer

Figure 8.x.2.2-1 illustrates the procedure for direct intermediate AI/ML information transfer. The corresponding procedure in detail is as follows:

- The source AI/ML member (e.g. an AIMLE Client) sends request to the target AI/ML member(s) for intermediate AI/ML operation information transfer. The request message contains the information of the source AI/ML member, the type of the AI/ML operation (e.g. ML model training), the type of the intermediate AI/ML information (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results), the requirement on the transfer (e.g. data volume, except time for finishing the transfer, requirement on the target AI/ML member on available resource and time).

- The target AI/ML member(s) authenticates and authorizes the request from the AIMLE Server. If the request is authorized, the target AI/ML member(s) performs the next steps.

- The target AI/ML member(s) checks its availability for continue the AI/ML operations (e.g. available resource and time).

- The target AI/ML member(s) responses to the source AI/ML member with information (e.g. available time for continue performing the AI/ML operation).

The source AI/ML member interacts with the target AI/ML member(s) for transfer parameters (e.g. transfer time window) and performs intermediate AI/ML operation information transfer to the target AI/ML member(s).

### 8.x.3	Information flows

Editor’s Note:	Details of the information flows is FFS.

* * * Next Change * * * *

### 9.Y.A	AIMLE Intermediate AI/ML Information Transfer API

Editor’s Note:	This clause provides the corresponding AIMLE intermediate AI/ML information transfer API.

# *** End of Changes ***