3GPP TSG-SA WG6 Meeting #62	S6-243214

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	Samsung

Title:	Pseudo-CR on solution for the key issue related to UE RAT connectivity analytics

Spec:	3GPP TR 23.700-01 V1.1.0

Agenda item:	8.6

Document for:	Approval

Contact:	Arunprasath R, arun.prasath@samsung.com

1. Introduction

This pCR proposes the solution for the key issue related to UE RAT connectivity analytics

2. Reason for Change

This pCR proposes the solution for the open issue identifed as part of the key issue related to UE RAT connectivity analytics.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01 v1.1.0.

* * * First Change * * * *

### 7.2.x	Solution #x: Analytics related to UE RAT connectivity

#### 7.2.x.1	Solution description

This solution addresses the open issues listed as part of the key issue X which is related to UE RAT connectivity analytics. It introduces a new analytics type which needs to be supported by the ADAE (Application Data Analytics Enablement) server. It proposes the procedure for supporting the UE RAT connectivity analytics by the ADAE server which can be consumed by the analytics consumers (e.g. VAL server).

##### 7.2.x.1.1	Procedure on UE RAT connectivity analytics

Figure 7.2.x.1.1-1 illustrates the procedure support the UE RAT connectivity analytics.

Figure 7.2.x.1.1-1: Procedure of ADAE Server supporting UE RAT connectivity analytics

1.	The analytics consumer (e.g.VAL server) makes a subscription request to ADAE server for UE RAT connectivity prediction/stats, including an analytics event ID for UE RAT Connectivity analytics. The request may include also the target area, a target VAL service, or a VAL UE, or group of UEs of the VAL service, time validity. If the VAL UEs are provided by the VAL server, this request may also include the expected route or a set of waypoints for the UEs of the VAL application.

2.	The ADAE server sends a UE RAT connectivity analytics subscription response as an ACK to the VAL server.

3.	The ADAE server subscribes to the SEAL Location management server to get the location information of the VAL UE along with the RAT type. It can set the trigger criteria of receiving the location information of the UE whenever UE RAT type changes.

4.	The ADAE server subscribes for NWDAF UE mobility analytics per VAL UE (for all the VAL UEs) and gets notification on the per UE location/mobility analytics based on TS 23.288 clause 6.7.2.

5. 	If the data is collected from multiple sources, the ADAES combines or correlates the data/analytics from steps 3-4 and stores the data into A-ADRF if needed.

6.	The ADAE server optionally requests UE RAT connectivity historical analytics /data from A-ADRF for the corresponding VAL UEs.

7.	Based on the request, the ADAE server receives UE RAT connectivity historical analytics /data from A-ADRF for the corresponding VAL UEs.

8.	The ADAE server abstracts or correlates the data/analytics from steps 5-6 and provides analytics on the UE RAT connectivity for the target VAL application.

9.	The ADAE server sends the location accuracy analytics notification to the consumer

#### 7.2.x.2	Architecture Impacts

This solution has no architecture impacts to ADAE or any other SEAL services.

#### 7.2.x.3	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

This solution is for KI#X and addresses the open issues about the UE RAT connectivity analytics. This solution does not impact the architecture of ADAE or any other SEAL services.

The solution proposes and new analytics ID for the UE RAT connectivity. The analytics consumer can request the ADAE server for gathering the UE RAT connectivity analytics stats/predictions for a UE or set of UEs. The ADAE server interacts with the NWDAF and SEAL LMS for getting the mobility analytics and location of the UE whenever its RAT connectivity changes. It also fetches the historical analytics data from A-ADRF and derives at the UE RAT connectivity analytics stats/predictions. The solution is feasible.

* * * Next Change * * * *

### 10.4.X	Key issue #X:

Solution #AEX addresses the open issues for the key issue #X. It defines a new analytics for the UE RAT connectivity and proposes the procedures for the ADAE server. The solution proposed in this study will be added as part of 3GPP TS 23.436.

* * * Next Change * * * *

### 11.3.X	Conclusions of key issue #X

The study concludes with following solution considerations for the normative work:

-	Solution #AEX