3GPP TSG-SA WG6 Meeting #62	S6-243216

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	Samsung

Title:	Pseudo-CR on procedure to support AIML operation splitting

Spec:	3GPP TS 23.482 V0.1.0

Agenda item:	9.10

Document for:	Approval

Contact:	Arunprasath R, arun.prasath@samsung.com

1. Introduction

Solution #19 in 3GPP TR 23.700-82 illustrates the procedures for supporting AIML operation splitting as part of the AIML enablement layer. The solution introduces a procedure for enabling discovery of AIML split operation pipelines based on VAL client requirements, procedures for subscribing to split operation events notifications at the AIMLE client and VAL server, a procedure for registering AIML split operation pipelines by the VAL server, and a procedure for creating split operation pipeline by AIML enablement client.

The solution allows a VAL client to consume results from a discovered split operation pipeline, to produce intermediate inference results into a discovered split operation pipeline or both.

2. Reason for Change

This pCR introduces the procedure for AIML operation splitting discovery, subscription to the split operation events for 3GPP TS 23.482 V0.1.0

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.1.0.

* * * First Change * * * *

## 8.x	AIML operation splitting

### 8.x.1	General

This clause describes the procedure for the discovery of an AIML split operation pipeline for performing inference via the AIML enablement layer and addresses aspects related to consumption of the pipeline’s inference results.

### 8.x.2	Procedure for AIML operation splitting discovery

Figure 8.x.2-1 illustrates the procedure for AIML operation splitting discovery.

Pre-conditions:

-	AIML inference models are available in the ML model repository or A-ADRF;

-	the AIML enablement server is aware of or can discover compute nodes available for performing AIML inference using available models;

Figure 8.x.2-1: Support for Split AIML operation discovery

0.	An AIML VAL client needs support from the network for performing split AIML operation. The AIML VAL client provides split operation requirements to the AIML enablement client. The requirement may include information about inference stages (e.g., number, order, etc.), information about the trained AIML models (e.g., identifiers, versions, etc.) to be used at each stage, information about the planned usage of the AIML split operation (e.g., predicted inputs frequency/size, requested output frequency / size, etc.). The request may include a minimum number of FL members required for AIML operation splitting.

NOTE 1:	Split operation requirements may be further defined in the normative phase.

1.	The AIML enablement client requests split AIML operation discovery to the AIML enablement server; the request may be based on a VAL client requirement(s). The split operation request may include information in Table 8.x.4.3-2.

2.	Upon receiving the request, the AIML enablement server validates if the requestor is authorized to request discovery of split AIML operation. If the requestor is authorized, the AIML enablement server may determine if an existing AIML split operation pipeline is available and satisfies the request parameters. If an existing AIML split operation pipeline is available, the AIML enablement server may proceed to step 3.

If no AIML split operation pipeline satisfies the request parameters, the AIML enablement server:

-	determines if an AIML split operation pipeline can be created. The determination for AIML split operation pipeline creation may be based on availability of ML inference models in a ML model repository/A-ADRF and may be based on determination of available inference nodes (e.g., VAL server(s), NWDAF(s), etc.) to execute such ML inference models.

NOTE 2:	the AIML enablement server may discover available inference nodes if it has that capability. If the available nodes (at AIML enablement server) that fulfils discovery criteria are less than the required minimum number of nodes for AIML operation splitting, then the AIML enablement server may discover the remaining required nodes from other AIML enablement server over AIML-E reference point and include them in the response message.

NOTE 3:	inference nodes may register with the AIML enablement server and indicate their capabilities for AIML split operation.

-	request or notify the determined inference nodes of their enrolment in the AIML split operation pipeline instance; the request or notification may include configuration information.

-	create a AIML split operation profile if needed. The AIML split operation profile may contain information about the AIML split operation pipeline nodes, configuration and expected usage.

3.	The AIML enablement server sends a split operation response to the AIML enablement client. If the AIML enablement server has determined AIML split operation profile (s) (e.g., existing, or newly formed), the response includes the AIML split operation profile(s). Otherwise, the response may include a failure and reason for failure.

4.	Upon receiving a response including AIML split operation profile(s), the AIML enablement client may store the AIML split operation profile(s). In case, the AIML enablement server does not provide the required number of the nodes in the discovery response (i.e. the discovered nodes in the discovery response message are less than the required minimum number of nodes) for AIML operation splitting, then the AIML enablement consumer can discover the remaining required nodes from other AIML enablement server indicating the required number of nodes. The AIML enablement client provide the discovered split operation information (e.g., endpoints) to the VAL client.

The VAL client may use the provided information to access the AIML split operation pipeline.

If the VAL client is a data provider to the pipeline (e.g., if the VAL client participates in the pipeline), the VAL client may use the split operation discovery information to send data (e.g., intermediate inference data) towards the AIML split operation pipeline head node.

If the VAL client is a data consumer of the AIML split operation inference results from the pipeline (e.g., if the VAL client is a consumer of the pipeline final inference), the VAL client may use the split operation discovery information to request results or subscribe to results from the pipeline tail node.

### 8.x.3	Procedure for subscribe-notify for split operation pipeline events

Figure 8.x.3-1 illustrates the procedure for subscribe-notify for AIML split operation pipeline events.

Figure 8.x.3-1: Split operation pipeline event subscription

1.	The requestor (e.g., AIML enablement client or VAL server) sends a request to subscribe to split operation pipeline events to the AIML enablement server. The request includes requestor identifier, security credentials, events for which the requestor is subscribing, expiration time and pipeline identifier.

2.	Upon receiving the request, the AIML enablement server validates if the requestor is authorized to subscribe for split operation events. If the requestor is authorized, the AIML enablement server creates the subscription and subscription identity.

3.	The AIML enablement server sends subscription response message to the requestor indicating success or failure of the request. In case of success, the response includes the subscription identity and may include an expiration time; to maintain the subscription, the requestor shall send a subscription update request before the expiration time, otherwise the split operation subscription expires. In case of failure, the response includes the failure indication and cause.

4.	The AIML enablement server detects event(s) for which the requestor is subscribed, that is, the AIML enablement client is either added to the pipeline or removed from the pipeline, a new pipeline is added, an existing pipeline is modified (e.g., stages, AIML model, etc.) or deleted. The AIML enablement server sends notification message (4a) to the requestor indicating the event. The notification includes a pipeline identifier or AIML split operation profile dependent on the associated event.

### 8.x.4	Information flows

#### 8.x.4.1	General

The following information flows are specified for AIML operaton splitting.

#### 8.x.4.2	Split operation discovery request

Table 8.x.4.2-1 shows the request sent by a AIML enablement client to the AIML enablement server for split operation discovery request.

Table 8.x.4.2-1: Split operation discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL client ID, AIML client ID, UE identifier) |
| Security credentials | M | The security credentials of the requestor. |
| Split operation requirements | M | Split operation requirements |
| > pipeline stage information | M | Information about the pipeline stages (e.g., number, order, etc.) |
| > model information | M | Information about the AIML models to be used in each stage (e.g., identifiers, versions, etc.) |
| > usage information | O | Information about the planned usage of the AIML split operation pipeline (e.g., inputs frequency/size, output frequency/size, etc.) |
| > required number of nodes | O | Minimum number of nodes required for AIML operation splitting |

#### 8.x.4.3	Split operation discovery response

Table 8.x.4.3-1 shows the response sent by sent by the AIML enablement server to the AIML enablement client for split operation discovery response.

Table 8.x.4.3-1: Split operation discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the discovery request was successful. |
| >list of nodes (NOTE) | O | List of all discovered nodes |
| >List of AIML split operation profiles (NOTE) | O | The list of AIML split operation profiles as specified in Table 8.x.4.3-2. |
| Failure response | O | Indicates that the discovery request failed. |
| >Failure cause | O | Indicates the failure cause. |
| NOTE: Any one of the IE shall be present for successful response. |  |  |

Table 8.x.4.3-2: AIML split operation profiles

| Information element | Status | Description |
| --- | --- | --- |
| pipeline identifier | M | Identifier of the split operation pipeline |
| pipeline head endpoint | M | Endpoint of the pipeline head node (e.g., for providing inference data). |
| pipeline tail endpoint | M | Endpoint of the pipeline head node (e.g., for obtaining inference results). |
| pipeline expected usage | O | Information about usage of the AIML split operation pipeline (e.g., inputs frequency/size, output frequency/size, etc.). |
| pipeline model information | M | Information about the AIML models used in each stage (e.g., identifiers, versions, etc.). |
| pipeline stage information | M | Information about each stage of the pipeline (e.g., number, order, etc.). |

#### 8.x.4.4	Split operation event subscription request

Table 8.x.4.4-1 shows the request sent by a AIML enablement client to the AIML enablement server to subscribe for split operation events.

Table 8.x.4.4-1: Split operation event subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL client ID, AIML client ID, UE identifier). |
| Security credentials | M | The security credentials of the requestor. |
| Subscribed events | M | Indicates the list of subscribed split operation events (e.g., pipeline created, modified, removed). |
| pipeline identifier | O | Identifier of the split operation pipeline if the requestor is subscribing to a specific pipeline related events. |
| Expiration Time | O | Proposed expiration time for the subscription. |

#### 8.x.4.5	Split operation event subscription response

Table 8.x.4.5-1 shows the response sent by the AIML enablement server to the AIML enablement client for Split operation event subscription response.

Table 8.x.4.5-1: Split operation event subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| >Subscription identifier | O | Identifier of the subscription. |
| >Expiration time | O | Indicates the expiration time of the subscription. To maintain an active registration, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the subscription request failed. |
| >Failure cause | O | Indicates the failure cause. |

#### 8.x.4.6	Split operation event notification

Table 8.x.4.6-1 shows the notification sent by the AIML enablement server to the AIML enablement client to notify the client about pipeline events.

Table 8.x.4.6-1: Split operation event notification

| Information element | Status | Description |
| --- | --- | --- |
| Event | M | Indicates the detected split operation event. |
| AIML split operation profile | O | The AIML split operation profile associated with the event (see Table 8.x.4.3-2). |

* * * End Change * * * *