3GPP TSG-SA WG6 Meeting #62	S6-243222

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0076 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multi-flow synchronization for multi-modal XR application |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the conclusion of key issue #2, the solution #5 of TR 23.700-23 can be specified into SEALDD specification to achieve the multi-modal flow synchronization by requesting the 5G network with the required QoS for the adjusted flow(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper is proposed to add the solution #5 of TR 23.700-23 can be specified into SEALDD specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The multi-modal flow synchornization can not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.x (new), 9.x.1 (new), 9.x.2 (new), 9.x.3 (new), 9.x.4 (new), 9.10.3.1, 4.x (new), 6.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.x	SEALDD enabled data transmission for XR application

9.x.1	General

The following clauses specify procedures, information flows and APIs about SEALDD enabled data transmission for XR application.

9.x.2	Multi-flow synchronization for multi-modal XR application

Pre-condition:

-	The VAL server can discover and select the SEALDD server by CAPIF functions.

-	The SEALDD server has been provisioned to multi-modal XR policy including the synchronization threshold, as specified in clause 9.10.3.1.

Figure 9.x.2-1: SEALDD enabled multi-flow synchronization procedure

1.	The VAL server sends a SEALDD Transmission request for multi-modal XR to the SEALDD server. The request includes the identifier of the application traffic (e.g. VAL service ID, VAL server ID), the SEALDD-S connection information for multiple flows from VAL server side (e.g. address/port for audio flow, address/port for tactile flow, etc).

2.	The SEALDD server determines to enable data transmission and synchronization for the multi-modal application (e.g. VAL service ID, VAL server ID). The SEALDD server allocates the SEALDD-S connection information for multiple flows from SEALDD server side, and maintains the multi-flow association information for SEALDD-S interface (i.e., SEALDD-S connection information from VAL server side, and/or SEALDD server side), and sends a response to the VAL server.

3.	The VAL client sends a SEALDD service request to SEALDD client.

4.	The VAL/SEALDD client discovers and selects the proper SEALDD server for the VAL application.

5.	The SEALDD client sends SEALDD Transmission Connection_Establish request for multi-modal XR to SEALDD server with the SEALDD client ID, VAL server ID, VAL service ID, and the SEALDD traffic descriptor of the SEALDD client side.

6.	According to the determination in step 2, if the data transmission and synchronization is enabled for the specified application identifier (e.g. VAL service ID, VAL server ID), the SEALDD server sends the SEALDD traffic descriptors for multiple flows from SEALDD server side (e.g. address/port for multiple SEALDD-Uu flow) to the SEALDD client. The SEALDD server maintains the multi-flow association information for SEALDD-Uu interface (i.e., SEALDD traffic descriptor from SEALDD client side, and/or SEALDD server side).

7.	The SEALDD server performs data transmission quality measurement in SEALDD-Uu interface based on the mapping information for multiple flow association information between SEALDD-S interface and SEALDD-Uu interface. Upon receiving the packets from multiple associated flows in SEALDD-S interface as descripted in step 2, the SEALDD server performs the packet encapsulation with sending timestamp information in the corresponding SEALDD-Uu interface, and can calculate the transmission delay measurement result of multiple associated flows after obtaining the receiving timestamp from the SEALDD client.

8.	Based on the calculated transmission delay for multiple associated flow over SEALDD-Uu interface in step 7, and the synchronization threshold for multi-modal application as described in pre-condition, the SEALDD server can determine the service flow(s) (i.e. address/port for SEALDD-Uu flow) that needs to be adjusted among the multiple associated flows in SEALDD-Uu interface, and the corresponding required QoS information (i.e. transmission delay).

9.	The SEALDD server sends the AF request to 5GC via N33/N5 with the SEALDD traffic descriptor of the adjusted flow(s) (i.e. address/port for the adjusted SEALDD-Uu flow) and the corresponding required QoS information determined in step 8, by utilizing the AF session with required QoS procedure in clause 4.15.6.6 of TS 23.502 [6]. The SEALDD traffic descriptor of the adjusted flow(s) contains the address or port in SEALDD server side, and/or SEALDD client side.

NOTE 1:	This procedure is applicable for both downlink and uplink synchronization of multi-modal flow. For downlink and/or uplink, the step 8 is determined according to the measured downlink and/or uplink transmission delay in step 7.

After requesting the transmission quality optimization on 5G network with the required QoS for the adjusted flow(s), the multi-flow synchronization of multi-modal application can be satisfied.

9.x.3	Information flows

9.x.3.1	SEALDD enabled multi-modal XR transmission request

Table 9.x.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the multi-modal XR transmission service.

Table 9.x.3.1-1: SEALDD enabled multi-modal XR transmission request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Identity | O | Identifier of specific UE or VAL user |
| SEALDD-S Data transmission connection information for multi-modal XR | M | Address(s)/port(s) and/or URL(s) of the VAL server to receive the multi-flow packets from the SEALDD server (e.g. audio flow, tactile flow) |

9.x.3.2	SEALDD enabled multi-modal XR transmission request

Table 9.x.3.2-1 describes the information flow from the SEALDD server to the VAL server for responding to the multi-modal XR transmission.

Table 9.x.3.2-1: SEALDD enabled multi-modal XR transmission response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| SEALDD-S information Data transmission connection information for multi-modal XR | O | Address(s)/port(s) and/or URL(s) of the SEALDD server to receive the multi-flow packets from the VAL server for application traffic transfer (e.g. audio flow, tactile flow) |
| Cause | O See NOTE | Indicates the reason for the failure |
| NOTE: The IE is only present if the Result is failure. |  |  |

9.x.3.3	SEALDD multi-modal XR transmission connection establishment request

Table 9.x.3.3-1 describes the information flow from the SEALDD client to the SEALDD server for requesting the multi-modal XR transmission connection establishment.

Table 9.x.3.3-1: SEALDD multi-modal XR transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| SEALDD client ID | M | Identity of the SEALDD client. |
| Identity | O | The VAL user ID of the VAL user or VAL UE ID |
| SEALDD-UU flow IDs | M | Identity of the SEALDD-UU flows. |
| VAL server ID | O | Identity of the VAL server. |
| VAL service ID | O | Identity of the VAL service. |
| SEALDD traffic descriptors | O | SEALDD traffic descriptors for multiple flows (e.g. address(s), port(s), transport layer protocol) of the SEALDD client side used to establish SEALDD multi-modal XR connection. |

9.x.3.4	SEALDD multi-modal XR transmission connection establishment response

Table 9.x.3.4-1 describes the information flow from the SEALDD server to the SEALDD client for responding to the multi-modal XR transmission connection establishment.

Table 9.x.3.4-1: SEALDD multi-modal XR transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| SEALDD traffic descriptors | O | SEALDD traffic descriptors for multiple flows (e.g. address(s), port(s), transport layer protocol) of the SEALDD server side used to establish SEALDD multi-modal connection. |
| Cause | O (See NOTE) | Indicates the reason for the failure |
| NOTE: The IE is only present if the Result is failure. |  |  |

9.x.4	APIs

9.x.4.1	General

Table 9.3.4.1-1 illustrates the APIs exposed by SEALDD server for XR transmission.

Table 9.3.4.1-1: List of SEALDD server APIs for multi-modal XR transmission

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_XRTransmission | Request | Request/Response | VAL server |
| Sdd_XRTransmissionConnnection | Establish | Request/Response | SEALDD client |

9.x.4.2	Sdd_XRTransmission Request operation

API operation name: Sdd_XRTransmission Request

Description: The consumer requests for one time for multi-modal XR transmission service.

Inputs: See clause 9.x.3.1.

Outputs: See clause 9.x.3.2

See clause 9.x.2 for details of usage of this operation.

9.x.4.3	Sdd_XRTransmissionConnection_Establish operation

API operation name: Sdd_XRTransmissionConnection_Establish

Description: The consumer requests for multi-modal transmission connection establishment.

Inputs: See clause 9.x.3.3.

Outputs: See clause 9.x.3.4.

See clause 9.x.2 for details of usage of this operation.

* * * Next Change * * * *

9.10.3.1	SEALDD policy configuration request

Table 9.10.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the SEALDD policy configuration.

Table 9.10.3.1-1: SEALDD policy configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Application traffic identifiers | M | Identify of the application traffic (e.g. VAL server ID, VAL service ID) |
| -Identity | O | Identifier of the VAL UE or VAL user for which SEALDD policy applies |
| SEALDD policy | M | The SEALDD policy associated with application traffic identifiers, VAL UE identity |
| > Quality guarantee policy | O (See NOTE 1) | Indicates the event (e.g. measurement threshold) to be measured for the quality guarantee |
| > Quality optimization policy | O (See NOTE 1) | Indicates the data transmission adjustment policy (e.g., adjustment need) to be performed in SEALDD layer. |
| > Bandwidth control policy | O (See NOTE 2) | Indicate the bandwidth control preference, e.g. re-allocating the bandwidth limit between different VAL users, including UL/DL |
| > Geofence policy | O (See NOTE 3) | Indicates the geofence policy per VAL service |
| >> Geofence location | O (See NOTE 3) | Indicates the geofence location information for the mentioned VAL service |
| >>Geofence policy action | O (See NOTE 3) | Indicates the policy action like allowed or blocked |
| > temporal policy | O (See NOTE 3) | Indicates the time period for which the sealdd traffic is allowed |
| > Multi-modal XR policy | O (See NOTE 4) | Indicates the SEALDD policy for multi-modal XR application |
| >> synchronization threshold | O (See NOTE 4) | Indicates the synchronization threshold for multi-modal application, as specified in TS 22.261 [2] |
| > policy expiration time | O | Indicates the validity of the policy |
| NOTE 1: This IE is used for the SEALDD enabled transmission quality guarantee, as specified in clause 9.9. NOTE 2: This IE is used for the SEALDD enabled bandwidth control, as specified in clause 9.8. NOTE 3: This IE is used for the SEALDD connection establishment and data delivery, as specified in clause 9.2 NOTE 4: This IE is used for the SEALDD enabled multi-flow synchronization, as specified in 9.x.2. |  |  |

* * * Next Change * * * *

4	Overview

4.1	General

This clause gives a functionality overview for SEALDD service.

4.2	Application signalling and data transmission

This SEALDD functionality provides a mechanism for application signalling data transmission and application media data transmission between VAL client(s) and VAL server(s). The SEALDD enabled regular connection management procedures (e.g. connection establishment, connection deletion) are specified in clause 9.2. For supporting URLLC feature, the SEALDD layer can establish E2E redundant transmission with packet duplication and elimination, as specified in clause 9.3.

4.3	Transmission quality measurement and transmission optimization

The transmission quality measurement procedure specified in clause 9.7, supports the E2E transmission quality measurement between SEALDD client and SEALDD server, and exposes the transmission reports to VAL servers and other consumers (e.g. SEALDD server, NSCE server, etc). Based on the SEALDD enabled E2E transmission measurement result, the SEALDD layer can provide transmission optimization scheme (e.g. triggering redundant transmission, switching another SEALDD server) to improve transmission quality by interacting with 5GC. The transmission optimization procedures are specified in clause 9.9.

4.4	Data transmission rate control

The SEALDD layer can provide the differentiated data delivery service with different bandwidth/transmission rate experience for VAL users, considering the network conditions (e.g. QoS monitoring, ECN marking for L4S report), which is described in clause 9.8.

4.5	Service continuity support

This functionality is provided to support service continuity due to UE mobility or load balance. The SEALDD layer can maintain the transport layer connection by interacting SEALDD context, and requesting 5GC to perform seamless data transmission (e.g. IP replacement procedure, simultaneous connectivity). The service continuity support procedures are specified in clause 9.6.

4.6	Data storage

The SEALDD server supports the data storage and storage management for VAL server, SEALDD client and other SEALDD servers, etc, the corresponding procedure is specified in clause 9.5.

4.x	Multi-modal synchronization for XR applictaion

The SEALDD server supports the E2E multi-flow synchronization for multi-modal XR application, i.e. by requesting the 5G network with the determined QoS for the adjusted flow(s), the corresponding procedure is specified in clause 9.x.

* * * Next Change * * * *

6	Architectural requirements

6.1	General

6.1.1	Description

The general architecture requirements specified in clause 4.1 of 3GPP TS 23.434 [4] are applicable for SEALDD service. This clause specifies the general requirements for SEALDD service.

6.1.2	Requirements

[AR-6.1.2-a] The SEALDD service shall provide a discovery mechanism to support data delivery between VAL client(s) and VAL servers(s) considering different deployments of VAL server(s) (e.g. cloud or edge).

6.2	Data transmission requirements

6.2.1	Description

This clause specifies the data transmission requirements for SEALDD service.

6.2.2	Requirements

[AR-6.2.2-a] The SEALDD service shall provide a mechanism for application signalling data transmission and application media data transmission between VAL client(s) and VAL server(s).

[AR-6.2.2-b] The SEALDD service shall provide a mechanism to support the data transmission quality requirement configurations and measurements for the application data transmission between VAL client(s) and VAL server(s).

[AR-6.2.2-c] The SEALDD service shall provide a mechanism for application data transmission between VAL client(s) and VAL server(s) with guaranteed quality.

[AR-6.2.2-d] The SEALDD service shall provide a mechanism for application data packaging and un-packaging to support the data transmission between VAL client(s) and VAL server(s).

[AR-6.2.2-e] The SEALDD service shall provide a mechanism for E2E redundant data transmission between VAL client and VAL server.

[AR-6.2.2-f] The SEALDD service shall provide a mechanism to support the packet/data duplication, elimination and error recovery between VAL client and VAL server.

6.3	Data storage requirements

6.3.1	Description

This clause specifies the data storage requirements for SEALDD service.

6.3.2	Requirements

[AR-6.3.2-a] The SEALDD service shall provide a mechanism for data storage supporting the CRUD operations.

[AR-6.3.2-b] The SEALDD service shall provide a mechanism to support the data storage status management.

6.4	SEALDD server discovery and selection requirements

6.4.1	Description

This clause specifies the SEALDD server discovery and selection requirements for SEALDD service.

6.4.2	Requirements

[AR-6.4.2-a] The SEALDD service shall provide a mechanism for supporting the SEALDD server discovery and selection for VAL server.

[AR-6.4.2-b] The SEALDD service shall provide a mechanism to provide the information of SEALDD server to VAL/SEALDD client.

6.5	MSGin5G message transfer requirements

6.5.1	Description

This clause specifies the MSGin5G message transfer requirements for SEALDD service.

6.5.2	Requirements

[AR-6.5.2-a] The SEALDD service shall provide a mechanism to support the SEALDD traffic transmission using MSGin5G message.

6.6	Data transmission bandwidth control requirements

6.6.1	Description

This clause specifies the data transmission bandwidth control requirements for SEALDD service.

6.6.2	Requirements

[AR-6.6.2-a] The SEALDD service shall provide a mechanism to support the transmission bandwidth control for VAL application.

6.x	Data transmission optimization for XR application

6.x.1	Description

This clause specifies the data transmission optimization requirements for SEALDD service to support XR application.

6.x.2	Requirements

[AR-6.x.2-a] The SEALDD service shall provide a mechanism to support the E2E multi-flow synchronization for multi-modal XR application.