3GPP TSG-SA WG6 Meeting #62	S6-243229

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	Convida Wireless LLC, KPN N.V.

Title:	AIMLE client discovery/selection

Spec:	3GPP TR 23.482 V0.1.0

Agenda item:	9.10

Document for:	Approval

Contact:	Ly.Quang@convidawireless.com, liu.lu@convidawireless.com

1. Introduction

The conclusion for key issue #3 in TR 23.700-82 stated that solutions #2, #6, and #7 can be merged and be considered for normative work. This contribution focuses on merging portions of the three solutions to support discovery, retrieval, and selection of AIMLE clients. Note that the AIMLE client monitoring procedure in solution #6 is not part of this contribution.

2. Reason for Change

In some scenarios (e.g. FL), an AIMLE service consumer does not have visibility into capabilities supported by AIMLE clients or the available datasets provided by the AIMLE clients. Hence the consumer needs to first discover AIMLE clients with the required capabilities and dataset for the AI/ML application and operations. The AIMLE service consumer may perform multiple discovery/retrieval requests until all the AI/ML requirements (e.g. enough AIMLE clients, available datasets, appropriate age of dataset, enough dataset samples, required QoS, access and/or RAT type, current and historical location) are met before initiating AI/ML operations.

After enough AIMLE clients are discovered that meets all of the AI/ML requirements, the AIMLE service consumer requests the AIMLE server to create a set of the selected AIMLE clients to enable future AI/ML operations (e.g. data management, training, inferencing). The forming of the AIMLE client set is an indication that the AIMLE clients are available and have the required data (or is able to collect and process the required data) to support AI/ML operations and the AIMLE server uses the AIMLE client set to manage the AI/ML operations.

In certain scenarios, the AIMLE service consumer can request the AIMLE server to dynamically perform AIMLE client selection by providing AI/ML policy IDs. The AIMLE server uses the information in the AI/ML policy IDs to select appropriate AIMLE clients for participation in AI/ML operations.

Therefore, AIMLE client discovery and selection procedure needs to be defined to support AIMLE services.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.1.0.

* * * First Change (all new text) * * * *

## 8.x	AIMLE client discovery/selection

### 8.x.1	General

Discovery and selection of AIMLE clients are an important step in the AIML process for distributed, federated, split AI/ML, and transfer learning. Due to the nature of such learning, AIMLE service consumers have limited visibility to the capabilities and available datasets provided by AIMLE clients. The discovery/selection procedure provides the appropriate visibility to allow AIMLE service consumers the ability to find suitable AIMLE clients to fulfil the requirements for the AI/ML application.

The following clauses specify procedures, information flows, and APIs for AIMLE client discovery and selection.

The following definition is used:

-	AIMLE client set identifier is an identifier of the selected AIMLE clients. The VAL group ID may be used as AIMLE client set identifier and AIMLE client may be identified by the VAL UE ID or VAL User ID.

### 8.x.2	Procedure

#### 8.x.2.1	AIMLE Client Discovery/Selection Procedure

Pre-conditions:

1.	AIMLE clients that support AI/ML operations have registered with the AIMLE server and included their AIMLE client profiles and optionally a list of supported services.

2.	The AIMLE server maintains a repository (e.g. ML repository) of AIMLE clients and associated AIMLE client profiles and supported services.

3.	An AIMLE service consumer may provision the required AI/ML policies and have the corresponding policy ID(s).

Figure 8.x.2.1-1: AIMLE client discovery/selection procedure

1.	An AIMLE service consumer (e.g. VAL server) sends a request to an AIMLE server to discover a list of AIMLE clients that are available to participate in AI/ML operations (e.g. is available and have required data to train an ML model) and/or to select a list of AIMLE clients that meet the requirements for AIML operations. The AIMLE client discovery/selection request includes information as described in Table 8.x.3.1-1.

2.	The AIMLE server validates the discovery/selection request and verifies the security credentials provided in the request. The AIMLE server further performs authentication and authorization checks to determine if the requestor is able to discover AIMLE clients or create an AIMLE client set.

3.	If the requestor is authorized, the AIMLE server processes the request as indicated by the discovery/selection indicator.

If the request is for AIMLE client discovery, the AIMLE server determines a list of AIMLE clients that meets the discovery criteria.

NOTE 1:	The AIMLE server may provide a list of target UE(s) and the provided Member UE filtering criteria to utilize the Member UE selection assistance procedure from 3GPP TS 23.502 [x], clause 4.15.13 to get assistance in selecting candidate UEs. The AIMLE server then determines a list of UEs that fulfils the discovery criteria based on information received from AIMLE client registrations (e.g. from AIMLE client profiles).

NOTE 2:	If the available AIMLE clients (as determined by the AIMLE server) that fulfil discovery criteria are less than the required minimum number of AIMLE clients for AI/ML operation (e.g. split AI/ML), then the AIMLE server may discover the remaining required AIMLE clients from other AIMLE servers over AIML-E reference point and include them in the response message.

NOTE 3:	A ML repository can be used to enable this step, depending on deployment choices.

If the request is for AIMLE client selection and the List of AIMLE client IDs IE was provided in step 1, the AIMLE server negotiates with the selected AIMLE clients (i.e., the AIMLE server request the AIMLE clients to confirm their selection) and creates an AIMLE client set with the AIMLE clients that confirmed their selection as selected members of the set and assigns an identifier for the set (e.g., VAL group ID).

If the request is for AIMLE client selection and the Number of the required AIMLE clients IE was provided in step 1, the AIMLE Server shall discover the AIMLE Clients based on provisiond list of AI/ML Policy IDs and AIMLE client profiles. The AIMLE Server may interact with NEF (e.g., uses the NEF procedures for the AFsessionWithQoS described in clause 5.2.6.9 of 3GPP TS 23.502 [x] and the NEF procedures for the AnalyticsExposure described in clause 5.2.6.16 of 3GPP TS 23.502 [x] and in particular the UE Communication Analytics described in clause 6.7.3 of 3GPP TS 23.288 [x] and DN Performance Analytics described in clause 6.14 of 3GPP TS 23.288 [x]), SEAL services (e.g., SS_NetworkResourceMonitoring, SS_LocationAreaInfoRetrieval APIs), NWDAF and/or ADAE services to obtain the historical data, measurements, and/or predictions for relevant parameters (e.g., QoS, location information) in order to check that the selected AIMLE clients will match the AI/ML policies conditions. The AIMLE Server shall obtain the identifiers for AIMLE clients that match AI/ML policy conditions and negotiate with the selected AIMLE Clients (i.e., the AIMLE Server request the AIMLE Clients to confirm their selection) and create an AIMLE Client set with the AIMLE Clients that confirmed their participation as selected members and assign an identifier for the set (e.g., VAL group ID).

NOTE 4:	The member selection mechanism in step 3 is implementation specific.

NOTE 5:	In steps 3, the AIMLE Server provisions the endpoint at the AIMLE service consumer for receiving ML model update from the AIMLE Clients.

NOTE 6: The AIMLE server can reuse SEAL group management for any necessary group management.

4.	After AIMLE clients have been selected, the AIMLE server sends a request to each AIMLE client to verify their participation in the AI/ML operation(s). The request includes information as described in Table 8.x.3.1-4.

5.	The AIMLE client acknowledge its willingness to be part of (or removed from) the AIMLE client set in a response sent to the AIMLE server. The response includes information as described in Table 8.x.3.1-5.

6.	The AIMLE server sends an AIMLE client discovery/selection response that includes information in Table 8.x.3.2-1.

NOTE 7:	In the case the AIMLE server does not provide the required number of the AIMLE clients in the discovery response (i.e., the discovered AIMLE clients in the discovery response message are less than the required minimum number of AIMLE clients) for AI/ML operation (e.g. split AI/ML), the AIMLE service consumer can discover the remaining required AIMLE clients from other AIMLE servers to fulfill the required number of AIMLE clients.

### 8.x.3	Information flows

#### 8.x.3.1	AIMLE client discovery/selection request

Table 8.x.3.1-1 shows the request sent by an AIMLE service consumer to an AIMLE server for the AIMLE client discovery/selection procedure.

Table 8.x.3.1-1: Request for AIMLE client discovery/selection procedure

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g. AIMLE service consumer). |
| VAL service identifier | O | An identifier for the VAL service associated with the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Discovery/selection indicator | M | Indicates whether the request is for AIMLE client discovery or selection. |
| AIMLE client discovery criteria | M (NOTE1) | Discovery criteria for finding suitable AIMLE clients for AI/ML operations as detailed in table 8.x.3.1-2. |
| List of AIMLE client IDs | O (NOTE2) (NOTE3) | A list of AIMLE client IDs that was previously discovered for inclusion into an AIMLE client set. |
| Number of the required AIMLE clients | O (NOTE2) (NOTE3) | Indicates the requested number of AIMLE clients to be selected based on selection policies. |
| List of AI/ML policy IDs for the AIMLE client selection | O (NOTE2) | Represents the list of policies that shall be enforced for the client selection procedure. |
| Consumer Endpoint | M | Represents the endpoint (pair Address-Port or URL) at the AIMLE service consumer for receiving ML model update from the selected AIMLE Client(s). |
| NOTE1: This information element is only applicable when the request is for discovery. NOTE2: These information elements are only applicable when the request is for selection. NOTE3: These information elements are mutually exclusive. |  |  |

Table 8.x.3.1-2: AIMLE client discovery criteria

| Information element | Status | Description |
| --- | --- | --- |
| Requested ML models | O | Requested ML models |
| Requested AIML operations | M | Role for data collection, data preparation, training, and inferencing. |
| Target area | O | Target area for data collection purposes |
| Application layer AIMLE client capabilities | M | Application layer AIMLE client capability information (e.g., ML application type, client availability to support AIML operations at the UE). |
| Dataset requirements | O | Information about dataset |
| >Dataset availability | O | Dataset availability, including dataset identifiers, dataset size, age, list of dataset features. |
| >Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g. raw data), supported data processing capabilities (e.g. processed data) and supported exploratory data analysis functions. |
| Member UE filtering criteria | O | Filtering criteria from Table 4.15.13.2-1 of 3GPP TS 23.502. |
| Required service | O | Information about the service |
| >Required VAL service ID | O | Required VAL service ID that the client supports |
| >Required corresponding service permission level | O | Required corresponding service permission level (e.g., premium resource usage standard resource usage, limited resource usage) |

#### 8.x.3.2	AIMLE client discovery/selection response

Table 8.x.3.2-1 shows the response sent by the AIMLE server to the AIMLE service consumer for the AIMLE client discovery/selection procedure.

Table 8.x.3.2-1: Response for AIMLE client discovery/selection procedure

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| List of AIMLE client IDs | M (NOTE1) | A list of AIMLE client IDs that matches the AIML discovery criteria. |
| Number of discovered AIMLE clients | O | The number of AIMLE client profiles that was found to match the AIML discovery criteria. The number indicates the potential AIMLE clients that could participate in AIML operations. |
| AIMLE client set identifier | M (NOTE2) | An identifier to associate with the set of AIMLE clients that the AIMLE service consumer has provided in the request. The AIMLE client set can be updated by using this identifier. |
| NOTE1: This information element is only applicable for discovery responses. NOTE2: This information element is only applicable for selection responses. |  |  |

#### 8.x.3.3	AIMLE client participation request

Table 8.x.3.3-1 shows the request sent by the AIMLE server to each AIMLE client selected for AIMLE client participation procedure.

Table 8.x.3.3-1: Request for AIMLE client participation procedure

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| AIMLE client set identifier | M | An identifier for the AIMLE client set. |
| Client set operational indicator | M | Indicator for adding/removing the AIMLE client to/from the AIMLE client set. |
| AI/ML model | M | The AI/ML model to use for the AI/ML operation. |
| AI/ML operations | M | A list of AI/ML operations (e.g. data collection, data preparation, training, inferencing) required to be performed. |
| Operational schedule | O | Schedule for the AI/ML operations. |
| Dataset requirement | M | Dataset requirements for the AI/ML operations. Requirements includes dataset identifier, dataset size and age, and/or dataset features. |
| Service requirement | M | Information about the required VAL Service ID and its service permission level (e.g., premium resource usage standard resource usage, limited resource usage) |

#### 8.x.3.4	AIMLE client participation response

Table 8.x.3.4-1 shows the response sent by the AIMLE clients to the AIMLE server for the AIMLE client participation procedure.

Table 8.x.3.4-1: Response for AIMLE client participation procedure

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| AIMLE client set identifier | M | An identifier for the AIMLE client set. |
| Acknowledge | M | An indicator to acknowledge whether the AIMLE client accepts being added/removed from the AIMLE client set. |

* * * End of Change * * * *