3GPP TSG-SA WG6 Meeting #62	S6-243233

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	Convida Wireless LLC

Title:	New solution: Localization service

Spec:	3GPP TR 23.700-21 V1.1.0

Agenda item:	8.4

Document for:	Approval

Contact:	Ly.Quang@convidawireless.com

1. Introduction

The contribution introduces a procedure for metaverse localization service.

2. Reason for Change

Based of comments provided during the Metaverse conference call, the solution specified in S6a240329 that was postponed at SA6 #62 Ad-Hoc meeting have been updated. The procedure has been updated to reflect the discussion as outlined in S6-243142. A consumer can make a localization request based on the 3-dimensional location of the consumer in order to retrieve spatial anchors located within the field of view of the consumer as well as spatial map information based on the 3-dimensional area. Both the spatial map information and the spatial anchor information are returned to the user.

The solution addresses open issue 2 of KI #1 by providing the discovery of spatial anchors to the consumer. The solution also addresses open issue 1 of KI #8 by combining the discovered spatial anchors with other mobile metasverse services.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-21 V1.1.0.

* * * First Change (all new text) * * * *

## 7.x	Solution #x: Localization service

### 7.x.1	Solution description

This solution maps to KI# 1 open issue 2 and KI #8 open issue 1. The solution describes a procedure where a metaverse enabler consumer (e.g. LM client or VAL server) can receive localization service based on the 3-dimensional location of the consumer. The LM server discovers spatial anchors within the field of view of the 3-dimensional area of the consumer and combines the spatial anchor information with information about spatial maps within the 3-dimensional area.

Pre-conditions:

1)	Spatial anchors have been created in the LM server.

2)	Spatial maps have been created in the LM server.

Figure 7.x.1-1: Localization service procedure

1.	A LM client or VAL server sends a localization service request to a LM server. The request includes information as described in Table 7.x.3-1.

2.	The LM server performs an authentication and authorization check of the request. If the check is successful, processing continues to step 3. If not successfully, processing continues to step 5.

3.	Using the 3D location received from the request, the LM server obtains information of a spatial map that aligns with the field of view of the requestor.

4.	Using the 3D location received from the request, the LM server discovers a list of spatial anchors that fall within the field of view of the requestor and also fall within the area of the spatial map. The LM server includes the spatial anchors with the spatial map sent to the requestor.

5.	The LM server sends a response to the requestor. The response includes information as described in Table 7.x.3-2.

### 7.x.2	Architecture Impacts

The solution is based on the application enablement architecture described in clause 6.3.1 but can easily be adapted to other architectures.

### 7.x.3	Corresponding APIs

Table 7.x.3-1 shows the request sent by a LM client or VAL server to a LM server for the Localization service procedure.

Table 7.x.3-1: Request for localization service procedure

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Localized location | M | The 3-dimensional location of the requestor. The 3-dimensional location is used by the LM server to provide localization service to the requestor. |
| Localization area | M | The area extending from the field of view of the requestor for which localization information is to be provided. |

Table 7.x.3-2 shows the response sent by the LM server to the LM client or VAL server for the Localization service procedure.

Table 7.x.3-2: Response for localization service procedure

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The identifier of the requestor. |
| Localization output | M | Output information (spatial anchors that fall within the field of view of the requestor and also fall within the area of the spatial map) for the 3-dimensional localization area. |

### 7.x.4	Solution evaluation

The solution addresses open issue 2 of KI #1 by providing the discovery of spatial anchors to the consumer. The solution also addresses open issue 1 of KI #8 by combining the discovered spatial anchors with other mobile metasverse services. The solution provides localization service for LM client or VAL server and is based on the application enablement architecture described in clause 6.3.1 in which SEAL LM service is enhanced. The solution is a feasible solution.

* * * End of Changes * * * *