3GPP TSG-SA WG6 Meeting #62	S6-243236

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	Convida Wireless LLC

Title:	Digital assets collection management

Spec:	3GPP TS 23.437 V0.1.0

Agenda item:	9.11

Document for:	Approval

Contact:	Ly.Quang@convidawireless.com

1. Introduction

The conclusion for key issue #3 in TR 23.700-21 stated that solution #6 can be considered for normative work. In this  contribution, solution #6 has been adapted to use the A-DACM function to provide management of digital assets collection. The term “digital asset collection” replaces the term “MDRB” described in solution #6.

2. Reason for Change

A user may own many digital assets and requests an A-DACM function to manage the collection of digital assets on behalf of the owner. Having the digital assets in a collection allows for easier management of the digital assets by the A-DACM function. The following solution provides management operations (create, retrieve, update, delete, and subscription) for digital assets collection by the A-DACM function.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 V0.1.0.

* * * First Change (all new text) * * * *

## 8.x	Digital assets collection management

### 8.x.1	General

The following clauses specify procedures, information flows, and APIs for the management of a collection of digital assets.

### 8.x.2	Procedure

#### 8.x.2.1	Digital assets collection create request

Pre-conditions:

1.	Digital assets are available at the VAL application layer.

Figure 8.x.2-1: Digital assets collection create request

- A VAL server sends a request to an A-DACM function to create a digital assets collection. The digital assets collection stores all the digital assets for the owner of the digital assets. The request includes information as described in Table 8.x.3.1-1.

2.	The A-DACM function authenticates and authorizes the request. The A-DACM creates a context for the digital assets collection and associates the information received from step 1 with the digital assets collection. The A-DACM function assigns an identifier for the digital assets collection.

NOTE 1:	The A-DACM function stores information necessary for access and management of the digital assets collection and the exposure of information in the digital assets collection to/from the VAL application layer. This information is provided by the VAL layer or is derived based on the VAL layer request and policies.

Based on the information received from the VAL layer and from local policies, the A-DACM function optionally determines to send one or more requests to the underlying 3GPP network to determine information available in the 3GPP network for the digital assets collection.

NOTE 2:	The information available in the 3GPP network to be included as part of the digital assets collection is to be determined in the normative phase and aligned with specifications from SA2, SA3 and SA4.

3.	The A-DACM function generates a response and sends the response to the VAL Server. The response includes information as described in Table 8.x.3.1-2.

#### 8.x.2.2	Digital assets collection retrieve request

Pre-conditions:

1.	A digital assets collection was previously created.

Figure 8.x.2-1: Digital assets collection retrieve request

- A VAL server sends a request to an A-DACM function to retrieve policies from a digital assets collection. The request includes information as described in Table 8.x.3.2-1.

2.	The A-DACM function authenticates and authorizes the request and retrieves the requested policies from the digital assets collection if authorized.

3.	The A-DACM function generates a response and sends the response to the VAL Server. The response includes information as described in Table 8.x.3.2-2.

#### 8.x.2.3	Digital assets collection update request

Pre-conditions:

1.	A digital assets collection was previously created.

Figure 8.x.2-1: Digital assets collection update request

- A VAL server sends a request to an A-DACM function to update policies in a digital assets collection. The request includes information as described in Table 8.x.3.3-1.

2.	The A-DACM function authenticates and authorizes the request and updates the requested policies in the digital assets collection if authorized.

3.	The A-DACM function generates a response and sends the response to the VAL Server. The response includes information as described in Table 8.x.3.3-2.

#### 8.x.2.4	Digital assets collection delete request

Pre-conditions:

1.	A digital assets collection was previously created.

Figure 8.x.2-1: Digital assets collection delete request

- A VAL server sends a request to an A-DACM function to delete policies in a digital assets collection. The request includes information as described in Table 8.x.3.4-1.

2.	The A-DACM function authenticates and authorizes the request and deletes the requested policies in the digital assets collection if authorized.

3.	The A-DACM function generates a response and sends the response to the VAL Server. The response includes information as described in Table 8.x.3.4-2.

#### 8.x.2.5	Digital assets collection subscription request

Pre-conditions:

1.	A digital assets collection was previously created.

Figure 8.x.2-1: Digital assets collection subscription request

- A VAL server sends a request to an A-DACM function to manage digital assets in a digital assets collection. The request includes information as described in Table 8.x.3.5-1.

2.	The A-DACM function authenticates and authorizes the request and processes the subscription request for the digital assets collection if authorized.

3.	The A-DACM function generates a response and sends the response to the VAL Server. The response includes information as described in Table 8.x.3.5-2.

### 8.x.3	Information flows

#### 8.x.3.1	Digital assets collection create request procedure

Table 8.x.3.1-1 shows the request sent by a VAL server to an A-DACM function for the digital assets collection create procedure.

Table 8.x.3.1-1: Request for digital assets collection create procedure

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Digital asset owner identifier | M | An identifier to associate with the owner of the digital assets collection. |
| List of Metaverse service provider identifiers | M | List of service providers for the Metaverse services. |
| List of associated user information (NOTE 4) | M | List of user information as detailed below. |
| > 3GPP subscriber association information (NOTE 1, NOTE 3) | O | This information is used by the A-DACM function for interactions with 5GS which require subscriber and/or user information. |
| > Metaverse service user information (NOTE 3) | O | Application-level information related to one or more application users. The information may be anonymized, etc. to meet the metaverse application security requirements. This information may represent or be provided by a third party. |
| List of digital assets (NOTE 4) | M | List of digital asset parameters managed by the A-DACM function. |
| > Asset type | M | The type of the digital asset, e.g. avatar, wallet, etc. |
| > Asset type-specific profile | M | Profile information specific to the asset, e.g. an avatar profile. |
| > Asset profile information (NOTE 2) | O | Information used to obtain 5GS services, and which depends on asset type, metaverse use policies. For example, for an avatar asset, the information includes identifiers, access control information, etc necessary to utilize SA4 services. |
| > Communication parameters | O | Communication parameters and requirements for sessions associated with the digital asset. The parameters are also used to create or derive the communication policies applicable to the asset which are persistent within metaverse. |
| > Metaverse experience requirements | O | Information provided to create or derive associated metaverse experience parameters for each asset, e.g. information used to derive applicable rendering parameters. |
| Exposure application rules | O | Rules for the exposure of information in the digital assets collection. The policy may include location, temporal, etc. conditions. |
| NOTE 1: IE to be determined in the normative phase based on SA2 work for user identity exposure NOTE 2: IE to be determined in the normative phase based on SA4 work for avatar services NOTE 3: IE to be determined in the normative phase including SA3 security considerations NOTE 4: Information exposed is based on applicable security requirements and policies in both application and network domains. |  |  |

Table 8.x.3.1-2 shows the response sent by the A-DACM function to the VAL server for the digital assets collection create procedure.

Table 8.x.3.1-2: Response for digital assets collection create procedure

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | Status for the request: success, fail. |
| Digital assets collection identifier | M | Identifier for the digital assets collection for which the digital assets can be operated upon. |
| List of management services | O | List of A-DACM management services available for support of the digital assets collection. |
| Common exposure policy | O | Policy for the exposure of information received in the request, determined based on the rules in the request, local or underlying network policies, etc. Individual exposure rules and policies can be additionally applied by 5GS. The policy may include location, temporal, etc. conditions. |

#### 8.x.3.2	Digital assets collection retrieve request procedure

Table 8.x.3.2-1 shows the request sent by a VAL server to an A-DACM function for the digital assets collection retrieve procedure.

Table 8.x.3.2-1: Request for digital assets collection retrieve procedure

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Digital assets collection identifier | M | Identifier for the digital assets collection for which the digital assets can be operated upon. |
| List of digital asset policies to retrieve | M | List of digital asset policies to retrieve from the collection. |

Table 8.x.3.2-2 shows the response sent by the A-DACM function to the VAL server for the digital assets collection retrieve procedure.

Table 8.x.3.2-2: Response for digital assets collection retrieve procedure

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | Status for the request: success, fail. |
| List of digital asset policies | M | List of digital assets |

#### 8.x.3.3	Digital assets collection update request procedure

Table 8.x.3.3-1 shows the request sent by a VAL server to an A-DACM function for the digital assets collection update procedure.

Table 8.x.3.3-1: Request for digital assets collection update procedure

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Digital assets collection identifier | M | Identifier for the digital assets collection for which the digital assets can be operated upon. |
| List of digital asset policies to update | M | List of digital asset policies to update in the collection. |

Table 8.x.3.3-2 shows the response sent by the A-DACM function to the VAL server for the digital assets collection update procedure.

Table 8.x.3.3-2: Response for digital assets collection update procedure

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | Status for the request: success, fail. |
| List of digital asset policies | M | List of digital assets that was updated |

#### 8.x.3.4	Digital assets collection delete request procedure

Table 8.x.3.4-1 shows the request sent by a VAL server to an A-DACM function for the digital assets collection delete procedure.

Table 8.x.3.4-1: Request for digital assets collection delete procedure

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Digital assets collection identifier | M | Identifier for the digital assets collection for which the digital assets can be operated upon. |
| List of digital asset policies to delete | M | List of digital asset policies to update in the collection. |

Table 8.x.3.4-2 shows the response sent by the A-DACM function to the VAL server for the digital assets collection delete procedure.

Table 8.x.3.4-2: Response for digital assets collection delete procedure

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | Status for the request: success, fail. |
| List of digital asset policies | M | List of digital assets that was deleted |

#### 8.x.3.5	Digital assets collection subscription request procedure

Table 8.x.3.5-1 shows the request sent by a VAL server to an A-DACM function for the digital assets collection subscription procedure.

Table 8.x.3.5-1: Request for digital assets collection subscription procedure

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Digital assets collection identifier | M | Identifier for the digital assets collection for which the digital assets can be operated upon. |
| List of digital asset policies to subscribe | M | List of digital asset policies to subscribe to in the collection. |
| Notification settings | O |  |

Table 8.x.3.5-2 shows the response sent by the A-DACM function to the VAL server for the digital assets collection subscription procedure.

Table 8.x.3.5-2: Response for digital assets collection subscription procedure

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | Status for the request: success, fail. |
| Subscription identifier | M | Identifier for the subscription. |
| Expiration time | O | Expiration time for the subscription |

* * * End of Changes * * * *