3GPP TSG-SA WG6 Meeting #62	S6-243237

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0080 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SEALDD policy configuration for multi-modal flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2024-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update SEALDD policy configuration procedure to include SEALDD multi-modal policy configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the SEALDD policy configuration procedure, provisions were added to enable the configuration of SEALDD multi-modal policy. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without the changes, SEALDD would not support multi-modal connection required for XR communications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 9.10	SEALDD policy configuration

### 9.10.1	General

The following clauses specify procedures, information flow for SEALDD policy configuration. In clause 9.10, the VAL server is a specific server for configurating SEALDD policy, and is different from the VAL server used for VAL application processing in other clauses.

Depending on the configuration, a SEALDD policy may apply to individual SEALDD flows or to multi-modal SEALDD flows.

### 9.10.2	Procedures

#### 9.10.2.1	SEALDD policy configuration

Figure 9.10.2.1-1 illustrates the procedure for SEALDD policy configuration from the VAL server used for SEALDD policy configuration to the SEALDD server.

Figure 9.10.2.1-1: SEALDD policy configuration

1.  The VAL server sends the SEALDD policy configuration request to the SEALDD server. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), VAL UE identify, and the SEALDD policy.

2.  The SEALDD server performs authorization check to verify whether the VAL server can be accepted/authorized to configure the SEALDD policy.

3.  Upon successful authorization, the SEALDD server stores the SEALDD policy for later use (e.g. for bandwidth control, transmission quality guarantee, policy-based SEALDD multi-modal connection establishment) and replies to the VAL server with the SEALDD policy configuration response.

* * * Next Change * * * *

### 9.10.3	Information flows

#### 9.10.3.1	SEALDD policy configuration request

Table 9.10.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the SEALDD policy configuration.

Table 9.10.3.1-1: SEALDD policy configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Application traffic identifiers | M | Identify of the application traffic (e.g. VAL server ID, VAL service ID) |
| -Identity | O | Identifier of the VAL UE or VAL user for which SEALDD policy applies |
| SEALDD policy | O (See NOTE 4) | The SEALDD policy associated with application traffic identifiers, VAL UE identity |
| > Quality guarantee policy | O (See NOTE 1) | Indicates the event (e.g. measurement threshold) to be measured for the quality guarantee |
| > Quality optimization policy | O (See NOTE 1) | Indicates the data transmission adjustment policy (e.g., adjustment need) to be performed in SEALDD layer. |
| > Bandwidth control policy | O (See NOTE 2) | Indicate the bandwidth control preference, e.g. re-allocating the bandwidth limit between different VAL users, including UL/DL |
| > Geofence policy | O (See NOTE 3) | Indicates the geofence policy per VAL service |
| >> Geofence location | O (See NOTE 3) | Indicates the geofence location information for the mentioned VAL service |
| >>Geofence policy action | O (See NOTE 3) | Indicates the policy action like allowed or blocked |
| > temporal policy | O (See NOTE 3) | Indicates the time period for which the sealdd traffic is allowed |
| > policy expiration time | O | Indicates the validity of the policy |
| Multi-modal SEALDD policy | O (See NOTE 4) | Multi-modal SEALDD policy associated with set of individual SEALDD flows. |
| > SEALDD policy configuration ID list | M | List of SEALDD policies used to establish and manage individual SEALDD flows associated with this multi-modal SEALDD poicy |
| > Burst policy | O (See NOTE 5) | Policy used by the SEALDD layer to determine multi-modal flow configuration |
| > Synchronization policy | O (See NOTE 5) | Policy used by the SEALDD layer to perform multi-modal synchronization operations. |
| > Policy expiration time | O | Indicates the validity period of the policy |
| NOTE 1: This IE is used for the SEALDD enabled transmission quality guarantee, as specified in clause 9.9. NOTE 2: This IE is used for the SEALDD enabled bandwidth control, as specified in clause 9.8. NOTE 3: This IE is used for the SEALDD connection establishment and data delivery, as specified in clause 9.2 NOTE 4: Only one of these IEs shall be present and shall depend on whether policy is a SEALDD policy or Multi-modal SEALDD policy. NOTE 5: At least one of these IEs shall be present in a Multi-modal SEALDD policy. |  |  |

* * * End of Changes * * * *