3GPP TSG-SA WG6 Meeting #62	S6-243246

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0084 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Seamless SEALDD server relocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2024-07-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The SEALDD server relocation procedure is missing proper message definition for relocation coordination. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add message definition and API for ACR coordination. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing support for seamless SEALDD server relocation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clauses: 9.6.3.x, 9.6.3.y, 9.6.3.z, 9.6.4.x, 9.6.4.y Impacted existing clauses: 9.6.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.6.3.x	SEALDD Inform ACR event request

Table 9.6.3.x-1 describes the information flow from the VAL server to the SEALDD server to inform the ACR event, for ACR coordination purpose.

Table 9.6.3.x-1: SEALDD Inform ACR event request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |

#### 9.6.3.y	SEALDD Inform ACR event response

Table 9.6.3.y-1 describes the information flow from the SEALDD server to the VAL server for responding to the SEALDD Inform ACR event request.

Table 9.6.3.y-1: SEALDD Inform ACR event response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |

#### 9.6.3.z	SEALDD Inform ACR event notification

Table 9.6.3.z-1 describes the information flow from the SEALDD server to the VAL server for notifying ACR event related information.

Table 9.6.3.z-1: SEALDD Inform ACR event notifiation

| Information element | Status | Description |
| --- | --- | --- |
| SEALDD flow transfer result | M | Success or failure. |

* * * Next Change * * * *

#### 9.6.4.1	General

Table 9.6.4.1-1 illustrates the APIs exposed by SEALDD server for UE’s service continuity.

Table 9.6.4.1-1: List of SEALDD server APIs for UE’s service continuity

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_DDContext | Push Request | Request/Response | SEALDD server |
|  | Pull Request | Request/Response | SEALDD server |
| Sdd_InformACREvent | Request | Subscribe/Notify | VAL server |
|  | Notify | Subscribe/Notify | VAL server |

* * * Next Change * * * *

#### 9.6.4.x	Sdd_InformACREvent_Request operation

API operation name: Sdd_InformACREvent_Request

Description: The consumer informs ACR event to SEALDD server, for ACR coordination purpose.

Inputs: See clause 9.6.3.x.

Outputs: See clause 9.6.3.y.

See clause 9.6.2.2 for details of usage of this operation.

#### 9.6.4.y	Sdd_ InformACREvent _Notify operation

API operation name: Sdd_ InformACREvent_Notify

Description: SEALDD server notifies information about previously informed ACR event.

Inputs: See clause 9.6.3.z.

Outputs: See clause 9.6.3.z.

See clause 9.6.2.2 for details of usage of this operation.

* * * End of Changes * * * *