3GPP TSG-SA WG6 Meeting #62	S6-243248

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	Huawei, Hisilicon

Title:	Solution on Satellite edge computing EES and gNB onboard

Spec:	3GPP TR 23.700-01 v1.1.0

Agenda item:	8.5

Document for:	Approval

Contact:	Yajie Hu (Huyajie1@huawei.com)

1. Introduction

This paper proposes a solution on Satellite edge computing EES and RAN onboard.

2. Reason for Change

In Key issue#5, it is considered that UPF and gNB are deployed on the NGSO, in such case the EES and EAS can also considered to be deployed on the NGSO along with the UPF and gNB.

Thus the issue may happen that, the UE serving satellite is SAT ID#1, which is the gNB related Satellite ID connected by the UE. However the EES#2 may be identified for the UE when SAT ID#1 and SAT ID#2 can both provide service to the UE based on UE current location. Thus the UE may not be able to obtain optimal service via RAN#1-UPF#1-EES#2.

Thus how to optimize the service provisioning procedure to support the EAS and EES onboard should be studied.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01 v1.1.0.

* * * First Change * * * *

7.2.x	Solution #X: Satellite edge computing considering EES and gNB onboard

7.2.x.1	Solution description

7.2.x.1.1	General

Figure 7.2.7.1.1-1: EAS and EES onboard satellite deployment

In this deployment option, ECS is deployed on ground. The UE can be on the ground/sea or in the air (e.g. drone). The gNB and UPF to access satellite EDN is deployed on satellite, EAS and EES are deployed in one or more satellites. EASs and their registered EES are deployed on one or more satellites.

As for the EAS and EES on MEO or LEO satellites and the satellites also deployed with UPF and gNB, however the ECS is on the ground. In the service provisioning procedure, the EEC sends service provisioning request message to the ECS as defined in TS 23.558 clause 8.3.3.2.2, furthermore, the request message contains the UE serving Satellite ID which is the gNB related Satellite ID connected by the UE, then the ECS can determine EES based on the UE serving Satellite ID and EES Satellite ID. Then the ECS sends the service provisioning response message to the EEC contains the identified EES.

Alternatively, in the service provisioning procedure, the EEC sends service provisioning request message to the ECS as defined in TS 23.558 clause 8.3.3.2.2, then the ECS sends the service provisioning response message to the EEC contains list of EES along with EES Satellite ID. Upone receiving the response message, the EEC can determine EES based on the UE serving Satellite ID and EES Satellite ID.