3GPP TSG-SA WG6 Meeting #62	S6-243249

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	Huawei, Hisilicon

Title:	Update evaluation and conclusion on Key issue #2

Spec:	3GPP TR 23.700-01 v1.1.0

Agenda item:	8.5

Document for:	Approval

Contact:	Yajie Hu (Huyajie1@huawei.com)

1. Introduction

This paper proposes to update evaluation and conclusion on Key issue #2.

2. Reason for Change

Solution #AE5 proposes ECS and EES are deployed on ground while EAS is deployed on satellite, however, as for this deployment case, the each EES need to maintain all the EAS information which deployed on the satellite. In such deployement case, the each EES maintain the redundant EAS information which introduces unnecessary redundant information storage and increases the burden of and EES. Thus solution#AE5 is not suitable to be considered as a candidate solution for normative work

Thus the related evaluation and concludion on Key issue #2 should be updated accordingly.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01 v1.1.0.

* * * First Change * * * *

10.4.2	Key issue #2:

Solution #AE1, Solution #AE5 and Solution #AE7 addresses key issue #2.

Solution #AE1, Solution #AE5 and Solution #AE7 resolves open issue to investigate different deployment options for EDGEAPP. Solution #AE1 and Solution #AE7 proposes ECS is deployed on ground while EAS and EES are deployed on satellite.

Solution #AE5 proposes ECS and EES are deployed on ground while EAS is deployed on satellite, however, as for this deployment case, the each EES need to maintain all the EAS information which deployed on the satellite. In such deployement case, the each EES maintain the redundant EAS information which introduces unnecessary redundant information storage and increases the burden of and EES. Thus solution#AE5 is not suitable to be considered as a candidate solution for normative work.

Solution #AE1 (where ECS is deployed on ground while EAS and EES are deployed on satellite), Solution #AE5 (where where ECS and EES are deployed on ground while EAS is deployed on satellite) and Solution #AE7 (where ECS is deployed on ground while EAS and EES are deployed on satellite) resolves open issue to optimise EAS discovery in different deployment scenarios.

Solution #AE1, and Solution #AE7 can be considered for normative work which are for proposing different deployment scenarios and optimized EAS discovery.

* * * NEXT Change * * * *

11.3.2	Conclusions of key issue #1 and #2

The study concludes with following solution considerations for the normative work:

-	Solution #AE1

and

-	Solution #AE7.

* * * End of Change * * * *