3GPP TSG-SA WG6 Meeting #62	S6-243251

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-24xxxx)

Source:	Huawei, Hisilicon

Title:	clarification on relationship with SA4 on data collection

Spec:	3GPP TS 23.482

Agenda item:	9.10

Document for:	Approval

Contact:	Yajie Hu (Huyajie1@huawei.com)

1. Introduction

This paper provide clarification on relationship with SA4 on data collection.

2. Reason for Change

SA4 defined data collection procedure and related functionality, however if AI/ML enabler layer is ehnahnced with data collection then the data collection functionality will be deplicated in SA4 and SA6.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.1.0

* * * First Change * * * *

Annex X(informative):
Relationship with SA4 on data collection

SA4 has an ongoing study on AI and ML for Media (3GPP TR 26.927 [11). This study aims to identify relevant interoperability requirements and implementation constraints of AI/ML in 5G media services. This study includes media-based AI/ML use cases and architecture considerations related to media services.

SA4 defines data collection in TS 26.531 which application client and send the data to data colleciton fucntion via direct data collection client, meanwhile application server can send the data to also to the data collection AF. Thus, SA6 can directly invoke SA4’s capability to obtain the required application data, and does not need to define a duplicated data collection function in AI/ML enabler layer.

In particular, the AI/ML enabler client can obtain data from Direct data collection client on UE side, similarly, the AI/ML enabler server can obtain data from data collection on network side. Alternatively, the AI/ML enabler client or AI/ML enabler server could obtain data from Data Collection AF optionally via AI/ML enabler server.

Figure X-1: Relationship with SA4 on data collection