3GPP TSG-SA WG6 Meeting #62	S6-243256

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0676 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CES consumes EEL services details |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2024-07-30 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The CES, as a service consumer, consuming EEL services is not described in TS 23.558 in detail. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add CES as consumer in applicable procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS remains incomplete for consumer CES. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.5.1, 8.8.3.3, 8.8.4.6, 8.9.4.1, 8.9.2.3, 8.9.3.4, 8.9.3.5, 8.8.3.9, 8.8.4.24, 8.8.4.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.8.5.1	General

Table 8.8.5.1-1 illustrates the APIs for ACR.

Table 8.8.5.1-1: ACR APIs

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_TargetEASDiscovery | Request | Request/Response | EAS, EES |
| Eees_SelectedTargetEAS | Declare | Request/Response | EAS |
| Eecs_TargetEESDiscovery | Request | Request/Response | EES, CES |
| Eees_AppContextRelocation | Request | Request/Response | EEC, EAS |
| Eees_ACREvents | Subscribe | Subscribe/Notify | EEC |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |
| Eees_EELManagedACR | Request | Request/Response | EAS |
|  | Subscribe | Subscribe/Notify | EAS |
|  | Notify |  |  |
| Eees_ACRStatusUpdate | Request | Request/Response | EAS |
| Eees_ACRParameterInformation | Request | Request/Response | EES, CES |
| Ecas_SelectedEES | Declare | Request/Response | EES |
| Eecs_ACREvents | Subscribe | Subscribe/Notify | EES |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |

* * * Next Change * * * *

#### 8.8.3.3	Retrieve T-EES procedure

Figure 8.8.3.3-1 illustrates the procedure for the S-EES to retrieve the T-EES information from the ECS. This procedure is also applicable for the CES to retrieve the T-EES information.

Pre-condition:

1.	The S-EES has been pre-configured with the address of the ECS; and

2.	The AC at the UE already has on-going application traffic with the S-EAS.

Figure 8.8.3.3-1: Retrieve T-EES procedure

1.	The S-EES sends the Retrieve EES request (UE location information or UE identity, EASID of the S-EAS, bundle ID, bundle type (i.e. proxy bundle case), target DNAI and UE connectivity information) to the ECS in order to identify the T-EES which has an EAS available to serve the given AC in the UE. For obtaining announcement EES list, the Retrieve EES request includes application group id. The request message may also contain the AC, EEC service continuity support information.

2.	If the request contains the UE identity (e.g. GPSI) but the UE location is not known to the ECS, then the ECS interacts with 3GPP core network to retrieve the UE location. The ECS determines T-EES(s) as per the parameters (e.g. EASID, target DNAI) in the request and the UE location information. If the request message contains the AC, EEC service continuity support information, then the ECS may identify the T-EES taking the AC, EEC, T-EES service continuity support into consideration. The ECS may take the EAS(s) load information corresponding to the EASIDs registered with EES in the EDN(s) or use ADAES Edge load analytics as specified in clause 8.8.2 of TS 23.436 [27]) to take EDN load into consideration in identifying T-EES(s). When the bundle ID and is provided and bundle type indicating the proxy bundle case then the ECS can identify the T-EES based on the bundle ID in the EES profile and in the request message to ensure T-EAS is able to invoke the required proxy bundle EAS(s) as the S-EAS does.

Editor' Note:	Whether the ECS can obtain EDN load information via other mechanism (e.g., OAM) is FFS, and the system impact of using EAS load info registered in ECS is FFS.

If the Prediction expiration time is provided then the ECS may determine whether to identify T-EES with the instantiable but not instantiated EAS based on Prediction expiration time and predicted EAS deployment time information obtained from ADAES or from local configured maximum EAS deployment time.

Editor's Note: Whether and how the ECS can obtain the EAS deployment time (from ADAES) is FFS.

If no ECS-ER is available and when the Retrieve EES request includes application group id to the ECS then the request EES list retrieval is for the announcement of common EAS, ECS determines the list of EESs serving the EASs (with same EASID) for the Group ID included in the Retrieve EES request.

If the ECS does not identify any suitable EES(s) based on EDN configuration available at the ECS and UE’s location, the ECS determines a partner ECS that may satisfy the requirements. Based on ECSP policy, the ECS may use preconfigured or OAM configured information about the partner ECSs or ECS discovery via ECS-ER as specified in clause 8.17.2.3 or both.

If required by the ECSP policies, the ECS may use service provisioning information retrieval procedure as specified in clause 8.17.2.4 to obtain service provisioning information from the partner ECS.

When the bundle EAS information is provided, then if bundle EAS information includes a list of EASIDs, then the ECS identifies the one or more T-EES associated with the same EDN which support all of the EASs within the same EDN based on the EES EDN information obtained in the EES profile.

NOTE 1:	T-EES(s) may belongs to same EDN.

3.	The ECS sends the Retrieve EES response. If the ECS has determined the EDN configuration information, the retrieve EES response includes the list of EDN configuration information to the S-EES. The list of EDN configuration information includes the EDN details with the endpoint information of T-EES(s) as described in table 8.3.3.3.3-2. If the ECS has determined suitable partner ECS(s) instead, the retrieve EES response includes a list of ECS configuration information.

The ECS may provide associated T-EES(s) information (one or more T-EES information) to the S-EES in the Retrieve T-EES response along with the bundle EAS information (i.e. list of EASIDs). When S-EES receives multiple associated T-EES(s), then each associated T-EES information is provided along with the part of the EAS ID list.

If the retrieve EES response contains a list of ECS configuration information, the S-EES may initiate retrieve T-EES procedure with one or more ECS(s) listed in the retrieve EES response.

NOTE 2:	The Retrieve EES request initiated by the S-EES can be restricted only to its registered ECS.

* * * Next Change * * * *

#### 8.8.4.6	Retrieve EES request

Table 8.8.4.6-1 describes the information elements to retrieve T-EES information from the ECS.

Table 8.8.4.6-1: Retrieve EES request

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | Identifier of the requesting EES or CES. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EASID | M | The identifier of the EAS. |
| Application Group ID | O | Application group identifier as defined in 7.2.11. Indicates to the ECS that the request is to obtain EES list for the announcement of common EAS |
| Bundle ID (NOTE) | O | A bundle ID as described in clause 7.2.10. |
| > Bundle type | O | Type of the EAS bundle as described in clause 7.2.10 |
| Target DNAI | O | The target DNAI information which can be associated with potential T-EES(s) and/or T-EAS(s). |
| UE Identifier | O | The identifier of the UE (i.e,. GPSI) |
| UE location | O | The location information of the UE. The UE location is described in clause 7.3.2. |
| EEC service continuity support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC. |
| Prediction expiration time | O | The estimated time the UE may reach the Predicted/Expected UE location or EAS service area at the latest. |
| AC service continuity support | O | Indicates if the AC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the AC. |
| ENS indication | O | Indicates whether edge node sharing is used. |
| Serving MNO information | O | The serving MNO information (e.g. MNO name, PLMN ID) which is serving the subscriber. |
| NOTE: The bundle ID is provided only when bundle type indicates the proxy bundle. |  |  |

* * * Next Change * * * *

#### 8.9.4.1	General

Table 8.9.4.1-1 illustrates the EEC context management.

Table 8.9.4.1-1: EEC context management APIs

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_EECContextPull | Request | Request/Response | EES |
| Eees_EECContextPush | Request | Request/Response | EES, CES |

* * * Next Change * * * *

#### 8.9.2.3	EEC Context Push relocation

An EEC Context is relocated via an EEC Context Push request initiated by the source EES. This procedure is also applicable for the CES to push EEC Context to the T-EES.

Pre-conditions:

1.	The source EES has provided the EEC with an EEC Context ID.

Figure 8.9.2.3-2: EEC Context relocation procedure initiated by source EES

1.	The source EES determines to forward EEC Context for relocation to a target EES. The source EES determines the target and the EEC Context to be forwarded.

2.	The source EES sends EEC Context Push request to the target EES including the EEC Context determined.

3.	Upon receiving the request from the source EES, the target EES validates the request and verifies the security credentials. The target EES uses the EEC Context ID provided to authorize the EEC Context to be stored and managed. Then the target EES sends an EEC Context response indicating success. The T-EES performs implicit registration and creates the registration ID for the registration and includes it in the EEC context push response message for S-EAS decided ACR or S-EES executed ACR scenarios. The S-EES stores the registration details, and when required, notifies the EEC about registration details while sending ACR information notification.

If the request in step 2 includes the list of selected ACR scenarios within the EEC context, and if the list cannot be supported by T-EES or T-EAS, new list of selected ACR scenarios needs to be selected based on the request from S-EES. The T-EES selects a list of selected ACR scenarios based on T-EES and T-EAS capabilities and "EEC Service Continuity Support" if the IE has been provided in the EEC context and includes it in the Push EEC context response.

NOTE:	In this release, ACR scenario list transfer is not supported if EEC is not registered. The EEC can perform EEC Context establishment and ACR scenario selection with the target EES.

* * * Next Change * * * *

#### 8.9.3.4	EEC Context Push request

Table 8.9.3.4-1 describes information elements in the EEC Context Push request between two EESs or between CES and EES.

Table 8.9.3.4-1: EEC Context Push request

| Information element | Status | Description |
| --- | --- | --- |
| EES ID | M | Unique identifier of the requesting EES or CES. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EEC Context(s) | M (NOTE) | EEC Context list |
| T-EAS Endpoint | O | The endpoint of the selected T-EAS |
| ACR scenario selection request | O | Indicates T-EES to select the ACR scenario. |
| NOTE: This IE contains list only when EEC context push is initiated for more than one UEs connected to common EAS. Otherwise, it contains single EEC context to be pushed to T-EES. |  |  |

* * * Next Change * * * *

#### 8.9.3.5	EEC Context Push response

Table 8.9.3.5-1 describes information elements in the EEC Context Push response between two EES or between CES and EES.

Table 8.9.3.5-1: EEC Context Push response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful. |
| > registration ID (NOTE) | O | Identifier of the registration for the EEC |
| > expiration time (NOTE) | O | Indicates the expiration time of the EEC registration. |
| > Selected ACR scenario list | O | The list of ACR scenarios selected by the T-EES |
| Failure response | O | Indicates that the request failed. |
| > Cause | O | Indicates the cause of request failure, mandatory if the request failed. |
| NOTE: This IE shall be included if implicit registration is performed by T-EES. |  |  |

* * * Next Change * * * *

#### 8.8.3.9	ACR parameter information procedure

Figure 8.8.3.9-1 illustrates the procedure for sending ACR parameters from S-EES to the T-EES and T-EAS. The procedure may be used by the S-EES at the beginning of the ACR execution to provide ACR parameters, e.g. prediction expiration time, to the T-EES and T-EAS. The procedure may also be used during an ongoing ACR to update ACR parameters. This procedure is also applicable for the CES to send ACR parameter information to the T-EES.

Pre-condition:

1.	The ACR has been launched to the S-EES.

Figure 8.8.3.9-1: ACR parameter information procedure

1.	The S-EES sends the ACR parameter information request to the T-EES.

2.	The T-EES checks if the requestor is authorized for this operation. If authorized, the T-EES processes the request.

3.	If the request is authorized and if the T-EAS has subscribed to receive ACR notifications, the EES shall notify the T-EAS by sending an ACR management notification, with "ACT start" event including ACR parameters from the request in step 1, e.g. Prediction expiration time.

4.	In case of service continuity planning, if the T-EAS had included indication of EAS Acknowledgement within ACR management subscribe request, the T-EAS sends EAS Acknowledgement as a response to the ACR management notification. In the Acknowledgement, the T-EAS indicates the acceptance or rejection of the ACT considering ACR parameters (e.g. prediction expiration time).

NOTE:	The T-EAS can use the ACR parameters to handle the ACR. For example, the T-EAS can consider prediction expiration time in deciding whether and for how long to wait for the AC of the UE to connect to it. If the ACT is performed, and the AC does not connect to the T-EAS by "Prediction expiration time", the EAS can send ACT failure with the appropriate cause to the EES. In that case, the T-EAS can delete the transferred application context.

5.	The EES responds to the requestor's request with an ACR parameter information response message.

* * * Next Change * * * *

#### 8.8.4.24	ACR parameter information request

Table 8.8.4.24-1 describes information elements for the ACR parameter information request sent from the S-EES or CES to the T-EES.

Table 8.8.4.24-1: ACR parameter information request

| Information element | Status | Description |
| --- | --- | --- |
| EES ID | M | Unique identifier of the requesting S-EES or CES. |
| EECID (NOTE 1) | M | Unique identifier of the EEC. |
| ACID (NOTE 1) | M | The identifier of the AC. |
| S-EAS Endpoint (NOTE 1) | M | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the S-EAS or CAS. |
| T-EAS Endpoint (NOTE 1) | M | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the T-EAS. |
| ACR parameters | M | ACR parameters |
| > Prediction expiration time | O | The estimated time the UE may reach the Predicted/Expected UE location or EAS service area at the latest |
| NOTE 1: These IEs are used to identify the ACR. |  |  |

#### 8.8.4.25	ACR parameter information response

Table 8.8.4.25-1 describes information elements for the ACR parameter information response sent from the T-EES to the S-EES or CES.

Table 8.8.4.25-1: ACR parameter information response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the ACR parameter information request was successful. |
| Failure response (NOTE) | O | Indicates that the ACR parameter information request failed. |
| > Cause | O | Indicates the cause of ACR parameter information request failure |
| NOTE: One of these IEs shall be present in the message. |  |  |

* * * END of Change * * * *