3GPP TSG-SA WG6 Meeting #62	S6-243266

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	Apple, Telefonica, Lenovo, NCSRD, UMA

Title:	Pseudo-CR on JSON schema and examples for NEF API publication

Spec:	3GPP TR 23.946 v0.5.0

Agenda item:	9.7

Document for:	Approval

Contact:	Walter Featherstone w_featherstone@apple.com

1. Introduction

This pCR provides JSON examples for clause 6.4 on NEF Publishes an API.

2. Reason for Change

This pCR proposes to address the outstanding editor’s note regarding providing JSON examples for clause 6.4 on NEF Publishes an API.

The examples are sourced from the OpenCAPIF Software Development Group, which has offers an open source implementation of the CAPIF Core Function APIs plus the logic and additional services required to fulfill the 3GPP requirements and deliver the expected functionality.

The Application Server (AS) Session with QoS API (3gpp-as-session-with-qos) is leveraged as an example, see clause 5.14 of 3GPP TS 29.122, of the API Exposure Function offered service.

4. Proposal

It is proposed to approve this pCR for 3GPP TR 23.946 v0.5.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[3]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[4]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[5]	ETSI GS MEC 011 V3.1.1: "Multi-access Edge Computing (MEC); Edge Platform Application Enablement".

[6]	3GPP TR 23.958: "Edge Application Standards in 3GPP and Alignment with External Organizations ".

[7]	EVOLVED-5G ICT-41 project, https://evolved-5g.eu

[8]	NEF emulator version 2.2.3, February 2024. https://github.com/EVOLVED-5G/NEF_emulator

[9]	ETSI Software Development Group: "OpenCAPIF", https://ocf.etsi.org

[10]	ETSI Software Development Group GitLab repository: "OpenCAPIF", https://labs.etsi.org/rep/ocf/capif.

[11]		GSMA PRD OPG.02 - "Operator Platform Telco Edge Requirements Version 6.0", https://www.gsma.com/futurenetworks/wp-content/uploads/2024/02/OPG.02-v6.0-Operator-Platform-Requirements-and-Architecture.pdf.

[xx]		OCF Project Common Operations, https://ocf.etsi.org/documentation/develop/testing/testplan/common_operations/.

[xy]		OCF Project Postman Collection, https://ocf.etsi.org/documentation/develop/testing/postman/.

* * * Second Change * * * *

## 6.4	NEF Publishes an API

The 5GS network exposure function (NEF) can use CAPIF capabilities to publish and manage its APIs, including managing charging. One of several possible deployment arrangements is shown in Figure 6.4-1.

Figure 6.4-1: NEF implements the CAPIF API provdier domain functions

Signalling for the NEF to publish its APIs to the CAPIF core function is shown in Figure 6.4-2, followed by tables detailing the mandatory attributes of the associated request and response schema and example content for those requests and responses, which is based on publicly available examples provided by the OpenCAPIF [9].

A pre-condition is that the NEF (API management function) has been made aware of the appropriate CAPIF core function endpoint (apiRoot) with which to initiate the registration, as well as having been provided with a token to access the CAPIF core function.

Figure 6.4-2: Publish a service API

Details of the signalling flow in Figure 6.4-2 are as follows:

NOTE 1:	The JSON schema and example snippets provided in the following clauses are aligned with version 18.5.0 of 3GPP TS 29.222 [3], in which apiVersion is specified as "v1". The example snippets themselves can be tested using the OpenCAPIF Postman collection presented in Annex A and also in the OpenCAPIF Postman documentation [xy].

1. CAPIF_API_Provider_Managment_API

The API provider includes:

-	Authorization HTTP header (access token)

-	Security information enabling the CCF to validate the registration (regSec)

-	An array (apiProvFuncs) of at least one API provider domain function profiles

-	Optionally API provider domain information (apiProvDomInfo) and the API provider name (apiProvName)

Table 6.4-1: Register API provider request (schema)

HTTP POST {apiRoot}/api-provider-management/v1/registrations Request headers: Authorization: "string" Request body: { "regSec": "string", "apiProvFuncs": [ { "regInfo": { "apiProvPubKey": "string" }, "apiProvFuncRole": "string", "apiProvFuncInfo": "string" } ], "apiProvDomInfo": "string", "suppFeat": "string", "apiProvName": "string" }

Given the schema provided in Table 6.4-1, an example of the API Provider registration process using OpenCAPIF open-source implementation is now described and then provided in Table 6.4-1-OCF. The presented traces contain actual values to illustrate the usage of the information elements in the JSON schema.

NOTE 2:	The security related information provided in the examples are truncated for readability, e.g., the value associated with regSec and apiProvPubKey.

Before carrying out onboarding, the provider requires: the OpenCAPIF access token, that will allow the creation of the provider; the CA_ROOT certificate, necessary to validate the Certificates in all requests; and the URL to which to make the request, i.e., <{apiRoot}/api-provider-management/v1/registrations>.

NOTE 3:	In OpenCAPIF, all this information is obtained in a dedicated user registration process [xx].

In the presented example, the OpenCAPIF instance is deployed at capif.mobilesandbox.cloud:37211. This instance URL is assumed throughout the presented example traces.

To carry out the onboarding of a provider, the API Provider is required to provide the following fields:

- regSec: Contains the OpenCAPIF access token obtained during registration process for the creation of the provider. The Token is also provided in the Authorization HTTP Header.

- apiProvFuncRole: Type of provider function to be created. It can be AMF, APF or AEF.

- apiProvPubKey: It is the certificate signing request (CSR) of the Provider’s function that OpenCAPIF will use to issue the provider’s valid certificates. Each function needs to include its own CSR to request a valid Certificate.

- apiProvFuncInfo: Information about the provider function that is being created.

- apiProvDomInfo: Information about the provider domain that is being created.

The API Provider onboarding request example is shown in Table 6.4-1-OCF. The example request is to onboards an API Provider with three functions: AEF, APF and AMF. Therefore, 3 CSRs are provided to obtain 3 Certificates.

Table 6.4-1-OCF: Register API provider request (OCF example)

HTTP POST https://capif.mobilesandbox.cloud:37211/api-provider-management/v1/registrations Request headers: Authorization: Bearer eyJhbGci0iJSUzIlNiIsInR5cCI6IkpXVCJ9 Content-Type: application/json Request body: { "regSec": "eyJhbGci0iJSUzIlNiIsInR5cCI6IkpXVCJ9", "apiProvFuncs": [ { "regInfo": { "apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nMIICeTCCAWECAQAWNDEMMAoGA1UEAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST-----\n" }, "apiProvFuncRole": "AEF", "apiProvFuncInfo": "AEF for NEF example" }, { "regInfo": { "apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nMIICeTCCAWECAQAWNDEMMAoGA1UEAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST-----\n" }, "apiProvFuncRole": "APF", "apiProvFuncInfo": "APF for NEF example" }, { "regInfo": { "apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nMIICeTCCAWECAQAWNDEMMAoGA1UEAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST-----\n" }, "apiProvFuncRole": "AMF", "apiProvFuncInfo": "AMF for NEF example" } ], "apiProvDomInfo": "NEF example", "apiProvName": "OCF_NEF" }

Upon getting this Request, Open CAPIF core function assigns identity IDs for the API provider (apiProvDomId) and each of its functions (apiProvFuncId) and returns them in the response (see next section). Furthermore, it provides certificates (apiProvCert) for each of the API provider functions based on each function’s CSR (apiProvPubKey).

2. 201 Created

The API provider domain has been registered successfully. The URI of the created resource is returned in the "Location" HTTP header, including the assigned registration identifier (registrationId).

Table 6.4-2: Register API provider response (schema)

201 Response headers: Location: {apiRoot}/api-provider-management/v1/registrations/{registrationId} Response body: { "apiProvDomId": "string", "regSec": "string", "apiProvFuncs": [ { "apiProvFuncId": "string", "regInfo": { "apiProvPubKey": "string", "apiProvCert": "string" }, "apiProvFuncRole": "string", "apiProvFuncInfo": "string" } ], "apiProvDomInfo": "string", "suppFeat": "string", "apiProvName": "string" }

The response received from the successful onboarding request using OpenCAPIF project for the creation of a new API Provider contains the following fields:

- apiProvDomId: Identifier of the created provider, which is also the ID returned as part of the ‘Location’ header.

- apiProvCert: Certificate of the provider function created from the CSR sent in the onboarding request, which will be used for subsequent requests from the provider towards OpenCAPIF. Each function is provided with its own Certificate (i.e., one each for the AEF, APF and AMF functions).

- apiProvFuncId: Identifier of the provider function created.

The example Register API provider response is shown in Table 6.4-2-OCF.

Table 6.4-2-OCF: Register API provider response (OCF example)

Response headers: Location: https://capif.mobilesandbox.cloud:37211/api-provider-management/v1/registrations/3a4df5f6e30a5ee34de346004ee6bb Response body: { "apiProvDomId": "3a4df5f6e30a5ee34de346004ee6bb", "regSec": "eyJhbGci0iJSUzIlNiIsInR5cCI6IkpXVCJ9", "apiProvFuncs": [ { "apiProvFuncId": "AEFb1a522967511fd6d6def0cbb3b5f05", "regInfo": { "apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nMIICeTCCAWECAQAWNDEMMAoGA1UEAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST-----\n", "apiProvCert": "-----BEGIN CERTIFICATE -----\nMIIDRTCCA12gAwIBAgIUC2D11wSISz1CjKiFXJ3vwKxVhREwDQYJKoZ\n-----END CERTIFICATE-----\n" }, "apiProvFuncRole": "AEF", "apiProvFuncInfo": "AEF for NEF example" }, { "apiProvFuncId": "APF54f49ee44f67c81fc7d7f321195531", "regInfo": { "apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nMIICeTCCAWECAQAWNDEMMAoGA1UEAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST-----\n", "apiProvCert": "-----BEGIN CERTIFICATE -----\nMIIDRTCCA12gAwIBAgIUC2D11wSISz1CjKiFXJ3vwKxVhREwDQYJKoZ\n-----END CERTIFICATE-----\n" }, "apiProvFuncRole": "APF", "apiProvFuncInfo": "APF for NEF example" }, { "apiProvFuncId": "AMFOdbe115f6759f0fec943321dc2184d", "regInfo": { "apiProvPubKey": "-----BEGIN CERTIFICATE REQUEST-----\nMIICeTCCAWECAQAWNDEMMAoGA1UEAwwDQUGMRcwFQYDVQQ\n-----END CERTIFICATE REQUEST-----\n", "apiProvCert": "-----BEGIN CERTIFICATE -----\nMIIDRTCCA12gAwIBAgIUC2D11wSISz1CjKiFXJ3vwKxVhREwDQYJKoZ\n-----END CERTIFICATE-----\n" }, "apiProvFuncRole": "AMF", "apiProvFuncInfo": "AMF for NEF example" } ], "apiProvDomInfo": "NEF example", "apiProvName": "OCF_NEF" }

3. CAPIF_Publish_Service_API

The API publishing function sends information on the offered APIs by providing the API exposing function profiles array (aefProfiles). The published APIs resource created in response to the request represents all published service APIs of the NEF.

NOTE 4: The example content in Table 6.4.3 is based on the assumption that additional supportedFeatures (and is hence set to "fffff") and apiSuppFeats (and hence is omitted) aren’t supported by the Service API. In addition, the attribute apiStatus is omitted, implying the Service API is active at all AEF(s) present in the aefProfiles attribute. Furthermore, it is assumed the exchange is over CAPIF-4, rather than CAPIF-6/6e. As a consequence, the pubApiPath and ccfId attributes are omitted.

Table 6.4-3: Publish API request (schema)

HTTP POST {apiRoot}/published-apis/v1/{apfId}/service-apis Request body: { "apiName": "string", "apiStatus": { "aefIds": [ "string" ] }, "aefProfiles": [ { "aefId": "string", "versions": [ { "apiVersion": "string", "expiry": "string", "resources": [ { "resourceName": "string", "commType": "string", "uri": "string", "custOpName": "string", "operations": ["string"], "description": "string" }, { "resourceName": "string", "commType": "string", "uri": "string", "custOpName": "string", "operations": ["string"], "description": "string" } ], "custOperations": [ { "commType": "string", "custOpName": "string", "operations": [ "string" ], "description": "string" } ] } ], "protocol": "string", "dataFormat": "string", "securityMethods": [ "string", ], "interfaceDescriptions": [ { "ipv4Addr": "string", "port": "int", "securityMethods": [ "string" ] } ] } ], "description": "string", "supportedFeatures": "string", "shareableInfo": { "isShareable": "boolean", "capifProvDoms": [ "string" ] }, "serviceAPICategory": "string", "apiSuppFeats": "string", "pubApiPath": { "ccfIds": [ "string" ] }, "ccfId": "string", "apiProvName": "string" }

To publish a service using OpenCAPIF, it is necessary to specify the following fields:

- apiName: Name of the API to be published, which must be unique in CAPIF.

- apiStatus: Represents the API status. In addition, if this attribute is omitted, it is understood that the API is available in all AEFs of aefProfiles. It contains the aefIds field which is a list of the AEFs that expose the API. If this list is empty or the attribute is omitted, it is understood that the API is inactive in all aefProfiles.

- aefProfiles: List of Information about AEFs that expose the API. This information contains the following fields:

  - aefId: AEF identifier of the provider that exposes the service.

  - versions: List of information about API versions. Each version contains the resources supported by the API. Each version contains:

    - apiVersion: usable version of the API.

    - expiry: date on which the API version expires and will no longer be available.

    - resources: A list of resources that contain the API version. Each resource contains the following information:

      - resourceName: Name of the resource for which its information is specified.

      - commType: Type of use of the resource, which can be receiving a response when making a request (REQUEST_RESPONSE) or a subscription to receive notifications (SUBSCRIBE_NOTIFY)

      - uri: URI to make the request to use the described API resource.

      - custOpName: Part of the URI structure used for a custom operation associated with the resource. The attributes custOpName and custOperations are mutually exclusive.

      - custOperations: List of the resource's custom operations associated to this resource. The attributes custOpName and custOperations are mutually exclusive.

      - operations: List of HTTP methods supported by the API resource.

      - description: Descriptive text of the API resource.

    - custOperations: List of custom operations without resource association.

  - protocol: Indicates a protocol and protocol version used by the API.

  - dataFormat: Indicates the data sending format which can be JSON, XML or PROTOBUF3.

  - securityMethods: Indicates the possible security methods of the API.

  - domainName: Domain name to which the API belongs.

  - interfaceDescriptions: List of interfaces to which API requests can be made.

  - Only domainName or interfaceDescription can be specified.

- shareableinfo: Indicates whether the service API and/or the service API category can be shared to the list of CAPIF provider domains.

- serviceAPICategory: The service API category to which the service API belongs to.

- apiProvName: API provider Name that publishes the API.

The example API Publish request to publish “3gpp-as-session-with-qos” API from NEF is shown in Table 6.4-3-OCF.

Table 6.4-3-OCF: Publish API request (OCF example)

HTTP POST https://capif.mobilesandbox.cloud:37211/published-apis/v1/APF54f49ee44f67c81fc7d7f321195531/service-apis Request body: { "apiName": "3gpp-as-session-with-qos/", "apiStatus": { "aefIds": ["AEFb1a522967511fd6d6def0cbb3b5f05" ] }, "aefProfiles": [ { "aefId": "AEFb1a522967511fd6d6def0cbb3b5f05", "versions": [ { "apiVersion": "v1", "expiry": "2100-11-30T10:32:02.004Z", "resources": [ { "resourceName": "QOS_SUBSCRIPTIONS", "commType": "SUBSCRIBE_NOTIFY", "uri": "/{scsAsId}/subscriptions", "custOpName": "http_post", "operations": [ "GET", "POST" ], "description": "Endpoint to manage monitoring subscriptions" }, { "resourceName": "QOS_SUBSCRIPTION_SINGLE", "commType": "SUBSCRIBE_NOTIFY", "uri": "/{scsAsId}/subscriptions/{subscriptionId}", "custOpName": "http_get", "operations": [ "GET", "PUT", "DELETE" ], "description": "Endpoint to manage single subscription" } ], "custOperations": [ { "commType": "REQUEST_RESPONSE", "custOpName": "string", "operations": [ "POST" ], "description": "string" } ] } ], "protocol": "HTTP_1_1", "dataFormat": "JSON", "securityMethods": [ "OAUTH", "PSK" ], "interfaceDescriptions": [ { "ipv4Addr": "127.0.0.1", "port": 4443, "securityMethods": [ "OAUTH" ] } ] } ], "description": "3gpp-as-session-with-qos NEF API", "supportedFeatures": "fffff", "shareableInfo": { "isShareable": true, "capifProvDoms": [ "mobilesandbox.cloud" ] }, "serviceAPICategory": "NEF", "apiProvName": "OCF_NEF" }

4. 201 Created

The service API has been published successfully The URI of the created resource is returned in the "Location" HTTP header, including the assigned service API identifier (serviceApiId) that is also equal to the API identifier (apiId) in the response body.

Table 6.4-4: Publish API response (schema)

201 Response headers: Location: {apiRoot}/published-apis/v1/{apfId}/service-apis/{serviceApiId} Response body: { "apiName": "string", "apiId": "string", "apiStatus": { "aefIds": [ "string" ] }, "aefProfiles": [ "object" ], "description": "string", "supportedFeatures": "string", "shareableInfo": { "isShareable": "boolean", "capifProvDoms": [ "string" ] }, "serviceAPICategory": "string", "apiSuppFeats": "string", "pubApiPath": { "ccfIds": [ "string" ] }, "ccfId": "string" "apiProvName": "string" }

The response to a successful API Publish request using OpenCAPIF contains the following information:

- Location: URL with the resource created for the API Published.

- apiId: The assigned Identifier to the Published API. It is required for the UPDATE or DELETE API procedures once the API has been published.

The information from the API that was sent in the request will be obtained as a response with a new field, the "apiId", which is the identifier of the API that has been created and will enable subsequent modify or delete procedures.

Table 6.4-4-OCF: Publish API response (OCF example)

Response headers: Location: https://capif.mobilesandbox.cloud:37211/published-apis/v1/APF54f49ee44f67c81fc7d7f321195531/service-apis/f55731ec5f8ce703ac1f69605ad095 Response body: { "apiName": "3gpp-as-session-with-qos", "apiId": "f55731ec5f8ce703ac1f69605ad095", "aefProfiles": [ { "aefId": "AEFb1a522967511fd6d6def0cbb3b5f05", "versions": [ { "apiVersion": "v1", "expiry": "2100-11-30T10:32:02.004Z", "resources": [ { "resourceName": "QOS_SUBSCRIPTIONS", "commType": "SUBSCRIBE_NOTIFY", "uri": "/{scsAsId}/subscriptions", "custOpName": "http_post", "operations": [ "GET", "POST" ], "description": "Endpoint to manage monitoring subscriptions" }, { "resourceName": "QOS_SUBSCRIPTION_SINGLE", "commType": "SUBSCRIBE_NOTIFY", "uri": "/{scsAsId}/subscriptions/{subscriptionId}", "custOpName": "http_get", "operations": [ "GET", "PUT", "DELETE" ], "description": "Endpoint to manage single subscription" } ], "custOperations": [ { "commType": "REQUEST_RESPONSE", "custOpName": "string", "operations": [ "POST" ], "description": "string" } ] } ], "protocol": "HTTP_1_1", "dataFormat": "JSON", "securityMethods": [ "OAUTH", "PSK" ], "interfaceDescriptions": [ { "ipv4Addr": "127.0.0.1", "port": 4443, "securityMethods": [ "OAUTH" ] } ] } ], "description": "3gpp-as-session-with-qos NEF API", "supportedFeatures": "fffff", "shareableInfo": { "isShareable": true, "capifProvDoms": [ "mobilesandbox.cloud" ] }, "serviceAPICategory": "NEF", "apiProvName": "OCF_NEF" }

5. CAPIF administrator creates policy

The procedures for creation of the access control policy at the CAPIF core function by the CAPIF administrator is outside the scope of 3GPP TS 23.222 [2] and may by triggered because of the API management function issuing the "Register API provider" request or prior to that. Either way, sufficient information must be provided to enable the AEF specific access control policy list resource for the service API to be created. Specifically, each policy must include the CCF assigned API invoker identifier (apiInvokerId) and can include allowed total invocations (allowedTotalInvocations), invocations per second (allowedInvocationsPerSecond), invocation time range list (allowedInvocationTimeRangeList). Since an API invoker identifier is mandatory, it is a prerequisite that API invoker has been onboarded prior to invoking the highlighted access control policy procedures.

6. CAPIF_Access_Control_Policy_API

The Access Control List (ACL) service allows the provider to use AEF to obtain the access control list from invokers for an exposing API. The provider is in charge of making the request to the CAPIF Core Function and using the list received to control an invoker's access to one of its APIs.

Table 6.4-6: GET access control policy request (schema, then example content).

Table 6.4-6: ACL Request (schema)

HTTP GET {apiRoot}/access-control-policy/v1/accessControlPolicyList/{serviceApiId}?aef-id="string"

The AEF may make a request to OpenCAPIF to obtain all the ACLs of a service that it is exposing, for this it must specify in the URL of the request the identifier of the service (apiId received in the API Publish response) for which it wants the list of ACLs, in addition to the AEF’s own identifier to check that it has access to that list.

Table 6.4-6-OCF: ACL Request (OCF example)

HTTP GET https://capif.mobilesandbox.cloud:37211/access-control-policy/v1/accessControlPolicyList/f55731ec5f8ce703ac1f69605ad095?aef-id=AEFb1a522967511fd6d6def0cbb3b5f05

7. 200 OK

The request has been successfully received and the CAPIF Core Function returns the invoker access control information for the requested API. The AEF will use this information to allow or not the use of the API by the invoker.

Table 6.4-7: GET access control policy response (schema)

200 Response body: { "apiInvokerPolicies": [ { "apiInvokerId": "string", "allowedTotalInvocations": "integer", "allowedInvocationsPerSecond": "integer", "allowedInvocationTimeRangeList": [ { "startTime": "string", "stopTime": "string" } ] } ] }

OpenCAPIF provides the access control policy information for the NEF API Published "3gpp-as-session-with-qos" containing the following information:

- apiInvokerId: identifier of the invoker to which the access policy belongs.

- allowedTotalInvocations: Total number of invocations allowed on the service API by the API invoker.

- allowedInvocationsPerSecond: invocations per second allowed on the service API by the API invoker.

- allowedInvocationTimeRangeList: the time ranges during which the invocations are allowed on the service API by the API invoker.

Table 6.4-7-OCF: GET access control policy response (OCF example)

200 Response body: { "apiInvokerPolicies": [ { "apiInvokerId”: "INVd4d70e16c52bace2b8f40ed6f6badS", "allowedTotalInvocations": 1200, "allowedInvocationsPerSecond": 2, "allowedInvocationTimeRangeList": [ { "startTime": "2024-08-19T09:00:00.000000+02:00", "stopTime": "2024-08-23T16:00:00.000000+02:00" } ] } ] }

8. Offboarding API Provider

The provider can request its deletion from CAPIF using the offboarding service. By using this service, all information stored in the CAPIF Core Function related to the provider will be deleted, including APIs published by the provider, security contexts of those APIs, ACLs, etc.

Table 6.4-8: DELETE API provider request (schema)

HTTP DELETE {apiRoot}/api-provider-management/v1/registrations/{registrationId}

For the process of deleting a Provider in OpenCAPIF, you will need the AMF certificate and the identifier of the provider that you want to delete. This data is obtained in the provider creation process and by simply making the corresponding request we will eliminate the provider.

Table 6.4-8-OCF: DELETE API provider request (OCF example)

HTTP DELETE https://capif.mobilesandbox.cloud:37211/api-provider-management/v1/registrations/3a4df5f6e30a5ee34de346004ee6bb

9. 204 No Content

The response will be a 204 No Content, which indicates that the provider has been successfully deleted along with everything related to it, APIs Published, security contexts associated with its services, ACLs of its services, etc.

* * * End of Changes * * * *