3GPP TSG-SA WG6 Meeting #62	S6-243271

Maastricht, Netherlands, 19th – 23rd August 2024

Source:	InterDigital Inc.

Title:	Pseudo-CR on Spatial anchor functional model

Spec:	3GPP TS 23.437 v0.1.0

Agenda item:	9.11

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This contribution proposes normative text on spatial anchor management architecture.

2. Reason for Change

The following has been concluded in TR 23.700-21 regarding spatial anchor management architecture.

1.	For Key issue #1 Enabler support for managing spatial anchors, it is concluded to define new SEAL server to support spatial anchor management. The new SEAL server will consider to re-use the architecture as specified in option#1.

3. Conclusions

This pCR provides normative text for KI#1 architecture.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[r23434]		3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[r23401]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[r23501]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[r23502]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[r23222]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[r23682]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

* * * Next Change * * * *

# 8	Spatial Anchors (SAn) management

## 8.1	General

Spatial anchors management is a SEAL service that offers spatial anchors management related capabilities to one or more vertical applications.

* * * Next Change * * * *

## 8.2	Functional model for spatial anchors management

### 8.2.1	General

The functional model for spatial anchors management is based on the generic functional model for SEAL services specified in 3GPP TS 23.434 [r23434]. It is organized into functional entities to describe a functional architecture which addresses the support for spatial anchors management aspects for vertical applications. The on-network functional model is specified in this clause.

### 8.2.2	On-network functional model description

Figure 8.2.2-1 illustrates the generic on-network functional model for spatial anchors management.

Figure 8.2.2-1: On-network functional model for spatial anchors management

The spatial anchors management client communicates with the spatial anchors management server over the SAnM-UU reference point. The spatial anchors management client provides support for spatial anchors management functions to the VAL client(s) over the SAnM-C reference point. The VAL server(s) communicate with the spatial anchors management server over the SAnM-S reference point. The spatial anchors management server communicates with other spatial anchors management server instances over the SAnM-E reference point. The spatial anchors management server interacts with the NEF and the SCEF of the underlying 3GPP network system via the N33 and T8 reference points, respectively.

Figure 8.2.2-2 exhibits the service-based interfaces for providing and consuming spatial anchors management services. The spatial anchors management server could provide service to VAL server and spatial anchors management client through interface Ssanm.

Figure 8.2.2-2: Architecture for spatial anchors management – Service based representation

Figure 8.2.2-3 illustrates the service-based representation for utilization of the 5GS network services based on the 5GS SBA specified in 3GPP TS 23.501 [r23501].

Figure 8.2.2-3: Architecture for spatial anchors management utilizing the 5GS network services based on the 5GS SBA – Service based representation

Figure 8.2.2-4 illustrates the service-based representation for utilization of the Core Network northbound APIs via CAPIF.

Figure 8.2.2-4: Utilization of Core Network Northbound APIs via CAPIF – Service based representation

The spatial anchors management server acts as authorized API invoker to consume services from the Core Network northbound API entities like SCEF, NEF, SCEF+NEF which act as API Exposing Function as specified in 3GPP TS 23.222 [r23222].

### 8.2.3	Off-network functional model description

Editor’s Note: Whether an off-network functional model description is required is FFS.

### 8.2.4	Functional entities description

#### 8.2.4.1	General

The functional entities for the spatial anchors management SEAL service are described in the following subclauses.

#### 8.2.4.2	Spatial anchors management client

The spatial anchors management client functional entity acts as the application client for management of spatial anchors. It interacts with the spatial anchors management server to discover and obtain spatial anchors of interest within a location area.

#### 8.2.4.3	Spatial anchors management server

The spatial anchors management server is a functional entity that receives and stores spatial anchor information, and that provides spatial anchor information to spatial anchors management clients and VAL servers.

The spatial anchors management server acts as CAPIF's API exposing function as specified in 3GPP TS 23.222 [r23222]. The spatial anchors management server also supports interactions with the corresponding spatial anchors management server in distributed SEAL deployments.

### 8.2.5	Reference points description

#### 8.2.5.1	General

The reference points for the functional model for spatial anchors management are described in the following subclauses.

#### 8.2.5.2	SAnM-UU

The interactions related to spatial anchors management functions between the spatial anchors management client and the spatial anchors management server are supported by SAnM-UU reference point. This reference point utilizes Uu reference point as described in 3GPP TS 23.401 [r23401] and 3GPP TS 23.501 [r23501].

SAnM-UU reference point provides a means for the spatial anchors management client to discover and request spatial anchors available at the spatial anchors management server. The SAnM-UU reference point shall use SIP-1 and SIP-2 reference points for subscription/notification related signalling. For transport and routing of spatial anchors management related signalling, SAnM-UU reference point uses the HTTP-1 and HTTP-2 signalling control plane reference points.

#### 8.2.5.3	SAnM-C

The interactions related to spatial anchors management functions between the VAL client(s) and the spatial anchors management client within a VAL UE are supported by SAnM-C reference point.

#### 8.2.5.4	SAnM-S

The interactions related to spatial anchors management functions between the VAL server(s) and the spatial anchors management server are supported by SAnM-S reference point. This reference point is an instance of CAPIF-2 reference point as specified in 3GPP TS 23.222 [r23222].

SAnM-S reference point is used by the VAL server to provide and request information about spatial anchors from the spatial anchors management server. The SAnM-S reference point shall use SIP-1 and SIP-2 reference points for subscription/notification related signalling. For transport and routing of spatial anchors management related signalling, SAnM-S reference point uses the HTTP-1 and HTTP-2 signalling control plane reference points.

#### 8.2.5.5	SAnM-E

The interactions related to spatial anchors management functions between the spatial anchors management servers in a distributed deployment are supported by SAnM-E reference point.

#### 8.2.5.6	T8

The reference point T8 supports the interactions between the spatial anchors management server and the SCEF and is specified in 3GPP TS 23.682 [r23682].

Editor’s Note: Specific T8 functionality supported by spatial anchors management server is FFS.

#### 8.2.5.7	N33

The reference point N33 supports the interactions between the spatial anchors management server and the NEF and is specified in 3GPP TS 23.501 [r23501] and 3GPP TS 23.502 [r23502].

Editor’s Note: Specific N33 functionality supported by spatial anchors management server is FFS.