3GPP TSG-SA WG6 Meeting #62 adhoc-e	S6-243278

e-meeting, 10th – 18th July 2024	(revision of S6-24xxxx)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on new solution to User consent for nested API invocation

Spec:	3GPP TR 23.700-22 v0.4.0

Agenda item:	8.2

Document for:	Approval

Contact:	gecuili@huawei.com

- Introduction

This pCR is proposed to introduce a new new solution to user consent for nested API invocation and API invocation with topology hiding case.

2. Reason for Change

There are some issues with the nested API invocation and the API invocation with topology hiding, where more than one AEF is involved in the API invocation:

Issue #1 – Normally the AEF provides the service service API will do the ROC as the ROC maybe the internal logic of the service API. So it is possible that the ROC is performed by the AEF providing the servcei API.

Issue #3 – How the AEF is aware of the ROC is already performed by the first AEF, it may still perform the ROC check redundantly.

Issue #2 – AEF-1 to handle the ROC checking, but it is not clear which service API the border AEF needs to check the ROC.

This solution is proposed to resolve the above issues.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR  23.700-22 0.4.0.

* * * First Change * * * *

6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  | X |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |
| Sol #5 |  |  |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  | X |
| Sol #7 |  |  |  |  |  |  | X |
| Sol #8 |  |  |  |  |  |  | X |
| Sol #9 | X |  |  |  |  |  |  |
| Sol #10 | X |  |  |  |  |  |  |
| Sol #11 | X |  |  |  |  |  |  |
| Sol#X | X |  |  |  |  |  |  |

* * * Next Change * * * *

6.x	Solution #X: Provision user consent policy for nested API invocation and API invocation with topology hiding

6.x.1	Description

This solution proposes to nested API invocation and API invocation with topology hiding where there are more than one AEF are involved in the API invocation. To reduce the authorization information inquiry, this solution is proposed to perform ROC checking only once. To enable a flexible ROC enforcement check point, this solution also is proposed to make the CCF to select the AEF to check the ROC.

This solution introduces a new IE to the service API information to indicate it is a RNAA service API which requires the ROC check. The APF publishes such service API with ROC indication to the CAPIF core function, and the CAPIF core function determines which entity performs the ROC checking, i.e., either the entry point AEF or the AEF providing the service API in the topology case, or the AEF-1 as described in 3GPP TS 23.222 [] clause 8.32 in the nested API invocation case. Then the CCF provisions the corresponding ROC checking policy to the entry point AEF(or AEF-1) and the AEF providing the service API to instruct them to perform the ROC checking of the service API invocation only once.

Figure 6.x.1-1 illustrates the procedure to provision user consent policy for nested API invocation and API invocation with topology hiding after the service API with ROC indication is published.

Pre-conditions:

1.	The API exposing functions 1 and 2 are in the same trust domain.

2.	Authorization details of the APF are available with the CAPIF core function.

Figure 6.x.1-1: Procedure for obtaining authorization information in a nested API invocation

1.	The API publish function publishes the RNAA service API. A new ROC indication parameter is additionally included in the service API information as defined in clause 8.3.2.1. It indicates the service API is an RNAA type API and requires ROC checking upon the API invocation. Also, scope of the ROC checking which includes the attributes (e.g., operations, resources) to be checked, ROC checking enforcement point preference (i.e., either the entry point AEF or the AEF itself) may be included.

2.	The CAPIF core function, based on the ROC indication, determines the single entity (i.e., either the AEF 1 or the AEF 2) to perform the ROC checking in order to reduce the authorization information inquiries for the nested API invocation. The ROC supporting capability of the API exposing function 1, local policy may be considered to determine the entity to enforce the ROC checking.

3.	The CAPIF core function returns the service API publish response.

4. 	If the API exposing function 1 is selected to perform the ROC checking:

4a.	The CAPIF core function sends a ROC checking policy provision to the API exposing function 1. The list of service API(s), ROC checking indication, and the ROC checking scope are included. The API exposing function 1 stores the information for further usage.

4b.	The CAPIF core function also sends a ROC checking policy provision to the API exposing function 2. The list of service API(s), skip ROC checking indication are included. So, the API exposing function 2 will not repeat the ROC checking.

5. 	If the API exposing function 2 is determined to perform the ROC checking, the CAPIF core function sends a ROC checking policy provision to the API exposing function 2. The list of service API(s), ROC checking indication, and the ROC checking scope are included. The API exposing function 1 does not check any ROC upon the API invocation if not configured with the ROC checking policy.

Figure 6.x.1-2 illustrates the procedure of reducing the authorization information inquiries for the nested API invocation.

Figure 6.x.1-2: Procedure for obtaining authorization information in a nested API invocation

In step 3.	The API exposing function 1 based the ROC policy provisioned by CCF, determines to enforce the ROC checking upon this service API invocation.

In step 6. The API exposing function 2 based the ROC policy provisioned by CCF, skips the ROC checking.