3GPP TSG-SA WG6 Meeting #62	S6-243279

10th – 18th July 2024	(revision of S6-240xxx)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on CAPIF API publish and discovery enhancement

Spec:	3GPP TR 23.700-22 0.4.0

Agenda item:	8.7

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

This pCR proposes a new solution to address the key issue #1.

2. Reason for Change

Three optional OAuth 2.0 flows (Client credential flow (according to RFC 6749 [4]), Authorization code flow (according to RFC 6749 [4]), or Authorization code flow with PKCE (according to RFC 7636 [11])) for RNAA authorization are specified in TS 33.122. A service APIs or an API invoker may support only one or two of the optional flows. Currently, the CCF is not aware the OAuth 2.0 flows per each service API, then a OAuth 2.0 flow maybe negotiated between the CCF and the API invoker while the AEF/service API does not support this negotiated OAuth 2.0 flow. For example:

API invoker supports the Client credential flow and Authorization code flow, but the Service API A supports Authorization code flow and Authorization code flow with PKCE.  Then if Client credential flow is selected by the CCF due to not aware of the service API capabilities from the AEF, then failure occurs.

So, in order to avoid such failure, the service API publishing and discovery procedures need updates to reflect the fact that a service API does not have to support all the flows.

3. Conclusions

This pCR proposes a new solution to address the key issue #1.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-22 V0.4.0.

* * * First Change * * * *

6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  | X |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |
| Sol #5 |  |  |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  | X |
| Sol #7 |  |  |  |  |  |  | X |
| Sol #8 |  |  |  |  |  |  | X |
| Sol #9 | X |  |  |  |  |  |  |
| Sol #10 | X |  |  |  |  |  |  |
| Sol #11 | X |  |  |  |  |  |  |
| Sol#X | X |  |  |  |  |  |  |

* * * First Change * * * *

6.X	Solution #X: Support more service API statuses in RNAA

6.x.1	Solution description

6.x.1.1	General

This solution addresses the key issue #1, bullet 1 - How consent of the resource owner can be managed through communication between the resource owner and authorization function in the CAPIF core function. It proposes to provide more information about the service API when APF publishing the service API to the CCF. Specifically, three optional OAuth 2.0 flows for RNAA authorization are specified in TS 33.122 (clause 6.5.3.1). The CCF should be informed which OAuth flows are supported by the service API. This is to help API invoker to select the right service API with a supported authorization flow.

The CAPIF procedures, especially the API publishing and discovery procedures in 3GPP TS 23.222 [2] can be enhanced (highlighted in bold italics) as follows.

8.3.2.1	Service API publish request

Table 8.3.2.1-1 describes the information flow service API publish request from the API publishing function to the CAPIF core function.

Table 8.3.2.1-1: Service API publish request

| Information element | Status | Description |
| --- | --- | --- |
| API publisher information | M | The information of the API publisher may include identity, authentication and authorization information |
| Service API information | M | The service API information includes the service API name, API provider name (optional), List of public IP ranges of UEs (optional), service API type, service API status (e.g. active, inactive), communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format, Service KPIs (optional). supported OAuth Flows (optional) |
| Shareable information | O (see NOTE) | Indicates whether the service API or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API or the service API category can be published is contained. |
| NOTE: If the shareable information is not present, the service API is not allowed to be shared. |  |  |

The Service KPIs is defined as below:

Table 8.3.2.1-2: Service KPIs

| Information element | Status | Description |
| --- | --- | --- |
| Maximum Request rate | O | Maximum request rate from the API Invoker supported by the server. |
| Maximum Response time | O | The maximum response time advertised for the API Invoker's service requests. |
| Availability | O | Advertised percentage of time the server is available for the API Invoker's use. |
| Available Compute | O | The maximum compute resource available for the API Invoker. |
| Available Graphical Compute | O | The maximum graphical compute resource available for the API Invoker. |
| Available Memory | O | The maximum memory resource available for the API Invoker. |
| Available Storage | O | The maximum storage resource available for the API Invoker. |
| Connection Bandwidth | O | The connection bandwidth in Kbit/s advertised for the API Invoker's use. |

8.7.2.1	Service API discover request

Table 8.7.2.1-1 describes the information flow service API discover request from the API invoker to the CAPIF core function.

Table 8.7.2.1-1: Service API discover request

| Information element | Status | Description |
| --- | --- | --- |
| API invoker identity information | M | Identity information of the API invoker discovering service APIs |
| Query information | M | Criteria for discovering matching service APIs (e.g. service API type, Serving Area Information (optional), preferred AEF location (optional), required API provider name (optional), UE IP address (optional), interfaces, protocols), Service KPIs (optional) (see NOTE) |
| NOTE: It should be possible to discover all the service APIs. |  |  |

8.7.2.2	Service API discover response

Table 8.7.2.2-1 describes the information flow service API discover response from the CAPIF core function to the API invoker.

Table 8.7.2.2-1: Service API discover response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the discovery of the service API information |
| Service API information (see NOTE 2) | O (see NOTE 1) | List of service APIs corresponding to the request, including API description such as service API name, service API type, Serving Area Information (optional), interface details (e.g. IP address, port number, URI), protocols, version, data format, Service KPIs (optional), ). supported OAuth Flows (optional) |
| CAPIF core function identity information | O (see NOTE 1) | Indicates the CAPIF core function serving the service API category provided in the query criteria |
| NOTE 1: The service API information or the CAPIF core function identity information or both shall be present if the Result information element indicates that the service API discover operation is successful. Otherwise both shall not be present. NOTE 2: If topology hiding is enabled for the service API, the interface details shall be the interface details of AEF acting as service communication entry point for the service API. |  |  |

6.x.2	Architecture Impacts

No impact.

6.x.3	Corresponding APIs

Existing CAPIF service APIs supporting RNAA.

6.x.4	Solution evaluation

This solution addresses KI#1 with enhanced information, i.e., supported OAuth flows, of the service API in the context of RNAA. The impacted CAPIF procedures includes service API publish and service API discovery.

# *** End of Changes ***