3GPP TSG-SA WG6 Meeting #62	S6-243297

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	Huawei, HiSilicon

Title:	pCR on solution of data channel application related capability exposure

Spec:	3GPP TR 23.700-92 v1.1.0

Agenda item:	8.2

Document for:	Approval

Contact:	gecuili@huawei.com

1. Introduction

This pCR proposes solution of data channel application related capability exposure which solves the Key Issue#7.

2. Reason for Change

Contribution with new solution solving KI#7.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-92 v1.1.0.

* * * First Change * * * *

## 8.x	Solution #x: data channel application related capability exposure

### 8.x.1	Description

This clause provides solution against Key Issue #7 Support of data channel application related capability exposure.

As specified in clause 6.2.10 of 3GPP TS 26.114[14] that the data channel application is created prior to the DCMTSI call.

In this procedure, the eMMTel Enabler Server provides service for the DC application provider to support DC application upload, upgrade, deletion etc, based on the capabilities of the IMS subsystem.

### 8.x.2	Procedures

#### 8.x.2.1	Data channel application upload

Figure 8.X.2.1-1 below illustrates how to upload a data channel application to DCAR via eMMTel Enabler Server.

Pre-conditions:

1)	The MNO has specified the content to be contained in the DC application package.

2)	The MNO has provided a unified code and DC information format.

Figure 8.x.2.1-1: Upload a data channel application to DCAR via eMMTel Enabler Server

- The application provider invokes the API of the Controlling Application Server to set the application information, the request message includes information elements as specified in Table 8.x.2.1-1.

Table 8.x.2.1-1: Information elements in the application information set request

| Information element | Status | Description |
| --- | --- | --- |
| Application Provider | M | The Application Provider of the Data Channel Application |
| Service Type | O | Service type of the Data Channel Application, used to help the user understand the type of services provided by the application |
| Application Name | O | Name of the Data Channel Application |
| Application Icon | O | Icon of the Data Channel Application |
| If Work Without Peer DC | O | Indicates whether this Data Channel Application can be used if Data Channel is not supported by the other party of the call. |
| Supported Scenario | O | Indicates supported media type required for this Data Channel Application. The values of this IE include: d) Voice call only: this Data Channel Application can be used if and only if the corresponding call is a voice call. e) Video call only: this Data Channel Application can be used if and only if the corresponding call is a video call. f) Voice and video call: this Data Channel Application can be used in both voice call and video call. |
| Condition | O | Indicates whether this Data Channel Application can be used if under this condition, e.g. the application cannot be used beyond a certain area. |
| 3gpp Qos Hint | O | Indicates the QoS requirement of this Data Channel Application which is used to set the value of “a=3gpp-qos-hint” attribute in SDP for the data channel(s) used by the application. |
| PersonalDataCollection (see NOTE1) | O | Indicates whether this Data Channel Application will collect the user personal data. |
| PersonalDataCollectionInfoURL (see NOTE1) | O | URL to retrieve the description of the purpose of mandatory and optional personal data to be collected (along with related consent opt-in/opt-out procedures), their processing their protection etc. |
| NOTE1: These IEs are mandatory when regulatory requirements request that the user has to be informed if the personal data are collected by the application and how they are treated. NOTE2: Other application provider-specific IE's that belong to table 8.4.2.2 can also be added in Table 8.x.2.1-1. |  |  |

2.	The Controlling Application Server verifies that the Application Provider is authorized to create DC application, and store the DC application information.

3.	The Controlling Application Server sends the application information response to the Application Provider; the response message includes information elements as specified in Table 8.x.2.1-2.

Table 8.x.2.1-2: Information elements in the application information set response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of set application information. |
| Failure Cause (see NOTE) | O | The reason for failure |
| Application ID | M | Identifier of the Data Channel Application |
| NOTE: This IE shall only be present when the set application information result is Failure. |  |  |

4.	The application provider uploads data channel application package to Controlling Application Server, the request message includes information elements as specified in Table 8.x.2.1-3.

Table 8.x.2.1-3: Information elements in the upload DC application package request

| Information element | Status | Description |
| --- | --- | --- |
| Application ID | M | Identifier of the Data Channel Application |
| Application version | M | Version of the Data Channel Application |
| DC Application Package | M | Code package of the Data Channel Application |

5.	The Controlling Application Server audits the DC application and generates the DC application profile based on the information from steps 1 and 4.

NOTE1:	The approval process of DC applications may not be real-time.

NOTE2:	The MNO can generate other MNO-related parameters in the DC application profile according to specific policies.

6.	The Controlling Application Server send the DC application package configuration request to the eMMTel Enabler Server.

NOTE3:	The DC application profile configuration procedure from Controlling Application Server to eMMTel Enabler Server has been captured in solution#4.

7.	There are two options as below:

Option 1 SA2 defines the DCAR interface: The eMMTel Enabler Server uploads the DC application package to DCAR via NEF or DCSF.

Option 2 SA6 defines the DCAR interface: The eMMTel Enabler Server invokes the API of the DCAR to upload the DC application package.

Editor's note:	Whether chose Option 1 or 2 is based on the LS reply of SA2 that whether they have a plan to define an interface for application providers to manage the applications to DCAR.

8.	The eMMTel Enabler Server send the DC application package configuration response to the Controlling Application Server.

9.	The Controlling Application Server send upload DC application package response to the application provider, the response message includes information elements as specified in Table 8.x.2.1-4.

Table 8.x.2.1-4: Information elements in the upload DC application package response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of upload DC application package request. |
| Failure Cause (see NOTE) | O | The reason for failure |
| NOTE: This IE shall only be present when the upload DC application package result is Failure. |  |  |

#### 8.x.2.2	Data channel application upgrade

Figure 8.X.2.2-1 below illustrates how to upgrade the version of a data channel application via eMMTel Enabler Server.

Pre-conditions:

1)	The MNO has specified the content to be contained in the package.

2)	The DC application has been uploaded to DCAR via eMMTel Enabler Server.

Figure 8.x.2.2-1:  Upgrade the data channel application version via eMMTel Enabler Server

- The application provider invokes the API of the Controlling Application Server to upgrade the DC application, the request message includes information elements as specified in Table 8.x.2.2-1.

Table 8.x.2.2-1: Information elements in the upgrade DC application package request

| Information element | Status | Description |
| --- | --- | --- |
| Application ID | M | Identifier of the Data Channel Application |
| Application version | M | Version of the Data Channel Application |
| Application related parameters | O | Other application information that may need to be updated, e.g. the elements in DC application profile. |
| DC Application Package | M | Code package of the Data Channel Application |

- The Controlling Application Server checks whether the application version included in the upgrade application request is the newest version of the application, if yes, update the application version in the DC application profile based on the Application ID.

- The Controlling Application Server send the DC application package upgrade request to the eMMTel Enabler Server.

- There are two options as below:

Option 1 SA2 defines the DCAR interface: The eMMTel Enabler Server upgrades the DC application package to DCAR via NEF or DCSF.

Option 2 SA6 defines the DCAR interface: The eMMTel Enabler Server invokes the API of the DCAR to upgrade the DC application package.

Editor's note:	Whether chose Option 1 or 2 is based on the LS reply of SA2 that whether they have a plan to define an interface for application providers to manage the applications to DCAR.

- The eMMTel Enabler Server send the DC application package upgrade response to the Controlling Application Server.

- The Controlling Application Server send upgrade DC application package response to the application provider, the response message includes information elements as specified in Table 8.x.2.2-4.

Table 8.x.2.2-2: Information elements in the upgrade DC application package response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of upgrade DC application package request. |
| Failure Cause (see NOTE) | O | The reason for failure |
| NOTE: This IE shall only be present when the upgrade DC application package result is Failure. |  |  |

#### 8.x.2.3	Data channel application delete

Figure 8.X.2.3-1 below illustrates how to delete a data channel application via eMMTel Enabler Server.

Pre-conditions:

1)	The DC application has been uploaded to DCAR via eMMTel Enabler Server.

Figure 8.x.2.3-1:  Delete a data channel application via eMMTel Enabler Server

- The application provider invokes the API of the Controlling Application Server to delete the DC application, the request message includes information elements as specified in Table 8.x.2.3-1.

Table 8.x.2.3-1: Information elements in the delete DC application package request

| Information element | Status | Description |
| --- | --- | --- |
| Application ID | M | Identifier of the Data Channel Application |

- The Controlling Application Server verifies that the sender is authorized to delete the DC application, then delete the application profile based on the Application ID.

- The Controlling Application Server send the DC application package delete request to the eMMTel Enabler Server.

- There are two options as below:

Option 1 SA2 defines the DCAR interface: The eMMTel Enabler Server delete the DC application package via NEF or DCSF.

Option 2 SA6 defines the DCAR interface: The eMMTel Enabler Server invokes the API of the DCAR to delete the DC application package.

Editor's note:	Whether chose Option 1 or 2 is based on the LS reply of SA2 that whether they have a plan to define an interface for application providers to manage the applications to DCAR.

- The eMMTel Enabler Server send the DC application package delete response to the Controlling Application Server.

- The Controlling Application Server send delete DC application package response to the application provider, the response message includes information elements as specified in Table 8.x.2.3-4.

Table 8.x.2.3-2: Information elements in the delete DC application package response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of delete DC application package request. |
| Failure Cause (see NOTE) | O | The reason for failure |
| NOTE: This IE shall only be present when the delete DC application package result is Failure. |  |  |

### 8.x.3	Impacts on Existing Nodes and Functionality

In this solution, the procedures of application upload, application upgrade and application deletion need the capabilities provided by IMS core.

* * * End of Changes * * * *