3GPP TSG-SA WG6 Meeting #62	S6-243302

Maastricht, The Netherlands, 19th – 23rd August 2024

Source:	Ericsson

Title:	AIML Services Lifecycle Operations Control and Management

Spec:	3GPP TS 23.482 V0.1.0

Agenda item:	9.10

Document for:	Approval

Contact:	Ashish S Sharma (ashish.sanjay.sharma@ericsson.com)

1. Introduction

Adding AIML Services Lifecycle Operations Control and Management procedure.

2. Reason for Change

3GPP TR 23.700-82 Solution#11 is recommended for normative work for the AIML Enablement service lifecycle management. The procedure enables management of the service lifecycle with service operation modes like pause, continue etc. The service operation mode provides better resource optimization. For example in model training service, the training can be configured to stop automatically if it is using maximum memory or cpu. By using service operation mode, val server or aiml server can pause the aiml service and perform some action related to the service and again resume or continue the aiml service. For example, the val server can pause the model training, load new data set, and again resume the model training service to train on new dataset. There are cases where the model saturates or it does not improve the model performance, in such situations, the val server can stop the aiml service, which saves compute and associated costs. Sometimes it is important to pause the model training and do some hyperparameter optimization and then resume the model training. Similarly in FL model training, the VAL server may pause the training of devices which are in poor network coverage. There are numerous examples and above are only a few examples to describe how managing the aiml service life cycle is critical to aiml services.

Since the AIML service can be compute-heavy and due to resource constraints, it is important to consider the efficient usage of the resources. Therefore the solution provides the AIML service operation mode configurations to enable the conditional triggering and/or scheduled triggering of the AIML service operation modes. Also, the heterogeneity of AIML clients like tiny IoT devices which are low in resources(like network, compute, power) may require the network-controlled conditional triggering for efficient usage of client resources.

Also, the AIML service is dynamic. For example, the participating device in model training can go offline anytime(like the user shutdowns the application) or the device in poor network coverage. Therefore the solution provides the configuration of event-based state transition reporting of the current status of the AIML service operation. It can also be configured for periodic status reporting of the AIML service operation.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.1.0.

* * * First Change * * * *

## 8.x	AIML service lifecycle management procedure

### 8.x.1	General

The lifecycle management of the AIML services is an essential requirement for the applications to manage the AIML services like model training, inference, discovery etc. An AIML service (as defined in TR) is equivalent to an AIMLE service and assists in performing or enabling one or more AIML operations.

### 8.x.2	AIML service lifecycle management procedure

Figure 8.x.1-1: AIML service lifecycle management procedure

- The VAL server sends AIML service lifecycle management request to the AIML Enablement server. The request contains AIML service operation ID, AIML client ID, AIML clients group ID, AIML service operation mode like start, pause, continue, and finish. AIML service operation ID can represent both the process ID and service operation (e.g. model training) to identify the AIML service. The VAL server can also configure the AIML service operation mode status reporting configuration to receive the AIML service operation mode notifications from the AIML clients. Optionally, the VAL server can provide the AIML server with AIML service operation mode configuration like automatic AIML service operation mode configuration, compute, network utilization-based, time-limit or performance-based to configure conditional triggering of the AIML service operation modes for the AIML clients.

2.	The AIML Enablement server sends the AIML Enablement client service operation request to the AIML Enablement client(s). The request contains AIML client ID, AIML service operation mode, AIML service operation mode status reporting configuration, AIML service operation mode configuration. The AIML Enablement client on receiving the AIML service operation mode performs the service operation mode for the AIML service operation.

The Start operation mode defines the initiation of the AIML service. The pause operation mode defines the temporary stop of the AIML service. During the pause operation mode, the AIML client can save the current state of the AIML service operation like the current state of the model training. The continue operation mode defines the resuming of the paused operation mode. The AIML client can restore the saved state and resume the AIML operation without any interruptions. The finish mode is defined to stop the AIML operation. 

If the Automatic AIML service operation mode configuration is Yes then the AIML Enablement server monitors the AIML enablement clients as per the AIML service operation mode configuration and sends the required AIML service operation mode in the in the AIML Enablement client service operation request. If Automatic AIML service operation mode configuration is No, then the AIML Enablement server includes the AIML service operation mode configuration in the AIML Enablement client service operation request for the AIML enablement clients, which is applied by the AIML enablement client after receiving the step 2 request message.

3.	The AIML Enablement client sends a response indicating the success or failure of the AIML Enablement client service operation response. Based on the AIML service operation mode status reporting configuration, the AIML client reports the service operation mode status to the AIML Enablement server or to the configured endpoint. The reporting can be configured as periodic or event-based. For event-based the AIML client monitors the state transition of the AIML service operation and reports to the AIML Enablement server. The AIML Enablement client uses the AIML service operation mode configuration for the conditional triggering of the requested AIML service operation mode for the AIML service operation.

4.	The AIML Enablement server provides the AIML service lifecycle management response to the VAL server. The message includes AIML service operation ID and the status of the AIML service operation.

5.	The VAL server sends AIML service lifecycle management update request to the AIML Enablement server with the AIML service operation ID and update information.

6.	The AIML Enablement server receives the request and performs steps 2 and step 3 The AIML Enablement server provides the AIML service lifecycle management update response to the VAL server. The message includes the result of the update and the AIML service operation ID.

* * * Next change * * * *

### 8.x.3	Information flows

#### 8.x.3.1	AIML service lifecycle management request/update request

Table 8.x.3.1-1 shows the request sent by an AIML service consumer(e.g. VAL server) to an AIML Enablement server for the AIML service lifecycle management request /update request

Table 8.x.3.1-1: AIML service lifecycle management request/update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g. AIML service consumer). |
| VAL service identifier | O | An identifier for the VAL service associated with the requestor. |
| AIMLE client identifier | O | Indicates the AIML enablement client identity |
| AIML service operation ID | M | An identifier to identify the AIML service operation ID |
| AIML service operation mode | M | Indicates the required AIMLE service operation modes like start, continue, pause, finish. |
| Automatic AIML service operation mode configuration | O | Indicates the automatic enablement for the AIML service operation mode configuration for the network-driven or client-driven. Possible values Yes for network-driven, No for client-driven. |
| AIML service operation mode configuration | O See NOTE 1 | Indicates the configuration of the AIML service operation modes |
| > Compute utilization-based service operation mode configuration | O | Indicates the compute utilization (memory, cpu, gpu), power utilization(like battery level) threshold values as a condition for the requested AIML service operation mode. E.g. stop the AIML service when memory is 70% utilization |
| > Network utilization-based service operation mode configuration | O | Indicates the network utilization (bandwidth, coverage, latency) threshold values as a condition for the AIML service operation mode. E.g. stop the AIML service when latency is x or greater than x milliseconds |
| > Time limit-based service operation mode configuration | O | Indicates the time limit threshold values as a condition to for the requested AIML service operation mode. e.g. stop the AIML service after 100 ms. |
| > Performance-based service operation mode configuration | O | Indicates the performance as a condition for the requested AIML service operation mode. E.g. stop the AIML service when the performance metrics (e.g. model accuracy is 99% achieved) |
| AIML service operation mode status reporting configuration | O | Indicates the configuration to report status of the AIML service operation mode |
| > AIML service operation mode status report type | O | Indicates the reporting configuration like periodic or event based of the AIML service operation mode from the AIML clients |
| >AIML service operation mode periodic reporting | O See NOTE 2 | Indicates the time period-based reporting of the current service operation mode of the AIML service operation |
| >AIML service operation mode event-based reporting | O See NOTE 3 | Indicates the event-based reporting conditions of the AIML service operation state transition to/from any AIML service operation modes like transition to continue, pause, stop, |
| > AIML service operation mode status notification endpoint | O | Represents the endpoint (pair Address-Port or URL) for receiving AIMLE service operation mode status from AIMLE client. E.g. status report notification from AIMLE client transition from pause to continue |
| NOTE 1: One or combination of information elements can be applicable for the given AIML service operation mode. NOTE 2: This information element is only applicable when the AIML service operation mode status report type is periodic. NOTE 3: This information element is only applicable when the AIML service operation mode status report type is event-based. |  |  |

#### 8.x.3.2	AIML service lifecycle management response/update response

Table 8.x.3.2-1 shows the request sent by an AIML Enablement server to an AIML service consumer(e.g. VAL server) for the AIML service lifecycle management response or update response.

Table 8.x.3.1-1: AIML service lifecycle management response /update response

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g. AIML service consumer). |
| VAL service identifier | O | An identifier for the VAL service associated with the requestor. |
| AIML service operation ID | M | An identifier to identify the AIML service operation ID |
| AIML service operation mode status | M | Indicates the current state of AIMLE service operation. Possible values start, stop, continue, pause |

#### 8.x.3.3	AIML Enablement client service operation request

Table 8.x.3.3-1 shows the request sent by an AIML Enablement server to an AIML Enablement client for the AIML Enablement client service operation request.

Table 8.x.3.3-1: AIML Enablement client service operation request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g. AIML service consumer). |
| VAL service identifier | O | An identifier for the VAL service associated with the requestor. |
| AIMLE client identifier | M | Indicates the AIML enablement client identity |
| AIML service operation ID | M | An identifier to identify the AIML service operation ID |
| AIML service operation mode | O | Indicates the required AIMLE service operation modes like start, continue, pause, finish. |
| AIML service operation mode configuration | O See NOTE 1 | Indicates the configuration of the AIML service operation modes |
| > Compute Utilization-based service operation mode configuration | O | Indicates the compute utilization (memory, cpu, gpu) threshold values as a condition for the AIML service operation mode. E.g. stop the AIML service when memory is 70% utilization |
| > Network Utilization-based service operation mode configuration | O | Indicates the network utilization (bandwidth, coverage, latency) threshold values as a condition for the AIML service operation mode. E.g. stop the AIML service when latency is x or greater than x milliseconds |
| > Time limit-based service operation mode configuration | O | Indicates the time limit threshold values as a condition to for the AIML service operation mode. E.g. stop the AIML service after 100 ms. |
| > Performance-based service operation mode configuration | O | Indicates the performance as a condition for the the AIML service operation mode. E.g. stop the AIML service when the performance metrics (e.g. model accuracy is 99% achieved) |
| AIML service operation mode status reporting configuration | O | Indicates the configuration to report status of the AIML service operation mode |
| > AIML service operation mode status report type | O | Indicates the reporting configuration like periodic or event based of the AIML service operation mode from the AIML clients |
| >AIML service operation mode periodic reporting | O See NOTE 2 | Indicates the time period-based reporting of the current service operation mode of the AIML service operation |
| >AIML service operation mode event-based reporting | O See NOTE 3 | Indicates the event-based reporting conditions of the AIML service operation state transition to/from any AIML service operation modes like transition to continue, pause, stop, |
| > AIML service operation mode status notification endpoint | O | Represents the endpoint (pair Address-Port or URL) at the AIML service consumer for receiving AIMLEservice operation mode status. E.g. status report notification from when AIMLE client transition from pause to continue |
| NOTE 1: One or combination of information elements can be applicable for the given AIML service operation mode. NOTE 1: This information element is only applicable when the AIML service operation mode status report type is periodic. NOTE 2: This information element is only applicable when the AIML service operation mode status report type is event-based. |  |  |

#### 8.x.3.4	AIML Enablement client service operation response

Table 8.x.3.4-1 shows the request sent by an AIML Enablement client to an AIML Enablement server for the AIML Enablement client service operation response or update response.

Table 8.x.3.4-1: AIML Enablement client service operation response

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g. AIML service consumer). |
| VAL service identifier | O | An identifier for the VAL service associated with the requestor. |
| AIML service operation ID | M | An identifier to identify the AIML service operation ID |
| AIML service operation mode status | M | Indicates the current state of AIMLE service operation. Possible values start, stop, continue, pause |

* * * End of changes * * * *