3GPP TSG-SA WG6 Meeting #62	S6-243307

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-24abcd)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0577 | rev | - | Current version: | 19.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Client-triggered location reporting cancel procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If an authorized MC service user is interested in obtaining the location information of other MC service user based on the trigger conditions can use the existing procedure specified in the specification, where in authorized MC service user using location management client sends a location reporting trigger to the location management server to activate a location reporting procedure for obtaining the location information of another location management client. If an authorized MC service user is no longer interested in obtaining the location information of other MC service user based on the trigger should be able to deactivate a location reporting procedure by sending location reporting trigger cancel request to the location management server. Currently there are no procedure exists for location management client to send such cancelation request and to deactivate the location reporting procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new procedure is specified for the location management client to send a location reporting trigger cancel to the location management server to deactivate a location reporting procedure for obtaining the location information of another location management client. 10.9.2.4a: A new information flow “Location reporting trigger cancel” is defined. 10.9.3.4a: A new procedure is defined for the location management client for location reporting trigger cancelation 10.9.3.8.3: A new procedure is defined for the location management client for location reporting trigger cancelation, where in the target of the request is functional alias 10.9.3.4: Changing the heading to indicate that the procedure is for the Event-triggered and align with other procedure headings of Event-triggered 10.9.3.8.2: The NOTE 1 and NOTE 2 are updated to refer respective procedures when functional alias is newly activated or deactivated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The location management client will be reporting the location based on the trigger even if the requested location management client is no longer interested in obtaining the location, and also the location reporting trigger configurations are maintained unnecessarily. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9.2.4a (new), 10.9.3.4a (new), 10.9.3.8.3 (new), 10.9.3.4, 10.9.3.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.9.2.4a	Location reporting trigger cancel

Table 10.9.2.4a-1 describes the information flow from the location management client to the location management server to stop triggering a location reporting procedure.

Table 10.9.2.4a-1: Location reporting trigger cancel

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M (see NOTE 1) | Identity of the requesting authorized MC service user (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| Functional alias | O | Functional alias that corresponds to the requesting MC service user (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| MC service ID list | O (see NOTE 1) (see NOTE 2) | List of MC service users whose location information reporting is to be cancelled (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| Functional alias | O (see NOTE 2) | Functional alias that corresponds to the MC service users whose location information reporting is to be cancelled (e.g. MCPTT ID, MCVideo ID, MCData ID). |
| NOTE 1: The identity of the requesting MC service user and the requested MC service user shall belong to the same MC service. E.g. if requesting MC service user is using a MCPTT ID, then the requested MC service user identity shall be an MCPTT ID. NOTE 2: Either the MC service ID list or the Functional alias must be present. |  |  |

* * * * Next Change * * * *

#### 10.9.3.4a	Client-triggered location reporting cancel procedure

NOTE 1: This procedure is valid for single MC system operation only.

Figure 10.9.3.4a-1 illustrates the high level procedure of client-triggered location reporting cancel procedure.

Pre-conditions:

-	The location management client 1 has provided the location reporting trigger for the location management client 2 and location management client 3.

Figure 10.9.3.4a-1: Client-triggered location reporting cancel procedure

1.	Location management client 1 (authorized MC service user) sends a location reporting trigger cancel request to the location management server to deactivate a location reporting procedure for obtaining the location information of location management client 2 and location management client 3.

2.	Location management server checks whether location management client 1 is authorized to send a location reporting trigger cancel request for location management client 2's and location management client 3's location information. Depending on the location reporting configuration information, location management server may initiates a location reporting cancel procedure for the location management client 2 and location management client 3 as specified in the clause 10.9.3.4 or event-triggered location reporting procedure to update the configuration of the location management client 2 and location management client 3 as specified in the clause 10.9.3.1.

NOTE 2: The location management client (authorized MC service user) can cancel other location management clients (authorized MC service user) provided location reporting trigger for reporting location information.

* * * * Next Change * * * *

##### 10.9.3.8.3	Client-triggered periodic location information report cancellation

Figure 10.9.3.8.3-1 illustrates the procedure where a location management client requests to cancel the periodic location information from other location management clients for location information reporting using functional aliases which may be shared between several MC service users.

Pre-conditions:

1.	MC service users at location management client 2 and location management client 3 share the same functional alias.

2.	MC service users at location management client 2 and location management client 3 activated the functional alias.

3.	MC service user at location management client 1 may have an activated functional alias.

4.	The location management server has subscribed to the functional alias controlling MC service server within the MC system for functional alias activation/de-activation updates.

Figure 10.9.3.8.3-1: Periodic location information report cancellation for shared functional alias

1.	Location management client 1 (authorized MC service user) sends a location reporting trigger cancel request, limited to one MC service at the time, to the location management server to deactivate a periodic location reporting procedure which shall retrieve the location information of the MC service users sharing the contained functional alias. Location management client 1 may include its own activated functional alias.

2.	Location management server checks whether location management client 1 is authorized to send a location reporting trigger cancel request for location information of the location management clients that have activated the functional alias. The location management server determines the functional alias to a corresponding list of related MC service IDs of location management client 2 and location management client 3 who have activated the functional alias.

3a-3b.	Depending on the location reporting configuration information, location management server may initiates a location reporting cancel procedure for the location management client 2 and location management client 3 as specified in the clause 10.9.3.4 or event-triggered location reporting procedure to update the configuration of the location management client 2 and location management client 3 as specified in the clause 10.9.3.1.

* * * * Next Change * * * *

#### 10.9.3.4	Event-triggered location reporting cancel procedure

NOTE: This procedure is valid for single MC system operation only.

The event-triggered location reporting cancel procedure reuses the information flow of location reporting configuration.

Pre-conditions:

-	The location management client has been provided with location reporting configuration information.

Figure 10.9.3.4-1: Event-triggered location reporting cancel procedure

1.	The location management server sends location reporting configuration without any information element to the location management client to stop location reporting from the MC service UE.

2.	The location management client stops sending location information reports to the location management server.

* * * * Next Change * * * *

##### 10.9.3.8.2	Client-triggered periodic location information report

Figure 10.9.3.8.2-1 illustrates the procedure when a location management client requests periodic location information from other location management clients for location information reporting using functional aliases which may be shared between several MC service users. Under this condition, the actual location of all MC service users sharing the same functional alias are reported.

Pre-conditions:

1.	MC service client 2 and MC service client 3 share the same functional alias.

2.	MC service client 2 and MC service client 3 activated the functional alias.

3.	MC service client 1 may have an activated functional alias.

4.	The location management server has subscribed to the functional alias controlling MC service server within the MC system for functional alias activation/de-activation updates.

Figure 10.9.3.8.2-1: Periodic location information report for shared functional alias

1.	Location management client 1 sends a location reporting trigger, limited to one MC service at the time, to the location management server to activate a periodic location reporting procedure which shall retrieve the location information of the MC service users sharing the contained functional alias. Location management client 1 may include its own activated functional alias.

2.	Location management server checks whether location management client 1 is authorized to send a location reporting trigger for location information for location management clients that have activated the functional alias.

3.	Depending on the information given by the location reporting trigger, the location management server uses event-triggered location information procedure and immediately send location information request to the location management clients that contains the functional alias requested by the location management client 1.

4.	Upon receiving the reports, the location management server updates location of the reporting location management clients.

5.	The location management server checks whether location management client 1 is authorized to receive location information for all location management clients that have activated the functional alias.

6.	Based on the received location information reports, the location management server will periodically issue location report responses, one at a time for each location management client for which location management client 1 is authorized to receive location information, encompassing the MC service ID, the associated functional alias, the individual location information of the addressed MC service ID and the optional MC service UE label, if present.

NOTE 1:	If a functional alias is deactivated for an MC service client, the location management server deactivates location information reporting for this location management client using event-triggered location reporting cancel procedure as specified in the clause 10.9.3.4.

NOTE 2:	If a functional alias has been newly activated for an MC service client, the location management server activates location information reporting for this location management client using event-triggered location reporting procedure as specified in the clause 10.9.3.1.

NOTE 3:	If a functional alias is simultaneously shared between several MC service IDs, all location management clients with the associated MC service IDs will send the location report until the functional alias status change, e.g. take-over, for the individual MC service ID.

* * * * End of changes * * * *