3GPP TSG-SA WG6 Meeting #62	S6-243333

Maastricht, Netherlands 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	Nokia

Title:	New solution to Key Issue#5

Spec:	3GPP TR 23.700-22 v0.4.0

Agenda item:	8.7

Document for:	Approval

Contact:	Niranth (niranth.amogh@nokia.com)

1. Introduction

This contribution provides a proposal for New solution to Key Issue#5.

2. Reason for Change

In the TR 23.700-22, the following description is provided for key issue#5.

## 5.5 Key issue #5: Enhancing support to API Invoker on-boarding ### 5.5.1 Description Currently in 3GPP TS 23.222 clause 8.1 On-boarding of API Invoker to the CAPIF procedure, there is an assumption that API Invoker has sufficient API information to make decision of making an on-boarding request to CCF. However, in reality the API Provider exposes limited or only information that can be shared commonly to the API Invoker. The reason for this is beacause the trust relationship is yet to be established between the two parties (i.e. API Invoker and the API Provider). In another scenario, after the API Invoker successfully on boards to the CAPIF, the API Invoker realizes that certain features or services that the API Invoker wishes to consume with assistance of CAPIF, are not supported. Then the API Invoker may off board from the CAPIF or take certain other actions like not consuming northbound APIs via the registered CCF. In such a scenario, it is waste of resources for the API invoker and the CCF to perform the on boarding and maintain the API Invoker profile information. The features that API Invoker may be looking to consume from CCF and not supported by CCF, may include, the AEFs serving certain set of service API(s), availability of set of service APIs, support for certain security methods, support for certain security methods for certain AEFs / service APIs, interconnection with a given set of CCFs etc. To address these cases, it is essential to study enhancing support to API Invoker on-boarding to reduce unnecessary on-boarding and wastage of resources. ### 5.5.2 Open issues This key issue will study the following aspects: 1. How to enhance the support of API Invoker on-boarding to reduce unnecessary on-boarding and wastage of resources? 2. Any enhancements required to other CAPIF procedures e.g. Registering the API provider domain functions, Publish Service APIs to CCF?

Due to various considerations at CCF (e.g. expiry of secret) more events can be notified to the API invoker.

In TS 23.222, the following CAPIF events are specified

### 8.8.6 List of CAPIF events Table 8.8.6-1 provides a non-exhaustive list of CAPIF events. Table 8.8.6-1: List of CAPIF events Events; Events Description. Availability of service APIs; Availability events of service APIs (e.g. active, inactive). Service API updated; Events related to change in service API information. Monitoring service API invocations; Events corresponding to service API invocations. API invoker status; Events related to API invoker status in CAPIF (onboarded, offboarded). API topology hiding status; Events related to API topology hiding status in CAPIF (created, revoked). System related events; Alarm events providing fault information. Performance related events; Events related to system load conditions.

Enhancements to this list of notifications are proposed for the onboarded API invoker.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v0.4.0

* * * First Change * * * *

## 6.x	Solution #x: Enhanced CAPIF events for onboarded API invoker

### 6.x.1	Solution description

#### 6.x.1.1	General

This solution is related to key issue#5. Due to various considerations at CCF (e.g. expiry of secret) more events can be notified to the API invoker. New CAPIF events for onboarded API invoker are introduced based on the procedures in clause 8.8 of 3GPP TS 23.222 [2].

NOTE:	The considerations related to security for event notifications from CCF to API invoker require coordination with SA3.

#### 6.x.1.2	CAPIF events to support onboarded API invoker

The following updates (in bold) to clause 8.8.6 of 3GPP TS 23.222 [2] are proposed for supporting new events for onboarded API invoker.

### 8.8.6 List of CAPIF events Table 8.8.6-1 provides a non-exhaustive list of CAPIF events. Table 8.8.6-1: List of CAPIF events Events; Events Description. Availability of service APIs; Availability events of service APIs (e.g. active, inactive). Service API updated; Events related to change in service API information. Monitoring service API invocations; Events corresponding to service API invocations. API invoker status; Events related to API invoker status in CAPIF (onboarded, offboarded, secret expired, secret due to be expired). API topology hiding status; Events related to API topology hiding status in CAPIF (created, revoked). System related events; Alarm events providing fault information. Performance related events; Events related to system load conditions.

### 6.x.2	Architecture Impacts

It is based on existing CAPIF architecture specified in 3GPP TS 23.222 [2].

### 6.x.3	Corresponding APIs

The CAPIF_Events API specified in clause 10.4 of 3GPP TS 23.222 [2] will be enhanced to accommodate the new events related to onboarded API invoker.

### 6.x.4	Solution evaluation

This solution addresses the key issue#5 and proposes a viable technical solution for addressing new events for onboarded API invoker.