3GPP TSG-SA WG6 Meeting #62	S6-243343

Maastricht, Netherlands 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	Nokia

Title:	Updates to Spatial Map Creation

Spec:	3GPP TR 23.700-21 v1.1.0

Agenda item:	8.4

Document for:	Approval

Contact:	Niranth (niranth.amogh@nokia.com)

1. Introduction

This contribution provides a proposal for Updates to Spatial Map Creation

2. Reason for Change

In the TR 23.700-21, the following is the description for KI#4

## 4.4 Key issue #4: Spatial mapping ### 4.4.1 Description In clause 5.2.1 "Localized mobile metaverse service" of 3GPP TS 22.156 [2], Localized mobile metaverse services are immersive and integrated into a user's ordinary experiences. Such service experiences are location-related and can include presentation of AR, MR media. Users' localization is important in order to discover spatial anchors. The 5G system offers a spatial localization service to determine this information. [R-5.2.1-003] Subject to operator policy, regulatory requirements and user consent, the 5G system shall provide a means for a UE to provide sensor data, (e.g., from UE sensors, cameras, etc.) to the network in order to derive localization information, e.g., to produce or modify a spatial map or discover or find spatial anchors. The 5G system shall enable an authorized third party to obtain all the spatial anchors in a given three dimensional area. NOTE 1: How an authorized third party identifies which three-dimensional area to request spatial anchors in is not in scope of the 3GPP standard. Spatial localization and mapping information could be used to identify areas of interest. [R-5.2.1-004] Subject to operator policy and regulatory requirements, the 5G system shall support mechanisms to expose a spatial map or derived localization information to authorized third parties. SA6 already supports SEAL layer. SA6 can enhance the existing SEAL capability (like location management) to enable support for spatial mapping. ### 4.4.2 Open issues SA6 defined enablers as of now do not support operations required for spatial mapping. The open issues are: 1) How to produce or modify a spatial map by the consumer (e.g. UE, VAL server)? 2) How to expose a spatial map to authorized third parties?

In this proposal, an update is proposed to the solution#8 to include the layering information which can constitute spatial map. The layers in spatial map represent for example a layer with only basic space information, a layer with objects information. These layers help ease the management of the spatial map information as each layer is associated with different data.

This approach is similar to the HDMaps structure which consists of several layers as adopted by V2X vertical. Examples: https://www.researchgate.net/figure/The-features-and-layers-of-an-HD-map-130_fig4_364470302, https://www.researchgate.net/figure/HD-map-Layers-Source7_fig4_350104659. In SA6, we have defined mechanisms (e.g. SEAL LMS) in terms of locating UEs/Ojects which act as UEs to support HDMaps.

It is proposed that Spatial Maps SEAL service should enable Spatial map to be structurally defined as layers.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-21 v1.1.0

* * * First Change * * * *

## 7.8	Solution #8: Support for spatial map management

### 7.8.1	Solution description

This solution maps to KI#4. This solution enhances SEAL LM service to provide spatial map management service.

### 7.8.2	Architecture Impacts

This clause provides the architecture impacts of the solution. The solution provides enhancements to the SEAL LM functionalities for managing (i.e. produce, update, get, subscribe) spatial maps. To manage spatial maps between VAL server/SEAL LM client and SEAL LM server, reference points LM-S and LM-UU are used in this solution.

### 7.8.3	Procedures

#### 7.8.3.1	Producing spatial map

Figure 7.8.3.1-1 depicts the procedure for producing a spatial map. For the request from the spatial map requestor (VAL server or SEAL LM client), the service is provided by SEAL LM server.

Figure 7.8.3.1-1: Producing spatial map

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to produce the spatial map. The request includes requestor ID, three-dimensional area of interest, information to be included in the spatial map such as access control rules defining which entities are permitted to discover and access the spatial map and layering information parameters (e.g. layer 1-basic space, layer 2-objects).

NOTE 1:	The detailed information to be included in the spatial map, in the request message, will be further defined in the normative work phase.

The SEAL LM server authorizes VAL server (or SEAL LM client), and validates the request.

2)	The SEAL LM server produces a requested spatial map using the layering information and the processed sensor data required to estimate the number of objects, type of the object, position, direction and speed of the object, etc. obtained from various data sources (e.g. VAL server, VAL database or VAL UE) mapped to one or more layers.

NOTE 2:	In this study, for current release, some of the processed sensor data is stored at VAL layer (e.g. in database at VAL layer) and the SEAL LM server gets access to such database to produce spatial map via application specific way which is out of SA6.

3)	The SEAL LM server sends response message to the requestor with produced spatial map ID and information which includes three-dimensional space defined by the spatial map, stationary or moving objects with attributes of them and/or list of layers and its corresponding identities.

NOTE 3:	The detailed spatial map information in the response message will be further defined in the normative work phase. As an example, a list of spatial anchors which belong to the spatial map could be included.

NOTE 4:	The necessary of supporting third party triggered spatial map producing, in addition to spatial map service provider self-determined of map producing requires further discussion in normative phase.

#### 7.8.3.2	Update spatial map

Figure 7.8.3.2-1 dipicsts the procedure to update the spatial map. For the request from the spatial map consumer (VAL server or SEAL LM client), modifying or deleting the spatial map is provided by SEAL LM server.

Figure 7.8.3.2-1: Update spatial map

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to update a spatial map. The request includes requestor ID, spatial map ID, indicator to modify or delete, information on what to modify such as layering information parameters, updated access control rules and updated spatial map coverage area, etc. The SEAL LM server authorizes VAL server (or SEAL LM client) and validates the request.

2)	The SEAL LM server delete or modifies the spatial map as requested.

3)	Then it sends response message to the requestor with result of the request and modified spatial map information with the spatial map ID, list of layers and its corresponding identities.

#### 7.8.3.3	Get spatial map

Figure 7.8.3.3-1 dipicts the procedure for the authorized spatial map consumer (VAL server or SEAL LM client) to get the spatial map. The service is provided by SEAL LM server.

Figure 7.8.3.3-1: Get spatial map

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to get the spatial map. The request includes requestor ID, spatial map ID to get the information on it, three-dimensional area of interest or localization information to discover spatial maps in the area, optionally discovery filters, i.e. a set of characteristics to search the matching spatial maps when multiple spatial maps are allowed in the area of interest or the specific list of layers with corresponding identities.

NOTE:	How the consumer identifies which three-dimensional area of interest to get the spatial map is not in the scope of this study.

2)	The SEAL LM server authorizes VAL server (or SEAL LM client), and validates the request. Then the server collects the requested spatial map information. The SEAL LM server sends the get spatial map response message to the authorized third party consumer with spatial map ID and spatial map information when matching spatial map is found, otherwise it sends failure indication with reason.

#### 7.8.3.4	Subscribe/unsubscribe spatial map

Figure 7.8.3.4-1 dipicts the procedure for the authorized spatial map consumer (VAL server or SEAL LM client) to subscribe the spatial map. The service is provided by SEAL LM server.

Figure 7.8.3.4-1: Subscribe/unsubscribe spatial map

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to subscribe/unsubscribe spatial map(s). The request includes requestor ID, spatial map ID(s) or three-dimensional area of interest (or localization information) to identify the applicable spatial map(s), a list of events to subscribe/unsubscribe, which trigger the notification, with triggering criteria, minimum time between consecutive notifications, list of layers with its corresponding identities, etc. For the unsubscribe spatial map request, the subscription ID is included in the message.

2)	The SEAL LM server authorizes VAL server (or SEAL LM client), and validates the request. Then server add/delete the requestor into/from the subscriber list and event list of the spatial map, and sends response message with the result. For the subscribe spatial map request, the subscription ID is included in the message as well.

NOTE 1:	For the VAL server (or SEAL LM client) which unsubscribed all the events of the spatial map, the SEAL LM server removes the VAL server (or SEAL LM client) completely from the subscriber list of the spatial map. In this case, step 3) will not be initiated.

3)	When an event is detected, the SEAL LM server notifies the VAL servers (or SEAL LM clients) which subscribed the corresponding event. Spatial map notification message includes the information on the detected triggering event, e.g. added or removed objects, position or direction changes of the objects in the spatial map, added or updated or removed layer information, etc.

NOTE 2:	Further triggering events and triggering criteria etc. will be defined in the normative work phase.

#### 7.8.3.5	Spatial localization services

Figure 7.8.3.5-1 depicts the procedure for the authorized spatial map consumer (VAL server or SEAL LM client) to invoke spatial localization services. The service is provided by SEAL LM server.

Pre-condition:

1)	Spatial map is created and the authorized consumer is aware of the spatial map identity.

Figure 7.8.3.5-1: Spatial localization services

1)	The VAL server (or SEAL LMC) sends request to perform localization service in the spatial map to the SEAL LMS. The request includes identity of the map on which the action is to be performed.

Table 7.8.3.5-1: Spatial localization service request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., SEAL client, VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial Map Id | M | Identity of the spatial map |
| Area of interest | O | Contains three dimensional area within spatial map |
| Target user identity | O | Identity of the user to localize |

2)	The SEAL LMS authorizes the requestor (e.g. VAL server or SEAL LMC). If the requestor is not authorized then the SEAL LMS sends failure response. If the request is authorized:  the SEAL LMS identifies the target users along with its three dimensional localization in the spatial map. The SEAL LMS identifies the target user and also assigns unique identity within maps. If area of interest IE is present then the SEAL LMS identifies the target users within the area of interest only, otherwise, the SEAL LMS identifies the target users for whole map (according to the area associated with spatial map identifier).

3)	The SEAL LMS sends response to the VAL server (or SEAL LMC). The response includes (as shown in Table-2) the target user localization and identity.

Table 7.8.3.5-2: Spatial localization service response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the action |
| User details | O | A list of users |
| > User id | O | Identity of the User |
| > User localization | O | Current localization information of the user in the spatial map |

NOTE:	Whether and how localization of objects can be realized needs further discussions.

### 7.8.4	Corresponding APIs

#### 7.8.4.1	Produce spatial map

Table 7.8.4.1-1 shows the produce spatial map request sent by VAL server (or SEAL LM client) to SEAL LM server to produce a spatial map.

Table 7.8.4.1-1: Produce spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL LM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Area of interest | M | Three-dimensional area information to produce the spatial map |
| Requested information | O | Information to be included in the spatial map (e.g. layers information, access control rules) |

NOTE 1:	The requested information will be further defined during the normative work phase.

Table 7.8.4.1-2 shows the produce spatial map response sent by SEAL LM server to VAL server (or SEAL LM client) as the response to the produce spatial map request.

Table 7.8.4.1-2: Produce spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the producing spatial map (e.g., success or fail), including cause of failure if needed |
| Spatial map ID | O | ID of the produced spatial map |
| Spatial map information | O | If Spatial map ID is valid, a collection of information is included. The information corresponds to the produced spatial map including dimensional information, stationary or moving objects with attributes, e.g., object type, appearance, position, direction and speed of them or layers information with corresponding identities |

NOTE 2:	Details about Spatial map information is FFS and will be considered during the normative work phase.

#### 7.8.4.2	Update spatial map

Table 7.8.4.2-1 shows the update spatial map request sent by VAL server (or SEAL LM client) to SEAL LM server to update a spatial map.

Table 7.8.4.2-1: Update spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL LM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial map ID | M | ID of the spatial map to update |
| Instruction | M | Instruction to update or delete |
| Requested update information | O | Information on what to update/modify or delete, e.g. layers information, access control rules or spatial map coverage area, etc. |

NOTE 1:	The CRUD operations for spatial map management APIs will be enhanced in normative work.

NOTE 2:	The Requested update information will be further defined during the normative work phase.

Table 7.8.4.2-2 shows the update spatial map response sent by SEAL LM server to VAL server (or SEAL LM client) as the response to the update spatial map request.

Table 7.8.4.2-2: Update spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the producing spatial map (e.g., success or fail), including cause of failure if needed |
| Spatial map ID | M | If the Result is success, ID of the updated spatial map is included. |
| Spatial map information | O | If Spatial map ID is valid, updated spatial map information is included (e.g. layers information, access control rules or spatial map converage area). |

#### 7.8.4.3	Get spatial map

Table 7.8.4.3-1 shows the get spatial map request sent by VAL server (or SEAL LM client) to SEAL LM server to get  spatial maps.

Table 7.8.4.3-1: Get spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL LM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Filtering criteria | M | Criteria to get spatial map(s) |
| > Spatial map ID(s) | O | ID(s) of the spatial map to get information of it |
| > Area of interest | O | Three-dimensional area information to discover spatial maps |
| > Localization information | O | A known location to discover spatial maps in the three dimensional space, including an orientation |
| > Layers information | O | The layers information which is to be retrieved from the spatial map |
| > Discovery filters | O | A set of characteristics to search the matching spatial maps |

NOTE:	The Discovery filters will be further defined during the normative work phase.

Table 7.8.4.3-2 shows the get spatial map response sent by SEAL LM server to VAL server (or SEAL LM client) as the response to the get spatial map request.

Table 7.8.4.3-2: Get spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the producing spatial map (e.g., success or fail), including cause of failure if needed |
| List of discovered spatial maps | O | List of discovered spatial anchor(s). Each element includes the information described below |
| > Spatial map ID | M | ID of a spatial map discovered |
| > Spatial map information | M | A collection of information that corresponds to the discovered spatial map including dimensional information, stationary or moving objects with attributes, e.g., object type, appearance, position, direction and speed of them or layers information |

#### 7.8.4.4	Subscribe/unsubscribe spatial map

Table 7.8.4.4-1 shows the subscribe spatial map request sent by VAL server (or SEAL LM client) to SEAL LM server to subscribe spatial maps.

Table 7.8.4.4-1: Subscribe spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL LM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Target spatial map(s) | M | Spatial map(s) to subscribe |
| > List of spatial map IDs | O | ID(s) of the spatial map(s) to subscribe |
| > Area of interest | O | Three-dimensional area information to discover spatial map(s) to subscribe |
| > Localization information | O | Localization information of the UE/User to discover spatial map(s) which include the localized location in 3 dimensional space. |
| > Layers information | O | The layers information within the spatial map(s) to subscribe |
| List of events for discovered target spatial map(s) | M | Events to subscribe. Each element includes the information described below. |
| > Event type | O | Type of event to subscribe |
| > Event ID | O | ID of the event to subscribe |
| > Triggering criteria | O | Specifies the condition to determine whether the event is occurred, e.g., added or removed objects, position or direction changes of the objects in the spatial map etc. |
| > Notification intervals | O | Specifies when to notify the detected event, e.g., promptly or periodically with minimum time between consecutive notifications, etc. |

NOTE 1:	Details of how to define events, including by whom and what, will be specified during the normative work phase.

Table 7.8.4.4-2 shows the subscribe spatial map response sent by SEAL LM server to VAL server (or SEAL LM client) as the response to the subscribe spatial map request.

Table 7.8.4.4-2: Subscribe spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| List of results | M | List of the results for the subscribed events requested. Each element includes the information described below. |
| > Spatial map ID | M | ID of a spatial map to which requested events are belong |
| >> Event list subscribed | M | List of the events subscribed in a spatial map. Each element includes the information described below. |
| >>> Event type | O | Type of event subscribed |
| >>> Event ID | M | ID of the event subscribed |
| >>> Result | M | Result of the subscribe request for the event (e.g., success or fail), including cause of failure if needed. |
| >>> Subscription ID | O | Allocated ID for the successful subscription to the event |

Table 7.8.4.4-3 shows the unsubscribe spatial map request sent by VAL server (or SEAL LM client) to SEAL LM server to unsubscribe spatial maps.

Table 7.8.4.4-3: Unsubscribe spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL LM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Target spatial map(s) | M | Spatial map(s) to unsubscribe |
| > List of spatial map IDs | O | ID(s) of the spatial map(s) to unsubscribe |
| > Area of interest | O | Three-dimensional area information to discover spatial map(s) to unsubscribe |
| > Localization information | O | Localization information of the UE/User to discover spatial map(s) which include the localized location in 3 dimensional space. |
| > Layers information | O | The layers information within the spatial map(s) to unsubscribe |
| List of events for discovered target spatial map(s) | M | Events to unsubscribe. Each element includes the information described below. |
| > Event type | O | Type of event to unsubscribe |
| > Event ID | O | ID of the event to unsubscribe |
| > Subscription ID | O | Subscription ID to unsubscribe, which was allocated by the SEAL server when the requestor subscribed the event |

Table 7.8.4.4-4 shows the unsubscribe spatial map response sent by SEAL LM server to VAL server (or SEAL LM client) as the response to the subscribe/unsubscribe spatial map request.

Table 7.8.4.4-4: Unsubscribe spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| List of results | M | List of the results for the unsubscribed events requested. Each element includes the information described below. |
| > Spatial map ID | M | ID of a spatial map to which requested events are belong |
| >> Event list unsubscribed | M | List of the events unsubscribed in a spatial map. Each element includes the information described below. |
| >>> Event type | O | Type of event unsubscribed |
| >>> Event ID | M | ID of the event unsubscribed |
| >>> Result | M | Result of the unsubscribe request for the event (e.g., success or fail), including cause of failure if needed. |

Table 7.8.4.4-5 shows the spatial map notification sent by SEAL LM server to VAL server (or SEAL LM client) when a subscribed triggering event is detected.

Table 7.8.4.4-5: Notification

| Information element | Status | Description |
| --- | --- | --- |
| Spatial map ID | M | ID of the spatial map that detected the triggering event |
| Event ID | M | ID of the detected triggering event |
| Subscription ID | M | Subscription ID for the event |
| Notification information | M | Specifies what matches the triggering notification condition, including related information |

NOTE 2:	The Notification information will be further defined during the normative work phase.

### 7.8.5	Solution evaluation

This solution addresses KI#4. It provides spatial map management service procedures to publish, update/delete, get and subscribe to the spatial map. The solution is technically feasible for providing spatial map service. A new SEAL service and enhanced SEAL LM service are suitable architecture options to enable this solution. The architecture for this solution is yet to be determined. The solution will be adapted to the conclusion of the architecture evaluations.