3GPP TSG-SA WG6 Meeting #62	S6-243348

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	KPN N.V.

Title:	Solution 1: Resolve Editor’s notes

Spec:	3GPP TR 23700-01 v1.1.0

Agenda item:	8.6

Document for:	Approval

Contact:	Yonatan Shiferaw (yonatan.shiferaw@tno.nl), Anthony Pagès (anthony.pages@tno.nl)

1. Introduction

This contribution resolves the Editor’s notes in Solution 1.

2. Reason for Change

The previous Editor’s notes indicating that the Trajectory ID definition and its expected value in the AC Profile were 'Further For Study' are no longer relevant due to advancements in previous contributions. The Trajectory ID has been defined, while the concept of an Expected Trajectory ID is no longer applicable to this solution. Indeed, the Trajectory ID is defined in Solution 1 as a unique identifier assigned to each satellite and its orbit path in LEO or MEO. It is further explained in a note that the satellite operator or service provider are responsible for determining the trajectory, which can be calculated using instantaneous/osculating or TLE-based mean orbital ephemeris, and assigning the Trajectory ID. As a result, these notes have been removed.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23700-01 v1.1.0.

* * * First Change * * * *

## 7.2	Solutions for Application Enablers

### 7.2.1	Solution #AE1: Satellite edge computing

#### 7.2.1.1	Solution description

Editor's Note:	This clause will describe the solution. Each solution should clearly describe which of the key issues it covers and how.

##### 7.2.1.1.1	General

Figure 7.2.1.1.1-1: satellite EDN deployment

In this deployment option, ECS is deployed on ground. The UE can be on the ground/sea or in the air (e.g. drone). The  UPF to access satellite EDN is deployed on satellite, EAS and EES are deployed in one or more satellites. RAN (e.g. gNB) can either be deployed on ground/sea (e.g. in a ship) and connected to satellite UPF or be deployed on regenerative satellite (see Annex A and key issue #1 in 3GPP TR 23.700-29 [15]). The 5GS control plane functions (e.g. AMF, SMF) are deployed on the ground, which is not depicted in the figure for simplicity. EASs and their registered EES are deployed on one or more satellites which constitutes an EDN and these satellites’ coverage areas may correspond to an EDN service area. UPF can be deployed on ground to access the ECS, the EEC (in UE) can reach the ECS by EDGE-4 (on ground or via space).

If there are multiple EESs in a satellite EDN, EEC may trigger EAS discovery towards each EES which increases delay due to EDGE-1 interactions. It is recommended to deploy a single EES per EDN to reduce complexity in satellite.

NOTE:	The UPF and RAN deployment are described above for the sake of clarity which is related to EDN on-board satellite (e.g. EDGE-4 path), see also 3GPP TS 23.501 [5] clause 5.43.2 and 3GPP TR 23.700-29 [15]).

During initial service discovery, the EEC (in UE) contacts the ECS via EDGE-4 (on ground or via space) to find an appropriate EES, then an appropriate EAS instance is selected during EAS discovery via EDGE-1 interaction. Finally, AC (in UE) communicates with the selected EAS.

Editor's Note:	Other deployment options and associated latency aspects are FFS.

##### 7.2.1.1.2	Further consideration for service provisioning

An external server responsible for providing satellite availability information for each satellite based on real-time satellite orbit information (e.g. eccentricity, inclination, true anomaly) may exist (e.g. see 3GPP TS 23.501 [5], Annex Q). During service provisioning, the EEC can provide needed satellite capability information (e.g. satellite frequency bands) to the ECS and the ECS will be responsible to monitor UE location and UE moving prediction, and query the external server(s) to get a list of satellites (each identified by satellite ID) that can service the list of waypoints of the UE, then determine a suitable EES with a matching satellite ID to serve the UE with longest service time in service provisioning. Once the ECS identifies the suitable EES, it responds/notifies the EEC with the EES information. In addition, the corresponding ECS selected satellite ID is sent to the EEC.

##### 7.2.1.1.3	EAS discovery for EAS on board satellite

Unlike EAS discovery procedures EAS(s) on the ground, the EAS discovery for EAS(s) on board the satellite could be mobile depending on the satellite they are deployed on: GEO, MEO, or LEO. For GEO satellites, since the satellites are stationary relative to the Earth's surface, the EAS is also static. However, for MEO and LEO satellites, the EAS(s) are mobile as the satellites are moving relative to the Earth's surface. In this case, if the trajectory of the EAS is not considered for the discovery, frequent interruption, ACR, and/or service degradation may be experienced when there is a mismatch between the movement of the UE and the movement of the EAS on board satellite. This can be alleviated by considering the trajectory of the EAS on board satellite during the discovery. This solution is based on the assumption that the actual trajectory calculation for the satellite is performed by the satellite service provider and/or the operator. This trajectory calculation can be based on tlr-based mean orbital ephemeris, instantaneous/osculating ephemeris or any other means as per the satellite provider and/or operator. It is further assumed, the satellite provider and/or operator can also recommend a trajectory pattern that closely matches the planned route of the UE given the planned or expected route of a UE. The EEL does not need to deal with the actual trajectory calculation and only deals with trajectory ID that is provided by the satellite service provider and/or operator. This can be accomplished by adding mobility information and trajectory ID for the EAS in its profile.

1.	EAS mobility: to indicate the mobile EASs (EAS on board MEO or LEO satellite) and non-mobile EAS (EAS on board GEO satellite and EAS on-ground)

2.	Trajectory ID: to indicate the trajectory of the EAS on board MEO and LEO satellites. This trajectory ID is unique for each satellite and its trajectory.

NOTE 1:	It is up to the satellite service provider or operator to associate the Trajectory ID with the actual trajectory of the satellite. The trajectory and position of the satellite can be calculated based on instantaneous/osculating ephemeris versus using TLE-based mean orbital ephemeris.

NOTE 2:	The generation of the trajectory ID for the EAS(s) is done by the operator and/or the satellite service provider.

The Trajectory ID provided during the service provisioning response as per clause 7.2.1.1.4 can be provided by the EEC in the EAS discovery filters as per 3GPP TS 23.558 [10] clause 8.5.3.2 table 8.5.3.2-2. In case of EAS and EES deployed on the same satellite, the Trajectory ID provided matches the Trajectory ID of both EAS and the EES on board the satellite.

##### 7.2.1.1.4	Service provisioning for EES on board satellite

Similar to EAS on board a MEO or LEO satellite, the EES will also be mobile when the EES is also on board a MEO or LEO satellite. Therefore considering the trajectory of the EES is important for efficient service provisioning for selecting EDN matched with movement of the AC hosting UE during the AC’s operation schedule.

1.	Dynamic service area indication: Indicates if the service area is dynamic or not. The service area is dynamic in case of EES on board a MEO or LEO satellite due to EES mobility and not dynamic in case of a EES on board a GEO satellite or EES on ground.

2.	Trajectory ID: to indicate the trajectory of the EES on board MEO and LEO satellites. . This trajectory ID is unique for each satellite and its trajectory.

NOTE 1:	It is up to the satellite service provider or operator to associate the Trajectory ID with the actual trajectory of the satellite. The trajectory and position of the satellite can be calculated based on instantaneous/osculating ephemeris versus using TLE-based mean orbital ephemeris.

NOTE 2:	The generation and assignment of the trajectory ID for the EES(s) is done by the operator, the satellite service provider.

The "Expected AC Geographical Service Area" in the AC profile as in 3GPP TS 23.558 [10] clause 8.2.2 can be used by the ECS to request the external  server (by the satellite service provider or operator) with list of trajectory IDs that match the UEs planned route and its location. The ECS then uses the list of the trajectory ID for the UE to provision EES(s) by matching the trajectory IDs in the EES profile with the list of the trajectory ID determined for the UE according to its position and its planned route. Furthermore, the list of the trajectory IDs are provided in the service provisioning response to be used for further EAS discovery as per clause 7.2.1.1.3.

#### 7.2.1.2	Architecture Impacts

This solution has no architecture impacts to EDGEAPP. Under the existing EDGEAPP architecture, the service provisioning procedure is enhanced to support different cases in service continuity.

#### 7.2.1.3	Corresponding APIs

##### 7.2.1.3.1	EAS Profile, EES Profile and AC Profile for discovery and service provisioning

According to clause 7.2.1.1.3 EAS discovery for EAS onboard satellite and clause 7.2.1.1.4 Service provisioning for EES on board satellite, the following change is needed in 3GPP TS 23.558 [10].

NOTE:	The Trajectory ID of the EAS(s) and the EES(s) on the same satellite is identical.

-	3GPP TS 23.558 [10] Clause 8.2.4 EAS Profile EAS mobility on board LEO or MEO Satellite

3GPP TS 23.558 [10] Table 8.2.4-1: EAS Profile

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |
| EAS Type | O | The category or type of EAS (e.g. V2X, UAV, application enabler) |
| EAS description | O | Human-readable description of the EAS |
| … | … | … |
| Dynamic service area indication | O | Indicates if the service area is dynamic or not. The service area is dynamic in case of EAS on board a MEO or LEO satellite and not dynamic in case of a GEO satellite. |
| Trajectory ID (NOTE 1) | O | For mobile EAS, indicates an identifier for that relates to trajectory of the EAS on board a MEO and LEO satellites. |
| … NOTE 1: The assignment of the trajectory ID for EAS(s) can be done by the operator and/or the satellite service provider. The trajectory and position of the satellite can be calculated for example based on instantaneous/osculating ephemeris versus using TLE-based mean orbital ephemeris. |  |  |

-	3GPP TS 23.558 [10] Clause 8.2.6 EES Profile EES and EAS mobility on board LEO or MEO Satellite

3GPP TS 23.558 [10] Table 8.2.6-1: EES Profile

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | The identifier of the EES |
| EES Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |
| … | … | … |
| Dynamic service area indication | O | Indicates if the service area is dynamic or not. The service area is dynamic in case of EES on board a MEO or LEO satellite and not dynamic in case of a GEO satellite. |
| Trajectory ID (NOTE 2) | O | For mobile EES, indicates the trajectory of the EES on board MEO and LEO satellites. |
| … NOTE 2: The assignment of the trajectory ID for the EES can be done by the operator and/or the satellite service provider. The trajectory and position of the satellite can be calculated for example based on instantaneous/osculating ephemeris versus using TLE-based mean orbital ephemeris. |  |  |

-	3GPP TS 23.558 [10] Clause 8.5.3 EAS discovery request

3GPP TS 23.558 [10] Table 8.5.3.2-2: EAS discovery filters

| Information element | Status | Description |
| --- | --- | --- |
| List of AC characteristics (NOTE 1) | O | Describes the ACs for which a matching EAS is needed. |
| > AC profile (NOTE 2) | M | AC profile containing parameters used to determine matching EAS. AC profiles are further described in Table 8.2.2-1. |
| … | … | … |
| List of EAS characteristics (NOTE 1, NOTE 3) | O | Describes the characteristic of required EASs. |
| … | … | … |
| > Trajectory ID | O | Trajectory ID matching the trajectory of the UE which considers the current UE location as well as its planned route. |

#### 7.2.1.4	Solution evaluation

This solution is for KI#2 and addresses the open issue about deployment option for EDGEAPP and related enhancements to the EDGEAPP procedures including service provisioning and EAS discovery. The solution does not impact the architecture entity in EDGEAPP.

The solution proposes ECS is deployed on ground while EAS and EES are deployed on satellite. In this solution the EEC can be provide satellite capability information during service provisioning and then ECS will monitor UE location and UE moving prediction to determine and provision matching satellite ID to serve. In case of NGSO satellites, EEC contacts the ECS to find appropriate EES and EAS is discovered considering trajectory of the EES and EAS using Trajectory ID to match with the movement of the UE, to avoid frequent interruption, and/or service degradation. The Trajectory ID is defined as a unique identifier assigned to each satellite and its orbit path in Medium Earth Orbit (MEO) and Low Earth Orbit (LEO). Both EAS and EES systems utilize this ID. The satellite operator or service provider is responsible for determining the trajectory and assigning the Trajectory ID.

The solution is feasible.

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>