3GPP TSG-SA WG6 Meeting #62	S6-243350

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	KPN N.V.

Title:	Dynamic AIMLE client monitoring, discovery and selection subscription

Spec:	3GPP TS 23.482 V0.1.0

Agenda item:	9.10

Document for:	Approval

Contact:	Yonatan Shiferaw (yonatan.shiferaw@tno.nl)

1. Introduction

The conclusion for key issue #3 and key issue #7 in TR 23.700-82 stated that solutions #6 can be considered for normative work. The contribution introduces a procedure for the Dynamic AIML enablment client selection and monitoring subscription according to clause 8.6.1.2 and 8.6.3 of TR 23.700-82.

2. Reason for Change

In TR 23.700-82, Solution #6 addresses Key Issue #3 and Key Issue #7 and provides a selection mechanism for AIML enablement servers to select a group of AIML enablement clients for AIML operations and enforcment of AIML policies AIMLE clients selection and re-selection. Clause 8.6.1.2 allows the AIML enablement client discovery and selection to be performed in a dynamic manner in support of federated, split, and transfer learning workflows. The QoS application traffic session between selected AIML enablement client and requester can also be configured. This contributions brings this dynamic AIML client monitoring, discovery and selection subscription and notification procedure to the normative TS 23.482 according to the conclusion of the TR. The AIMLE client discovery/selection request-response is brought by another contribution S6-243229.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.1.0.

* * * First Change * * * *

## 8.x	Dynamic AIMLE client monitoring, discovery and selection subscription

### 8.x.1	General

Dynamic AIMLE client discovery and selection monitory subscription enables VAL Servers to subscribe for monitoring the AIMLE clients and receive notification when there is an update on the selected AI/ML client status such as re-selection according to AIML member selection and reselection policies and enforcement of QoS adjustment policies. Following the subscription, the AIMLE may also perform selection of AIML clients as per AIML selection policy, and may determine the QoS parameters and configure the AIML traffic session between the VAL Server and the selected AIML Enablment Clients.

For dynamic discovery and selection, the AIMLE server intracts with the NEF and/or SEAL services to monitor AIML members as per the AIML selection policy for the members who meet the conditions and obtain their identifiers, and configuring the AI/ML traffic session(s) between the VAL Server and the selected AI/ML Enablement Client(s) who meet the conditions. The AIML server also intracts with the NEF and/or SEAL services to monitor the selected AIML members and according to the AIML re-selection policies de-select the clients who do not fulfil the condition and select new ones that fulfil the conditions. When the AIMLE client is de-selected because it no longer meet the the conditions, the configured AIML traffic session is cancelled.

Instead of dynamic selection of the AIML members according to the required number of members, it is also possible to provide the AIML client set identifier is the selection has already been done according to Clause 8.x.2. The session IDs for the initial list of members may also be provided if the VAL server has already configured the AIML sessions between the VAL server and AIML clients.

The QoS adjustment policies are also enforced for those AIML members that get selected according the QoS adjustment policy conditions. The QoS adjustment is reversed and the monitoring of the AIML member when the the client is de-selected after finishing the data transfer between the de-selected member and newly selected member.

### 8.x.2	Procedure

Pre-conditions:

1.	The VAL server may have performed the AIML Enablement Client Selection with SEAL and 5GC support procedure as defined in clause 8.x.2 and has the AIML client set identifier.

2.	The VAL server may configure the AI/ML network sessions between VAL server and AI/ML Clients via SEALDD (Sdd_RegularTransmission API) or NEF services (AfSessionWithQoS API) and may have the list of session identifiers (e.g., traffic descriptors from SEALDD as defined in 3GPP TS 23.433 or Transaction Reference IDs as defined in 3GPP TS 23.502 [6]).

Figure 8.x.2.1-1: Selected AIML Enablement Client Status Monitoring Subscription

1.	A VAL server sends a request to an AI/ML Enablement Server to subscribe to the selected AI/ML Enablement Client status monitoring. The request shall include the requestor identifier, VAL service identifier, the selected AIML client set identifier or number of required AIML clients to be selected, notification endpoint for the selected AI/ML Enablement Client’s status update and may include list of the AI/ML session identifiers established by the requestor and list of AI/ML policy IDs for the AI/ML member re-selection.

2.	The AIML enablement server validates the selection request. The AIML enablement server further performs authentication and authorization checks to determine if the requestor is able to subscribe to the selected AI/ML Enablement Client status monitoring.

3.	[Optional] If the requestor is authorized and the list of the AI/ML session identifiers IE is not provided in step 1:

a.	If selected AIML client set identifier is not provided in step 1 and the number of required AIML clients to be selected is provided, then the AIML Enablement server monitors as per the AI/ML selection policy for the AI/ML members who meet the conditions and obtain their identifiers as described in step 4 in clause 8.x.2.

b.	The AI/ML Enablement Server may determine the QoS parameters for the AIML traffic session between the requestor and the selected AI/ML Enablement Client(s) and configure the AI/ML traffic session(s) via SEALDD (Sdd_RegularTransmission API) or NEF services (AfSessionWithQoS API).

4.	The AI/ML Enablement Server shall interact with the NEF and/or SEAL services (including SEALDD) to establish the associated monitoring subscriptions based on list of the AI/ML session identifiers provided in step 1 or established sessions in step 3. The AI/ML Enablement Server determines the relevant subscription procedures and the parameters for these subscriptions based on the inputs received from the AI/ML policy IDs.

5.	The AI/ML Enablement server shall response to the VAL server with the response for selected AIML enablement client status monitoring subscription.

6.	The AI/ML Enablement Server monitors the notifications of the established subscriptions in step 4 and based on the received notifications enforces the AI/ML policies which conditions are met, e.g.:

a.	if the conditions for the re-selection action within the member selection policy are met, the AI/ML Enablement Server shall initiate the re-selection procedure (as described in step 3.a) to select a new AI/ML Enablement Client as a selected member. The AI/ML Enabler Server may configure and initiate the AI/ML data transfer from the re-selected member to the selected member. When the AI/ML data transfer is finished or the selected members no longer match the conditions in the policy, the AI/ML Enablement Server shall cancel the related monitoring subscriptions configured in step 5b, release the established AI/ML sessions in step 3, and remove the enforced policies; and

b.	For the selected AI/ML members in step 6.a, if conditions for the QoS adjustment policy are met, the AI/ML Enablement Server shall generate and enforce the QoS policy(-ies) as per step 3 of clause 8.x.2. The AI/ML Enablement Server shall continue the QoS monitoring for the relevant AI/ML Enablement Client(s). If the AI/ML Enablement Client QoS adjustment conditions are no longer met the, the AI/ML member is de-selected and the QoS adjustment is reversed and the AI/ML Enablement Server shall initiate the re-selection procedure as described in step 6a.

7.	The AI/ML Enablement Server notifies the VAL server about the selected AI/ML client’s status update, e.g., the AI/ML Enablement Client A is re-selected and replaced by AI/ML Enablement Client B.

### 8.x.3	Information flows

Table 8.x.3-1 shows the request sent by a VAL server to an AIML enablement server for the Selected AIML Enablement Client Status Subscription.

Table 8.x.3-1: Request for AIML enablement client status monitoring subscription procedure

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| VAL service ID | O | An identifier for the VAL service. |
| AIML client set identifier | O (NOTE) | Represent the set of the AIML clients that events are subscribed to. |
| Number of the required AIML clients | O (NOTE) | Indicates the requested number of AIML clients to be selected based on member selection policies. |
| List of the AI/ML session identifiers | O | The list of session identifiers (e.g., traffic descriptors from SEALDD as defined in 3GPP TS 23.433 or Transaction Reference IDs as defined in 3GPP TS 23.502). |
| List of AI/ML policy IDs for the AI/ML member re-selection | O | Represents the list of policies that shall be enforced for the client re-selection procedure. |
| Notification endpoint for the selected AI/ML Enablement Client’s status update | M | Represents the endpoint at the VAL server for receiving the notifications on the selected AI/ML Enablement Client’s status update |
| NOTE: These information elements are mutually exclusive. |  |  |

Table 8.x.3-2 shows the response sent by the AIML enablement server to the VAL server for the selected AIML enablement client status monitoring subscription procedure.

Table 8.x.3-2: Response for AIML enablement client status monitoring subscription procedure

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |

Table 8.x.3-3 shows the notification sent by the AI/ML Enablement Server to the VAL server for the AIML enablement client selection status update.

Table 8.x.3-3: Notification for AIML enablement client status monitoring subscription procedure

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the requestor. |
| List of the selected AI/ML Enablement Client status update events | M | Represents the list of selected AI/ML Enablement Client status update events, e.g., the AI/ML Enablement Client A is re-selected and replaced by AI/ML Enablement Client B. |

* * * End of Changes * * * *