3GPP TSG-SA WG6 Meeting #62	S6-243359

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243269)

Source:	Apple

Title:	Pseudo-CR on a new solution for requesting user consent

Spec:	3GPP TR 23.700 v0.4.0

Agenda item:	8.7

Document for:	Approval

Contact:	Walter Featherstone w_featherstone@apple.com

1. Introduction

This pCR provides a new solution for requesting user consent.

2. Reason for Change

In Rel-18, the CAPIF-8 reference point was introduced between the CAPIF core function (CCF) and the resource owner function (ROF) to support:

- Providing authorization for resource access; and

- Managing and revoking the provided authorization.

However, specification of the functionalities over that reference point was not in scope of Rel-18 but are in scope of the Rel-19 study on CAPIF Phase 3. Specifically key issue 1 of the study considers managing resource owner consent, where open issue 1 of that KI is captured as “How consent of the resource owner can be managed through communication between the resource owner and authorization function in the CAPIF core function”.

To address open issue 1 of KI#1 a solution is proposed for requesting user consent that can reduce the number of interactions with the resource owner.

Of note, there are similarities with this solution to the solution for consent management proposed in contribution S6a240295. A significant difference is that rather than potentially waiting to aggregate consent requests from API invoker(s) before interacting with the ROF, it is proposed that consent is pre-emptively sought for multiple API invoker(s) and/or API exposing function(s) based on CCF configuration (assuming user consent is allowed to be maintained at the CCF). By obtaining user consent in a pre-emptive manner, the number of interactions with the resource owner via the ROF can be reduced.

An example use case for the solution includes pre-emptively seeking user consent in the case of a bundled EAS. With this solution, if an API invoker sends an obtain service API authorization request to the CCF for a service that is exposed by an EAS that is also configured as part of an EAS direct bundle (clause 4.11 of 3GPP TS 23.558) or may be in the future, then the CCF can pre-emptively seek user consent on behalf of the API invoker for all EASs in the bundle. In this manner, if at a future point the API invoker, wishes to utilise the services of the EASs in the bundle it can be provided with authorization to do so directly from the CCF without the CCF having to perform a further interaction with the resource owner. This can avoid the resource owner being contacted again for their consent, should they wish to utilise the services of an EAS bundle rather than just the initial EAS for which authorization was requested.

Another use case is that an application service provider (ASP) offers multiple services through various API exposing functions. In this scenario, the CCF can be configured to request user consent for additional services offered by the ASP at the point an API invoker requests authorization to use any of the ASP offered services.

Of note, it is proposed that the resource owner should be presented with the option to grant consent for only the specific API invoker triggering the request and then only for the specifically requested service and in this way the resource owner is provided with an alternative to agreeing to bulk consent.

4. Proposal

It is proposed to approve this pCR for 3GPP TR 23.700 v0.4.0.

* * * First Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  | X |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |
| Sol #5 |  |  |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  | X |
| Sol #7 |  |  |  |  |  |  | X |
| Sol #8 |  |  |  |  |  |  | X |
| Sol #9 | X |  |  |  |  |  |  |
| Sol #10 | X |  |  |  |  |  |  |
| Sol #11 | X |  |  |  |  |  |  |
| Sol #X | X |  |  |  |  |  |  |

* * * Second Change * * * *

## 6.X	Solution #X: Requesting user consent

### 6.X.1	Solution description

#### 6.X.1.1	General

This solution is for Key issue #1 on the issue of how the consent of the resource owner can be managed through communication between the resource owner and authorization function in the CAPIF core function (CCF).

3GPP TS 23.222 [2] provides a procedure in clause 8.31 for an API invoker to obtain authorization from resource owner. In the procedure it is the CCF authorization function that obtains the authorization by interacting with the resource owner via the resource owner function (ROF). The trigger for the interaction between the CCF and ROF to obtain such authorization is the API invoker request to obtain resource owner authorization information to invoke the service API exposed by the API exposing function.

In order to reduce the required number of interactions between the CCF and ROF, this solution enables the CCF authorization function to use such an API invoker request as a trigger to pre-emptively request for resource owner authorization information for additional API exposing functions and / or API invokers. The decision to make a request with a wider authorization scope is based on CCF configuration.

It is critical to highlight that any authorization response sent to an API invoker by the CCF will only include authorization information specific to that API invoker and for the specific service requested by that API invoker.

#### 6.X.1.2		Procedure for requesting user consent

Pre-condition:

1.	The API invoker is onboarded and has received an API invoker identity.

2.	The ROF can communicate with the CCF (Authorization function).

3.	The service API access requires obtaining consent from the resource owner.

Figure 6.X.1.2-1:	Procedure for resource owner consent revocation

1.	The CCF is configured for requesting bulk resource owner consent.

The bulk resource owner consent configuration includes lists of API invokers and associated AEF service APIs. The configuration indicates that should a request be received for one of the identified API invokers, resource owner consent should be sought for each of the associated AEF service APIs for all the API invokers in the list. For example, if the list includes API invoker 1 & 2 each with associated AEF service API service-A, resource owner consent should be requested for service-A for both API invoker 1 & 2 if either API invoker 1 or 2 makes a to service API authorization request to the CCF for service-A.

2. 	The API invoker sends an obtain service API authorization request to the CCF for obtaining permission to access the service API by including the API invoker identity information and any information required for authentication of the API invoker (reference step 1 of clause 8.11.3 of 3GPP TS 23.222 [2]).

3. 	The CCF validates the authentication of the API invoker (using authentication information) and checks whether the API invoker is permitted to access the requested service API (reference step 2 of clause 8.11.3 of 3GPP TS 23.222 [2]), which entails checking whether a consent record is already in place for the API invoker.

NOTE 1:	Whether the CCF is allowed to retain consent records depends on local regulations.

4.	If authorization information is not available regarding whether the API invoker is permitted to access the requested service API (i.e., there is no applicable consent record available at the CCF), the authorization function determines the authorization by interacting with the resource owner via the ROF. Based on the configured bulk resource owner consent information at the CCF, the scope of the request towards the ROF may be expanded to include additional API invokers and or AEFs, which is an enhancement to step 1 of clause 8.31.3 of 3GPP TS 23.222 [2]).

If authorization to access the service API for the API invoker the is already available at the CCF, i.e., through a previous interaction with the ROF, step 4 is skipped.

NOTE 2:	The detailed procedure to obtain the resource owner's authorization information is in scope of SA3. However, it is recommended that the resource owner is provided with at least the option to only grant authorization for only the API invoker initiating the authorization request and then only for the specific service being requested.

5.	Based on the RO provided consent, the authorization information (with scope that is specific to only that API invoker) to access the specifically requested service API is sent to the API invoker in the obtain service API authorization response (reference step 3 of clause 8.11.3 of 3GPP TS 23.222 [2]).

6.	The API invoker sends service API invocation request to the AEF with the resource owner authorization information received in step 5 (reference step 2 of clause 8.31.3 of 3GPP TS 23.222 [2]).

7.	The API invoker receives the service API invocation response resulting from the service API invocation once the AEF has checked whether the API invoker is authorized to invoke that service API based on the authorization information (reference step 3 of clause 8.31.3 of 3GPP TS 23.222 [2]).

### 6.X.2	Architecture Impacts

This solution is based on the existing architecture of CAPIF as described in 3GPP TS 23.222 [2] and relates to the CAPIF-8 reference point.

### 6.X.3	Corresponding APIs

There are no impacts anticipated for the existing obtain service API authorization request (or response) sent over CAPIF 1/1e.

This solution impacts the CAPIF-8 interface with new APIs.

NOTE: Security related API(s) will be addressed by SA3.

Editor’s note: Potential impacts to the reference points between the CCF and API provider domain functions, and their associated APIs, are FFS with regards to providing the bulk resource owner consent information configuration to the CCF.

### 6.X.4	Solution evaluation

This solution addresses open issue 1 of KI#1 and provides a mechanism for obtaining resource owner consent as part of the overall resource owner consent management solution that can reduce the required number of interactions between the CCF (Authorization Function) and ROF.

* * * Third Change * * * *

## 9.1	Evaluation of key issue#1

Clause 5.1 captures the key issue on managing resource owner consent. The following open issues are listed:

1.	How consent of the resource owner can be managed through communication between the resource owner and authorization function in the CAPIF core function

2.	Whether (and how) “purpose of data processing” will be captured and where will it be stored?

3.	How to align and manage access control that is more granular than simply granted/denied for service API (e.g., service operation level, resource level, service API originator/requestor details) with the provided resource owner consent to ensure appropriate usage of resource owner consent at the enabler layer.

Solution#3 specified in clause 6.4 addresses open issue#4 on finer granular access control for service API by enhancing the existing access control by introducing the resource level access.

Solution#Y specified in clause 6.Y addresses open issue#1 to manage consent of resource owner through communication between ROF and CCF (Authorization Function) using subscribe-notify mechanism for processing pending user consent requests related to one or more API invokers.

Solution #X specified in clause 6.X addresses open issue#1 on obtaining resource owner consent via the CAPIF-8 reference point, whereby a request for such consent may be on behalf of one or more API invokers and relate to one or more API exposing functions to reduce the required number of interactions between the CCF (Authorization Function) and resource owner via the ROF.

Editor's note:	Further evaluation of KI#1 considering the related solutions is FFS.

* * * End of Changes * * * *