3GPP TSG-SA WG6 Meeting #62	S6-243472

Maastricht, Netherlands 19th – 23rd August 2024	(revision of S6-243334)

Source:	Nokia

Title:	Updates to clause 7 on deployment scenarios

Spec:	3GPP TR 23.700-22 v0.4.0

Agenda item:	8.7

Document for:	Approval

Contact:	Niranth (niranth.amogh@nokia.com)

1. Introduction

This contribution provides a proposal for Updates to clause 7 on deployment scenarios.

2. Reason for Change

In the TS 23.222, the following description is specified for RNAA deployments.

## 7.5 RNAA deployments CAPIF supports RNAA and has enabled API invoker(s) to have authorized access to resources of a resource owner provided by service APIs offered by the AEF. The CCF acts as the Authorization Function and supports the authentication and authorization of the resource owner. Based on resource owner's authorization, the CCF provides the access token for a service API access to the API invoker. The API invoker performs service API invocations on the AEF by utilizing the access token. The API invoker may be deployed in the following ways: a. API invoker may be deployed as AF on the UE (i.e. 3rd party application). b. API invoker may be deployed as AF on the UE supporting several other 3rd party applications deployed on the UE. c. API invoker may be deployed on the network as AF. The resource owner is connected via a UE and can use a Resource Owner Function deployed on the UE to interact with the CCF acting as the Authorization Function for authentication and authorization i.e., granting permission to the API invoker to access resource(s) of the resource owner provided by the service API. NOTE: The details of the protocol for CAPIF supporting RNAA is specified in 3GPP TS 33.122 [12]. When API invoker is deployed on a UE (cases a and b), the API invoker is allowed to access the resources of the resource owner corresponding to the UE.

Additional deployments where API invoker in UE can now access resource not owned by that UE is to be added.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v0.4.0

* * * First Change * * * *

# 7	Deployment scenarios

## 7.1	General

This clause provides the additional or enhanced deployment scenarios for CAPIF as specified in clause 7 of 3GPP TS 23.222 [2].

## 7.x	Deployment model #1: Updates to RNAA deployments

The following updates (in bold) are proposed to the RNAA deployments specified in clause 7.5 of 3GPP TS 23.222 [2].

## 7.5 RNAA deployments CAPIF supports RNAA and has enabled API invoker(s) to have authorized access to resources of a resource owner provided by service APIs offered by the AEF. The CCF acts as the Authorization Function and supports the authentication and authorization of the resource owner. Based on resource owner's authorization, the CCF provides the access token for a service API access to the API invoker. The API invoker performs service API invocations on the AEF by utilizing the access token. The API invoker may be deployed in the following ways: a. API invoker may be deployed as AF on the UE (i.e. 3rd party application). b. API invoker may be deployed as AF on the UE supporting several other 3rd party applications deployed on the UE. c. API invoker may be deployed on the network as AF. The resource owner is connected via a UE and can use a Resource Owner Function deployed on the UE to interact with the CCF acting as the Authorization Function for authentication and authorization i.e., granting permission to the API invoker to access resource(s) of the resource owner provided by the service API. NOTE: The details of the protocol for CAPIF supporting RNAA is specified in 3GPP TS 33.122 [12]. When API invoker is deployed on a UE (cases a and b), with suitable authorization, the API invoker is allowed to access the resources of the resource owner corresponding to the UE. When API invoker is deployed on a UE (cases a and b), with suitable authorization, the API invoker is allowed to access the resources of other UEs.