3GPP TSG-SA WG6 Meeting #62	S6-243545

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243025)

Source:	China Mobile

Title:	Solve the ENs in sol#6

Spec:	3GPP TR 23.700-23

Agenda item:	8.5

Document for:	Approval

Contact:	Shaowen Zheng, zhengshaowen@chinamobile.com

1. Introduction

This paper proposes to solve the EN in Sol#6.

2. Reason for Change

It is proposed to solve the ENs.

For EN “Whether the implementation entity is XRApp or SELADD depends on the the Application enablement architecture.” it is discussed in the architecture to use option 2 defined in clause 6.1.3. So the the implementation entity is SELADD.

For EN “Whether and how to make use of existing CN functionality (e.g. PDU set related feature) is FFS.”, the PDU Set Information in RTP Header Extension (including the PDU Set Sequence Number, Indication of End PDU of the PDU Set) could be used to detect the target flows for Multi-modal flows alignment.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23.

* * * First Change * * * *

7.6	Solution #6: Multi-modal flows alignment and monitoring

7.6.1	Architecture Impacts

This solution requires a mechanism for data storage to cache the traffic flows.

7.6.2	Solution description

7.6.2.1	General

Synchronization between different multi-modal XR application components is crucial. In other words, the delay difference among the associated flows should be small. Otherwise, the arrived flows need to wait for other associated flows that do not arrived yet.

To avoid the downlink traffic flow delay difference introduced from (R)AN and Internet, the SEALDD client could do the multi-modal traffic flow alignment, by steering associated multi-modal traffic flows to be transferred to application client at specific time.

As shown in figure 7.6.2.1-1, flow#1 and flow#2 are associated flows. They are sent out from VAL server at the same time, while flow#1 arrives at SEALDD client earlier than flow#2 due to different routing path. To align those associated flows, the SEALDD client holds and buffers the flow#1 for a while. Until all the associated flows arrived(i.e., flow#2), the SEALDD client sends out those associated flows to application client at the same time.

Figure 7.6.2.1-1: DL traffic flow alignment

7.6.2.2	Procedure of flow alignment

Figure 7.6.2.2-1illustrates the procedures of multi-modal flows alignment.

Precondition:

-	The VAL server can discover and select the SEALDD server by CAPIF functions.

-	The connection has been established among the UE, SEALDD server and VAL server.

-	The SEALDD server has been provisioned to multi-modal XR policy including the synchronization threshold for multi-modal application, as specified in TS 22.261 [2].

SEALDD client

5GC

SEALDD server

1. Multi-modal flows coordination request

VAL server

5. Flows coordination

4. Multi-modal flows coordination response

Connection established

2. Multi-modal flows coordination request

3. Multi-modal flows coordination response

Application client

Figure 7.6.2.2-1: multi-modal flows alignment

- VAL server sends the multi-modal flows alignment request with VAL service ID, UE ID(s), flows descriptor,   Multi-modal Service ID, flows transmission requirement. Flows transmission requirement including the delay requirement, number of associated flows, maximum acceptable duration for traffic flow alignment. Maximum acceptable time duration for traffic flow alignment is used to limit the maximum waiting time for the associated flow.

2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server sends the multi-modal flows alignment request to the SEALDD client identified by the UE ID(s), along with the information received in step1, and the flows alignment assistance information (e.g. timestamp in the RTP header, RTCP).

If the maximum acceptable duration for traffic flow alignment is not provided, then the SEALDD server would determine the maximum acceptable duration for traffic flow alignment based on VAL service ID and/or flows transmission requirement. The server may translate the flows descriptor into SEALDD flow ID.

NOTE 1:	The flow alignment assistance information can be obtained by SEALDD server based on SEALDD policy.

NOTE 2: The SEALDD client performs the caching and transmission to align multi-modal flows based on the flow alignments assistance information and maximum acceptable time duration.

Editor's note: Whether and how to make use of existing CN functionality (e.g. PDU set related feature) is FFS.

NOTE 3: The NTP timestamp and RTP timestamp in RTCP sender report (SR) can be used to to identify the associated packet among muti-modal flow, and further be used to perform alignment in SEALDD client.

3.	The SEALDD client sends the multi-modal flows alignment response to the SEALDD server.

4.	The SEALDD server sends the multi-modal flows alignment response to the VAL server indicating the result of multi-modal flows alignment.

5.	When the traffic flow arrived, the SEALDD client initiate the multi-modal flows alignment based on the received information in step 1. The flows need to be aligned is detected by the VAL service ID, flows descriptor, and flow alignment assistance information.

If number of associated flows is provided, the number of associated flows is used to determine whether all the associated flows arrived. If the flow alignment assistance information is provided, the SEALDD client can identify the associated packets (e.g. with the same RTP timestamp) in the multi-modal flows. After all associated packet in multi-modal flows arrived, the SEALDD client sends the associated packets to the application client. If the maximum acceptable time duration is provided, once the maximum acceptable time is reached, the SEALDD client will no longer waits for the associated packets in multi-modal flows even if the associated packet is not arrived yet. The maximum acceptable time may be determined by the SEALDD server based on the transmission quality result for multi-modal flow and the synchronization threshold.

NOTE 4:	Step3 and step5 can be executed in parallel.

7.6.2.3	Procedure of flow alignment monitoring

Figure 7.6.2.3-1illustrates the procedure of multi-modal flows alignment monitoring.

Precondition:

-	The multi-modal flows alignment has been requested and triggered.

SEALDD client

5GC

SEALDD server

1. Multi-modal flows coordination monitoring request

VAL server

5. Flows coordination monitoring

7. Multi-modal flows coordination monitoring notification

Multi-modal flows alignment

2. Multi-modal flows coordination monitoring request

4. Multi-modal flows coordination monitoring response

6. Multi-modal flows coordination monitoring notification

3. Multi-modal flows coordination monitoring response

Figure 7.6.2.3-1: Traffic flow alignment monitoring

- VAL server sends the multi-modal flows alignment monitoring request. VAL service ID,UE ID, flows description, monitoring period are provided in the request.

- Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server sends the multi-modal flows alignment monitoring request to the SEALDD client.

- The SEALDD client sends the multi-modal flows alignment monitoring response to the SEALDD server.

4.	The SEALDD server sends the multi-modal flows alignment monitoring response to the VAL server, indicating whether the multi-modal flows alignment monitoring request is successful.

5.	The SEALDD client calculates the end-to-end delay difference based on the arrival time and timestamp(e.g., timestamp in the RTP header) among the flows.

Based on the flows description, delay difference for traffic flows identified needs to be calculated. The arrival time of first and last detected flow, T1 and T2, are recorded, and the delay difference is calculated based on T1 and T2.

6-7.SEALDD client sends the multi-modal flows alignment monitoring result to the SEALDD server, then to the VAL server.

7.6.3	Solution evaluation

This solution addresses Key issue #1and key issue #2: KPI measurement and exposure, and multi-modal communication. It proposes multi-modal flows alignment and monitoring to optimize the delay difference of multi-modal flows.

For multi-modal flows alignment, VAL server sends multi-modal flows alignment request to SEALDD client via SEALDD server, including VAL service ID, UE ID(s), flows description, flows transmission requirement and flows alignment assistance information. The SEALDD client utilizes these information to identifies multi-modal flows and perform the alignment.

For multi-modal flows monitoring, VAL server sends multi-modal flows alignment monitoring request to SEALDD client via SEALDD server, including VAL service ID, UE ID, flows description and monitoring period. The SEALDD client utilizes these information to identifies multi-modal flows and perform the monitoring.

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>