3GPP TSG-SA WG6 Meeting #62	S6-243547

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243026)

Source:	China Mobile, Huawei

Title:	Solve the ENs in sol#11

Spec:	3GPP TR 23.700-23

Agenda item:	8.5

Document for:	Approval

Contact:	Shaowen Zheng, zhengshaowen@chinamobile.com

1. Introduction

This paper proposes to solve the EN in Sol#11.

2. Reason for Change

It is proposed to solve the ENs.

For the “Editor’s Note: The relationship between the measurement methods and scenario is FFS.” and ”Editor’s Note: For the tethered link measurements between SEALDD client and XR client, the corresponding scenario is FFS.”

It is proposed to add the following clarifications. SEALDD and VAL coordination measurement is feasible on both types of tethered device (i.e.,Tethered XR device and Tethered display device), as long as the XR client on the tethered device has the agreement with the SEALDD client on the 3GPP UE to response the measurement request. SEALDD measurement is feasible on both types of tethered device (i.e.,Tethered XR device and Tethered display device), as long as the SEALDD client is installed on them.

For the Editor’s note: The detailed procedure for connection establishment between the 3GPP UE and tethered device is FFS,

A detailed procedure for Connection establishment between the 3GPP UE and tethered device is proposed.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23.

* * * First Change * * * *

7.11	Solution #11: Support the tethered UE based on PINAPP

7.11.1	Architecture Impacts

As discussed in TR 26.806 [8], there are three types of tethered UE, Tethered Standalone AR Glasses, Tethered Display AR Glasses, and Tethered AR Glasses with 5G Relay. To focus more on the interaction with applications, the architecture of the application enablement layer can be divided into the following two types based on the deployment of the XR application. One is the tethered XR device, where XR application client is deployed on the tethered XR device, i.e., Tethered Standalone AR Glasses, and Tethered AR Glasses with 5G Relay. The other is the Tethered Display XR device, who deploys the XR application on the 5G client.

Usually the tethered XR device is none-3GPP device so it should connecting to the 5G UE to access the 3GPP network. Considering the PEGC is a UE with subscription data related to PIN within the 5GS, so the 5G UE could be the  PEGC and PEMC, the tethered XR device can act as a PINE.

Then the proposed application layer architecture for the tethered XR device is shown in Figure 7.11.1-1. The XR application client runs on the tethered XR device. The tethered XR device is tethered to a 3GPP UE and potentially uses the PINAPP capabilities to support the XR application, e.g., 5G UE acting as the PEGC and PEMC could be enhanced to provide the measurement service.

Figure 7.11.1-1. Application enablement architecture based on PINAPP to support Tethered XR device

SEALDD could utilize PINAPP’s PIN management capability to do the tethered device discovery, then to do the tethering link measurement of the tethered XR device. Some enhancement is needed to support the interaction between the SEALDD client and XR client on different device over SEALDD-C. As shown in the figure, the SEALDD client on the 3GPP UE, acting as PIN client, could get tethered device information from the PEMC using the PIN-3 interface.

Figure 7.11.1-2. Application enablement architecture based on SEALDD and PINAPP to support Tethered XR device

For the Tethered Display device, the proposed application layer architecture is shown in Figure 7.11.1-2. The 3GPP UE includes the XR application and functions. The Tethered Display device is connected to the 3GPP UE for displaying. The PIN client is installed on the Tethered Display device to measure and manage the media transmission.

Figure 7.11.1-3. Application enablement architecture based on PINAPP to support Tethered display device

SEALDD could utilize PINAPP’s PIN management capability to do the tethered device discovery, then to do the tethering link measurement, with some architecture enhancement. As shown in the figure, the SEALDD client on the 3GPP UE, acting as PIN client, could get tethered device information from the PEMC using the PIN-3 interface. The SEALDD-UUc interface needs to be defined to support the interaction between the SEALDD client on the tethered UE and SEALDD client on the 3GPP UE.

Figure 7.11.1-4. Application enablement architecture based on SEALDD and PINAPP to support Tethered display device

7.11.2	Solution description

With the above architecture discussion, there are two ways to do the tethering link measurement. One is SEALDD and VAL coordination measurement based on PIN, the other is the SEALDD measurement based on PIN. The SEALDD and VAL coordination measurement is based on assumption that there is business agreement between the VAL provider and the SEALDD provider, which request the VAL client to respond to ICMP Ping packet. The SEALDD layer based measurement is based on the SEALDD client installed on tethered and 3GPP devices.

7.11.2.1	SEALDD and VAL coordination measurement based on PIN

Figure 7.11.2.1-1 illustrates the procedure of SEALDD and VAL coordination measurement based on PIN.

Pre-conditions:

1.	The UE or PINE has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server;

2.	The UE or PINE has already been registered in PIN server;

3.	The PEMC on the 3GPP device has already get the tethered device information from PIN server;

4.	There is business agreement between the VAL provider and the SEALDD provider, which request the VAL client to respond to ICMP Ping packet.

Figure 7.11.2.1-1: SEALDD and VAL coordination measurement based on PIN procedure

1.	The PIN is successfully created and in use. The 3GPP UE acting as PEGC and PEMC, the Tethered XR devices acting as a PINE. During the PIN creation, the detail information of the tethered device has been provide to PEMC on 3GPP UE from PINAPP server as defined in clause 8.5.2, TS 23.542[19].

2.	The regular data transmission connection is established, with the information received in step1.

The connection between the SEALDD server and 3GPP UE is established as defined in clause 9.2.2.2, TS 23.433.

- The VAL server sends a SEALDD transmission quality measurement subscription request to the SEALDD server. Optionally, the request contains the tethered device information. If authorization is successful, the SEALDD server sends a response to the VAL server with the subscription ID, expiration time.

- The SEALDD transmission quality measurement as defined in clause 9.7.2.1, TS 23.433.

- The SEALDD client gets the information of the tethered device from the PEMC over PIN-1, e.g., the MAC address, PINE Address, Port number.

Editor’s note: The specific service and API supporting the PINE discovery needs to be defined in PINAPP.

- The SEALDD client on the 3GPP UE interacts with XR client on the tethered UE to do the measurement based on ICMP ping protocol.

- The SEALDD client sends the report to the SEALDD server.

- The SEALDD server reports the data transmission quality measurement results (e.g. latency, jitter, bitrate, packet loss rate) to the VAL server via the notification message.

7.11.2.2	SEALDD measurement based on PIN

Figure 7.11.2.2-1 illustrates the procedure of SEALDD measurement based on PIN.

Pre-conditions:

1.	The UE or PINE has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server;

2.	The UE or PINE has already been registered in PIN server;

3.	The PEMC on the 3GPP device has already get the tethered device information from PIN server;

Figure 7.11.2.2-1: SEALDD measurement based on PIN procedure

1.	The PIN is successfully created and in use. The 3GPP UE acting as PEGC and PEMC, the Tethered XR devices acting as a PINE. During the PIN creation, the detail information of the tethered device has been provide to PEMC on 3GPP UE from PINAPP server as defined in clause 8.5.2, TS 23.542[19].

2.	The regular data transmission connection is established, with the information received in step1.

The connection between the SEALDD server and 3GPP UE is established as defined in clause 9.2.2.2, TS 23.433.

Optionally, the SEALDD client gets the tethered device information form PEMC(e.g., the MAC address, PINE Address, Port number.) and sends the direct data transmission connection request to the tethered UE using the information form PEMC to establish a connection between the 3GPP UE and tethered device as defined in 7.11.2.3.

- The VAL server sends a SEALDD transmission quality measurement subscription request to the SEALDD server. Optionally, the request contains the tethered device information. If authorization is successful, the SEALDD server sends a response to the VAL server with the subscription ID, expiration time.

- The SEALDD transmission quality measurement as defined in clause 9.7.2.1, TS 23.433.

- The SEALDD client gets the information of the tethered device from the PEMC over PIN-1, e.g., the MAC address, PINE Address, Port number.

Editor’s note: The specific service and API supporting the PINE discovery needs to be defined in PINAPP.

- The SEALDD client on the 3GPP UE interacts with SEALDD client on the tethered UE to do the measurement based on ICMP ping protocol, or using monitoring packet in established tethering link SEALDD connection.

- The SEALDD client sends the report to the SEALDD server.

- The SEALDD server reports the data transmission quality measurement results (e.g. latency, jitter, bitrate, packet loss rate) to the VAL server via the notification message.

7.11.2.3	Connection establishment between the SEALDD client on 3GPP UE and tethered device

Depicted in figure 7.11.2.3-1 is the procedure for establishment of direct data transmission connection between the SEALDD client on 3GPP UE and tethered device.

Figure 7.11.2.3-1: Establishment of Direct Connection between the SEALDD client on 3GPP UE and tethered device

1.	SEALDD client on 3GPP UE sends a Direct Connection Request to SEALDD client on tethered UE. This message includes the UE identity, SEALDD-UUc Data transmission connection information(e.g., address/port allocated).

2.	SEALDD client on Tethered UE initiates the procedure for mutual authentication. The successful completion of the authentication procedure completes the establishment of the Direct Connection.

7.11.3	Solution evaluation

This solution addresses KI#5 by to utilizing PINAPP to do the tethered device discovery, the SEALDD to do the traffic transmission and measurement (including tethered device link). For SEALDD and VAL coordination measurement, the SEALDD client on the 3GPP UE interacts with XR client on the tethered UE to do the measurement based on ICMP ping protocol. For SEALDD measurement, the SEALDD client on the 3GPP UE interacts with SEALDD client on the tethered UE to do the measurement based on ICMP ping protocol, or using monitoring packet in established tethering link SEALDD connection.

* * * Next Change * * * *

* * * Next Change * * * *

<Proposed change in revision marks>