3GPP TSG-SA WG6 Meeting #62	S6-243551

Maastricht, The Netherlands, 19th – 23rd August 2024	(revision of S6-243207)

Source:	Ericsson

Title:	Support Real-Time XR Service with Available 5G Network Information

Spec:	3GPP TR 23.700-23 V1.1.0

Agenda item:	8.5

Document for:	Approval

Contact:	Jing Yue (jing.yue@ericsson.com)

1. Introduction

As described in Key Issue #7, the study aspects on E2E KPI optimization for XR service include:

- How to support the E2E KPI optimization to meet real-time XR service requirements, taking the processing delay of XR application server into account, e.g. considering the potential enhancements to EDGEAPP in XR application server discovery or instantiation.

- What available information from 5G network can be used by enable layer to optimize the real-time XR service requirements.

Solve the E2E KPI for real-time XR service consisting of reduce transmission delay in 5G network and reduce processing delay in XR server, this paper focuses on the aspect of transmission delay. The information from 5G NFs, e.g. analytics from NWDAF for E2E data volume transfer time, service from NEF for setting up an AF session with required QoS, etc., can be used to support real-time XR service on e.g. XR data deliver.

This paper proposes to use the available information from 5G network to support real-time XR service.

2. Reason for Change

The information from 5G network, e.g. analytics from NWDAF, service from can be used to support real-time XR service on e.g. XR data delivery. How to obtain and use of the informaiton from 5G network to support real-time XR service need be studied.

3. Conclusions

This pCR proposes to use the 5G network information to support real-time XR service.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23 V1.1.0.

* * * First Change * * * *

## 7.0	Mapping of solutions to key issues

Table 7.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 | KI #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  | X |  |  |  |  |  |  |
| Sol #2 |  | X |  |  |  |  |  |  |
| Sol #3 |  | X |  |  |  |  |  |  |
| Sol #4 | X |  |  |  | X |  |  | X |
| Sol #5 |  | X |  |  |  |  |  |  |
| Sol #6 | X |  |  |  |  |  |  |  |
| Sol #7 |  |  |  |  |  |  |  | X |
| Sol #8 |  |  |  |  |  |  | X |  |
| Sol #9 |  |  |  |  | X |  |  |  |
| Sol #10 |  |  |  |  |  | X |  |  |
| Sol #11 |  |  |  |  | X |  |  |  |
| Sol #12 |  |  |  |  |  | X |  |  |
| Sol #13 |  |  |  |  |  | X |  |  |
| Sol #Y |  |  |  |  |  |  | X |  |

* * * Next Change * * * *

## 7.Y	Solution #Y: Support Real-Time XR Service with Available 5G Network Information

### 7.Y.1	General

The information from 5G NFs, e.g. analytics from NWDAF for E2E data volume transfer time, service from NEF for setting up an AF session with required QoS or creating multi-member AF session with required QoS, etc., can be used to support real-time XR service on e.g. XR data delivery.

The following clauses specify procedures, information flows and APIs for usage of 5G network information to support real-time XR service.

### 7.Y.2	Procedure

Pre-conditions:

-	Consumer decides real-time XR service, and real-time XR data delivery is needed to support the real-time XR service,

-	The 5G network (e.g. 5GC NFs) are known by the SEALDD Server, and the informaiton (e.g. analytics from NWDAF, service from NEF) is available for SEALDD Server.

Figure 7.Y.2-1: Support real-time XR service on real-time XR data delivery

- The consumer (e.g. VAL server, VAL client via SEALDD/XRApp client) sends request to SEALDD server for real-time XR data delivery (to single endpoint or to multiple endpoints). The request message contains message as defined in table 7.Y.3.2-1. Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful or the service request is rejected (e.g. lack of data size information in the request), the SEALDD server responses to the VAL server.

NOTE 1: The procedure in clause 9.2.2.2 of 3GPP TS 23.433[6] can be reused with enhancement to support the request for XR data delivery in step 1. The potential enhancement to the procedure will be discussed during normative phase.

- The SEALDD server derives information for the request, for example, based on the message in the request to determine downstream entities and services needed based on the information derived. For example, analytics from NWDAF (e.g. analytics on E2E data volume transfer time, UE mobility), service from NEF (e.g. set up an AF session with required QoS).

- The SEALDD server requests/subscribes to NWDAF (or via NEF) for analytics on e.g. E2E data volume transfer time (e.g. predictions of E2E data volume transfer time for uplink/downlink given data volume) and/or UE mobility (e.g. predictions of UE location) as defined in 3GPP TS 23.288[16].

- According to the received analytics from NWDAF (or via NEF), the SEALDD server sets (to create AF session(s))/updates (to modify the AF session(s)) the configuration of session(s) between UE(s) and AF for XR data delivery for satisfy the real-time requirements. For example, the predictions of E2E data volume transfer time for uplink/downlink given data volume can be used by the SEALDD server to set the individual QoS parameters (e.g. Requested 5GS Delay) and data volume, the prediction of UE location can be used to set the requirement on QoS monitoring (e.g. area of interest). In the request from SEALDD server to NWDAF for E2E data volume transfer time analytics, a pre-configured data size may be used (the pre-configured data size may be the data size provided in step 1, or be decided by the SEALDD server according to the data size range in step 1 and based on its local policy). The analytics subsceiption may be updated when the actual data size changed (the actual data size may be obtained via measurement described in Sdd_TransmissionQuality Management API).

- The SEALDD server requests service from NEF(s), e.g. set up an AF session with required QoS for XR data delivery to single endpoint as defined in clause 4.15.6.6 of 3GPP TS 23.502[4], or set up multiple AF sessions with required QoS for XR data delivery to multiple endpoints for XR data delivery. In the request may also contain QoS monitoring requirements (e.g. UE address, UL/DL packet delay).

NOTE 2:	In step 5, set up multiple AF sessions with required QoS can be done by performing the set up of AF session with required QoS multiple times in parallel.

NOTE 3:	The QoS monitoring service from NRM as defined in 3GPP TS 23.434 [12] with enhancement can also be considered to use in step 5. The potential enhancement to NRM QoS monitoring service can be discussed during normative phase.

NOTE 4:	In steps 3 and 5, the existing services from NWDAF and/or NEF are reused and without any impact to these 5GC NFs.

- The SEALDD server triggers to establish connection among the consumer, SEALDD server, SEALDD/XRApp client(s), and VAL client(s) and performs the XR data delivery. The SEALDD server may modify the connection according to the response from NEF on set up/update of AF session(s) and/or the QoS monitoring notification in step 5, that is the SEALDD server may adjust its configuration of session(s) between UE(s) and AF for XR data delivery. For example, if QoS degradation monitored from step 5, the SEALDD server adjusts it configuration to guarantee the required QoS in step 1 be satisfied.

NOTE 5:	The connection establishment and data delivery procedures could use the existing solutions as introduced in this TR and the existing mechinisms provided in 3GPP TS 23.433 [6].

The steps 3b-6 may repeat until the XR data delivery complete. The SEALDD server may receive updated analytics from NWDAF and/or QoS monitoring notification during the procedure. The SEALDD server may keep adjusting the configuration according to the updates.

### 7.Y.3	Information flows

#### 7.Y.3.1	General

The following information flows are specified for supporting real-time XR data delivery request based on clause 7.Y.2.

#### 7.Y.3.2	Supporting real-time XR data delivery request

Table 7.Y.3.2-1 describes the information flow from the consumer (e.g. VAL Server) as a request or update request for the XR service.

Table 7.Y.3.2-1: Supporting real-time XR data delivery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M (NOTE) | The identifier of the consumer (e.g. VAL server ID, SEALDD/XRApp client ID, VAL client ID). |
| VAL service ID | O | Identity of the VAL service. |
| VAL UE ID(s) | O | Identifier(s) of the endpoint(s) (i.e. VAL UE(s)) for the XR service. |
| Data delivery requirements | M | The QoS requirement, e.g. latency for UL/DL data delivery. |
| >VAL UE ID | M | Identifier of the endpoint (i.e. VAL UE) for the XR service. |
| >Data information | M | The information of the data needs to be delivered for the XR service. |
| >>Data type | M | The type of data, could be e.g. image, video, multi-model data (e.g. audio, video, positioning, haptic data). |
| >>Data size information | M | The maximum/minimum/average size of the data or the range of size for the data to be delivered. |
| >QoS requirement | M | The QoS requirement, e.g. latency for UL/DL data delivery to the VAL UE. |
| NOTE: This information element shall not be updated. |  |  |

NOTE:	Whether contains the IE data size will be discussed during normative phase.

#### 7.Y.3.3	Supporting real-time XR data delivery response

Table 7.Y.3.3-1 describes the information elements for the supporting real-time XR data delivery response from the SEALDD server to the consumer.

Table 7.Y.3.3-1: Supporting real-time XR data delivery response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the XR service request (positive or negative acknowledgement). |

### 7.Y.4	Solution evaluation

This solution addresses KI#7 and introcudes procedure on utilize of the information provided by 5GC NFs to support real-time XR service on real-time XR data delivery, for the following aspects:

- What available information from 5G network can be used by enable layer to optimize the real-time XR service requirements.

# *** End of Changes ***