3GPP TSG-SA WG6 Meeting #62	S6-243588

Maastricht, Netherlands 19th – 23rd August 2024	(revision of 3340, merged 3322)

Source:	Nokia, Samsung

Title:	Overall evaluation and conclusion of KI#5

Spec:	3GPP TR 23.700-22 v0.4.0

Agenda item:	8.7

Document for:	Approval

Contact:	Niranth (niranth.amogh@nokia.com)

1. Introduction

This contribution provides a proposal for Overall evaluation and conclusion of KI#5.

2. Reason for Change

In TR 23.700-22 v0.4.0, to address KI#5, following are the solutions – Sol#5 and Sol#x (S6-243468)

The overall evaluation and conclusion for KI#5 is proposed.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v0.4.0.

* * * First Change * * * *

## 9.x	Evaluation of key issue#5

Clause 5.5 captures the key issue on Enhancing support to API Invoker on-boarding. The following open issues are listed:

1.	How to enhance the support of API Invoker on-boarding to reduce unnecessary on-boarding and wastage of resources?

2.	Any enhancements required to other CAPIF procedures e.g. Registering the API provider domain functions, Publish Service APIs to CCF?

Solution#5 specified in clause 6.6 provides a proposal to address open issue#1 and open issue#2 by enhancing the API invoker onboarding mechanism. Further, the solution proposes CCF sending notification to the API invoker when CCF cannot meet the onboarding criteria request from the API invoker.

Solution#x specified in clause 6.x provides a proposal to address open issue#1 and open issue#2 by re-using existing CAPIF event notification procedures and introducing new events to support the onboarded API invokers. The events related to security are to be coordinated with SA3.

* * * Next Change * * * *

## 10.x	Conclusions of key issue #5

Solution#5 and Solution#x can be considered as candidate for normative work for addressing the CAPIF event notifications for API invoker onboarding. The events related to security are to be coordinated with SA3.

Solution#5 can be considered as candidate for normative work for enhancements related to API invoker onboarding.