3GPP TSG-SA WG6 Meeting #62	S6-243589

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243170)

Source:	Lenovo, China Mobile

Title:	Architecture

Spec:	3GPP TS 23.482 v0.1.0

Agenda item:	9.10

Document for:	Approval

Contact:	Emmanouil Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This paper provides the architecture clause, based on the concluded architecture in TR 23.700-82 clause 7.

2. Reason for Change

This paper provides the architecture clause, based on the concluded architecture in TR 23.700-82 clause 7.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 v0.1.0.

* * * First Change * * * *

# 5	Application architecture for enabling AI/ML services

## 5.1	General

The functional architecture enhancements for the AIML enablement service are based on the generic functional model specified in clause 6.2 of 3GPP TS 23.434 [5]. The architecture enhancements are organized into functional entities to describe a functional architecture enhancement which addresses the support for AIML enablement aspects for vertical applications.

## 5.2	Application enablement architecture

### 5.2.1	On-Network AIML Enablement (AIMLE) Functional Architecture

Figure 5.2.1-1: On-network AIMLE functional model

Figure 5.2.1-1 illustrates the on-network functional model of AIML enablement. In the vertical application layer, the VAL client communicates with the VAL server over VAL-UU reference point. VAL-UU supports both unicast and multicast delivery modes. The AIML enablement functional entities on the UE and the server are grouped into AIMLE client(s) and AIML enablement server(s) respectively.

The AIMLE includes of a common set of services for comprehensive enablement of AIML functionality, including federated and distributed learning (e.g., FL client registration management, FL client discovery and selection), and reference points. The AIMLE services are offered to the vertical application layer (VAL).

The AIML enablement client communicates with the AIMLE server(s) over the AIML-UU reference points. The AIMLE client provides functionality to the VAL client(s) over AIML-C reference point. The VAL server(s) communicate with the AIMLE server(s) over AIML-S reference points. The AIMLE servers communicate with the underlying 3GPP network systems using the respective 3GPP interfaces specified by the 3GPP network system. AIML-E reference point enables interactions between two AIMLE servers (e.g. central and edge AIMLE servers).

The AIMLE server interacts with the ML repository which serves as repository for ML model and ML participants over AIML-R.

#### 5.2.1.1	Service-based AIMLE architecture representation

Figure 5.2.1.1-1 exhibits the service-based interfaces for providing and consuming AIML enablement services. The AIMLE server could provide service to VAL server and AIML enablement client through interface SAiml.

Figure 5.2.1.1-1: Architecture for AIML enablement – Service based representation.

Figure 5.2.1.1-2 illustrates the service-based representation for utilization of the 5GS network services based on the 5GS SBA specified in 3GPP TS 23.501 [6].

Figure 5.2.1.1-2: Architecture for AIMLE utilizing the 5GS network services based on the 5GS SBA – Service based representation,

The AIML enablement server as well as ADAES is deployed as a SEAL server; hence enhancements to SEAL architecture (as specified in 3GPP TS 23.434 [5]) are needed to incorporate the AIML enablement service. Figure 5.2.1.1-3 illustrates the service-based representation including AIMLE server as part of the SEAL framework.

Figure 5.2.1.1-3: SEAL functional model representation using service-based interfaces and including AIMLE function.

### 5.2.2	Off-Network AIML Enablement Functional Architecture

Figure 5.2.2-1: Off-network AIML enablement functional model

Figure 5.2.2-1 illustrates the off-network (UE-to-UE) functional model of AIML enablement. In the vertical application layer, the VAL client communicates with a further VAL client over VAL-PC5 reference point. VAL-PC5 supports both unicast and multicast delivery modes. The UE1, if connected to the network via Uu reference point, can also act as a UE-to-network relay, to enable UE2 to access the VAL server(s) over the VAL-UU reference point.

The AIMLE client communicates with a further AIMLE client(s) over the AIML-PC5 reference points. The AIMLE client provides functionality to the VAL client(s) over AIML-C reference point. Such communication is performed for supporting local ML operations (training, distribution, inference) in a coordinated manner.

The off network functional architecture is similar to SEAL off-network architecture (as specified in 3GPP TS 23.434 [5]).

### 5.2.X	Reference Points Description

#### 5.2.X.1	General

The reference points for the functional model related to AIML enablement are described in the following subclauses.

#### 5.2.X.2	AIML-UU

The interactions related to AIML enablement functions between the AIML enablement client and AIML enablement server are supported by AIML-UU reference point. This reference point utilizes Uu reference point as described in 3GPP TS 23.401 [7] and 3GPP TS 23.501 [6].

#### 5.2.X.3	AIML-S

The interactions related to AIML enablement functions between the VAL server(s) and the AIML enablement server are supported by AIML-S reference point.

#### 5.2.X.4	AIML-C

The interactions related to AIML enablement functions between the VAL client(s) and the AIML enablement client within a VAL UE are supported by AIML-C reference point.

#### 5.2.X.5	AIML-R

The interactions related to AIML enablement functions between the AIML enablement server, and the ML repository are supported by AIML-R reference point.

#### 5.2.X.6	AIML-E

The interactions related to AIML enablement functions between AIML enablement servers (e.g., central and edge AIMLE servers).

#### 5.2.X.7	AIML-PC5

The interactions related to AIML enablement functions between AIML enablement clients in off-network deployments.

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[3]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[4]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

[5]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[6]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[7]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[8]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management".