3GPP TSG-SA WG6 Meeting #62	S6-243597

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243176)

Source:	Lenovo

Title:	Support for FL event notification

Spec:	3GPP TS 23.482 v0.1.0

Agenda item:	9.10

Document for:	Approval

Contact:	Emmanouil Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This contributions introduces a new ML repository capability for supporting FL related event subscriptions and notificaitons.

2. Reason for Change

This contributions introduces a new ML repository capability for supporting FL related event subscriptions and notificaitons. This is based on concluded Solution #16 of TR 23.700-82.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 v0.1.0.

* * * First Change * * * *

## 6.x	Support for FL events subscription and notification

This functionality enables a consumer (who can be the AIMLE server or a VAL server e.g. acting as FL server) to subscribe for FL related events and getting notified on changes on the availability of the FL members which are to be used for the FL-related task (e.g. training). This capability at the ML repository acting as an AIML service registry supports the subscription for events related to FL members and the notification to the consumer in case of changes. This solution assumes that such FL members (AIMLE or VAL server or AIMLE clients) have previously registered to this registry their availability and capabilities.

* * * Next Change * * * *

## 8.x	Support for FL events subscription and notification

### 8.x.1	General

This clause consists of the following procedures 1) the subscription for the FL related events in clause 8.x.2 and 2) the event notification procedure in clause 8.x.3.

This capability targets the enhancements to a global ML repository acting as AIML service registry which monitors the status and changes to the availability and capabilities of the FL members. The subscriber can be the AIMLE server or VAL server which requires to get notified on the FL member availability, since this information is vital for enabling the subscriber to perform the FL-related AIML operations (e.g. AIMLE server selecting FL members for a certain FL task).

Editor’s note: It is FFS whether the VAL server is subscribing directly or indirectly via AIMLE server for FL-related events.

### 8.x.2	Procedure on subscription for FL related events

This procedure, as illustrated in Figure 8.x.2-1, describes the subscription for events related to FL member availability.

Pre-conditions:

- The AIMLE server or VAL server has the authorization to subscribe for the FL-related events (events described in 8.x.4).

Figure 8.x.2-1: Procedure for FL-related event subscription

1.	The AIMLE server or VAL server (via AIMLE server) sends an FL-related event subscription request to the ML repository in order to receive notification of FL related events and in particular the availability of FL member for a target area and time.

2.	Upon receiving the event subscription request from the AIMLE server or VAL server, the ML repository checks for the relevant authorization for the event subscription.

3.	If the authorization is successful, the ML repository stores the subscription information.

4.	The ML repository sends an FL-related event subscription response to AIMLE server or VAL server (via AIMLE server) indicating successful operation.

### 8.x.3	Procedure on FL related event notification

This procedure describes the event notification procedure for the FL member availability. This can be triggered based on a change at the availability or capabilities of the candidate or selected FL members.

Pre-conditions:

- The subscription procedure in 8.x.2 was performed by the AIMLE server or VAL server.

- The FL member has registered its capabilities or availability to the ML repository based on the support for FL registration capability.

Figure 8.x.3-1: Procedure for FL-related event notifications

1.	The ML repository detects a change related to the availability of a Candidate or Selected FL member and generates events to be consumed by the AIMLE server, based on the trigger notification, or based on a received registration from a new FL member, who has already registered at the ML repository.

NOTE: This detection can be in form of a trigger report from the candidate /selected FL member or a registration and registration update of a new FL member at the ML repository as described in clause 8.y.3 and 8.y.4.

2.	For the generated event, the ML repository retrieves the list of corresponding subscriptions.

3.	The ML repository sends FL-related event notification to all entities (AIMLE server, VAL server) that have subscribed for the FL-related event matching the criteria. If a notification reception information is available as part of the FL-related event subscription, then the notification reception information is used by the ML repository to send event notifications to the corresponding AIMLE and VAL servers.

4.	The notified AIMLE server (s) or VAL server (s) (via AIMLE server) send an event notification acknowledgement to the ML repository.

### 8.x.4	Definition of FL-related Events

The definition of the FL related events is provided in Table 8.x.4-1.

Table 8.x.4-1: Definition of FL related events

| Events | Events Description |
| --- | --- |
| Availability changes of FL member | The event type relates to the availability change of a selected or candidate FL member, which can be a selected entity as a FL server or FL client. Such availability change can be an indication of availability or un-availability for a given service area, or VAL UE, or for a given ML model ID/profile. |
| FL model information | Information such as Accuracy, time schedule and latency for the FL training. Latency considering the target latency, i.e., when the FL model training shall be completed. |

Editor’s note: The definition of further FL-related events is FFS.

### 8.x.5	Information flows

#### 8.x.5.1	General

The following information flows are specified for supporting the FL events subscription and events notification based on 8.x.2 and 8.x.3.

#### 8.x.5.2	FL-related event subscription request

Table 8.x.5.2-1 describes information elements for FL-related event subscription request from the Subscriber (AIMLE server or VAL server) to the ML repository.

Table 8.x.5.2-1: FL-related event subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | The identifier to determine the identity of the requestor entity. This can be the VAL server ID or AIME server ID. |
| Security Credentials | M | The security credentials of the requestor. |
| Time of validity | M | The time validity for the subscription. |
| ML model information | M | Information of the ML model for which the subscription is needed. |
| >ML model ID | M | The identity of the ML model for which the subscription is needed. |
| FL member information | M | Information on the FL member (candidate or selected). |
| >FL member type | O (NOTE) | The type of FL member which can be an FL server or FL client. |
| >FL member ID | O (NOTE) | The ID of the entity serving as FL member. This can be used for events where the FL member is known and possibly selected, and the availability needs to be checked. |
| Event Information | M | Information on the FL-related event. |
| > Event ID | O | The identity of the FL-related event (if known by the requestor). |
| > Event type | M | The event type requested, based on the Definition of FL related events (shown in Table 8.x.4-1). |
| Notification endpoint | M | The information of the endpoint for receiving the notifications for the event. |
| NOTE: At least one of these shall be present |  |  |

#### 8.x.5.3	FL-related event subscription response

Table 8.x.5.3-1 describes information elements for the FL-related event subscription response to the Subscriber (e.g. AIMLE server or the VAL server / EAS) from the ML repository.

Table 8.x.5.3-1: FL-related event subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the event subscription operation |
| Subscription Identifier | M | The unique identifier for the event subscription. |

#### 8.x.5.4	FL related event notification

Table 8.x.5.4-1 describes information elements for the FL-related event notificaton from the ML repository to the requestor (e.g. AIMLE server or the VAL server, EAS).

Table 8.x.5.4-1: FL-related event notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | The unique identifier of the event subscription |
| Event identifier | M | The unique identifier for the event. For the definition of events, refer to the Table 8.x.4-1 with the list of FL related events. |
| Event related information | M | The event related information (e.g., time at which the event originated or is planned to be executed, location of event). |
| Event content | M | The content of the event information. |
| > List of FL member ID /address | O | The list of the FL members and their addresses. |
| >> FL member enter / leave | O | The indication of an FL member entering or leaving the available list. |
| >> FL member updated capability | O | The update of capabilities for the FL members. |
| >>Time availability of FL member | O | The time period for which the FL member is available. |
| >> Area of interest | O | The area of interest or exact / predicted location for which the event applies. |

#### 8.x.5.5	FL-related event notification acknowledgement

Table 8.x.5.5-1 describes information elements for the FL-related event notification acknowledgement from the VAL server or AIMLE server to the ML repository.

Table 8.x.5.5-1: FL-related event notification acknowledgement

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The positive or negative acknowledgement of the notification reception. |

* * * Next Change * * * *