3GPP TSG-SA WG6 Meeting #62	S6-243603

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243229)

Source:	Convida Wireless LLC, KPN N.V.

Title:	AIMLE client discovery/selection

Spec:	3GPP TR 23.482 V0.1.0

Agenda item:	9.10

Document for:	Approval

Contact:	Ly.Quang@convidawireless.com, liu.lu@convidawireless.com

1. Introduction

The conclusion for key issue #3 in TR 23.700-82 stated that solutions #2, #6, and #7 can be merged and be considered for normative work. This contribution focuses on merging portions of the three solutions to support discovery, retrieval, and selection of AIMLE clients. Note that the AIMLE client monitoring procedure in solution #6 is not part of this contribution.

2. Reason for Change

In some scenarios (e.g. FL), an AIMLE service consumer does not have visibility into capabilities supported by AIMLE clients or the available datasets provided by the AIMLE clients. Hence the consumer needs to first discover AIMLE clients with the required capabilities and dataset for the AI/ML application and operations. The AIMLE service consumer may perform multiple discovery/retrieval requests until all the AI/ML requirements (e.g. enough AIMLE clients, available datasets, appropriate age of dataset, enough dataset samples, required QoS, access and/or RAT type, current and historical location) are met before initiating AI/ML operations.

After enough AIMLE clients are discovered that meets all of the AI/ML requirements, the AIMLE service consumer requests the AIMLE server to create a set of the selected AIMLE clients to enable future AI/ML operations (e.g. data management, training, inferencing). The forming of the AIMLE client set is an indication that the AIMLE clients are available and have the required data (or is able to collect and process the required data) to support AI/ML operations and the AIMLE server uses the AIMLE client set to manage the AI/ML operations.

In certain scenarios, the AIMLE service consumer can request the AIMLE server to dynamically perform AIMLE client selection by providing AI/ML policy IDs. The AIMLE server uses the information in the AI/ML policy IDs to select appropriate AIMLE clients for participation in AI/ML operations.

Therefore, AIMLE client discovery and selection procedure needs to be defined to support AIMLE services.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[3]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[4]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

[5]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[a]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

* * * Next Change * * * *

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

AIMLE client set identifier: an identifier of the set of selected AIMLE clients. The VAL group ID may be used as AIMLE client set identifier and AIMLE client may be identified by the VAL UE ID or VAL User ID

* * * Next Change * * * *

## 8.x	AIMLE client discovery

### 8.x.1	General

Discovery of AIMLE clients is an important step in the AI/ML process for distributed, federated, split AI/ML, and transfer learning. Due to the nature of such learning, AIMLE service consumers need to discover suitable AIMLE clients to fulfil the requirements for the AI/ML application.

The following clauses specify procedures, information flows, and APIs for AIMLE client discovery.

### 8.x.2	Procedure

#### 8.x.2.1	AIMLE client discovery

Pre-conditions:

1.	AIMLE clients that support AI/ML operations have registered with the AIMLE server and included their AIMLE client profiles and optionally a list of supported services.

2.	The AIMLE server maintains a repository (e.g. ML repository) of AIMLE clients and associated AIMLE client profiles and supported services.

Figure 8.x.2.1-1: AIMLE client discovery

1.	An AIMLE service consumer (e.g. VAL server) sends an AIMLE client discovery request to an AIMLE server to discover a list of AIMLE clients that are available to participate in AI/ML operations (e.g. is available and have required data to train an ML model). The AIMLE client discovery request includes information as described in Table 8.x.3.1-1.

2.	The AIMLE server performs authentication and authorization checks to determine if the requestor is able to discover AIMLE clients. If the requestor is authorized, the AIMLE server performs the following to discover AIMLE clients fulfilling the provided AIMLE client discovery criteria.

If Member UE filtering criteria is provided, the AIMLE server utilizes the Member UE selection assistance procedure from 3GPP TS 23.502 [a], clause 4.15.13 to get assistance in selecting candidate UEs. The AIMLE server also obtains candidate UEs from the ML repository.

From the list of candidate UEs, the AIMLE server discovers a list of UEs (i.e. AIMLE clients) that fulfils the discovery criteria based on AIMLE client profiles and the provisioned the list of AI/ML Policy IDs.

3.	The AIMLE server sends an AIMLE client discovery response that includes information in Table 8.x.3.2-1.

### 8.x.3	Information flows

#### 8.x.3.1	AIMLE client discovery request

Table 8.x.3.1-1 shows the request sent by an AIMLE service consumer to an AIMLE server for the AIMLE client discovery/selection procedure.

Table 8.x.3.1-1: AIMLE client discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g. AIMLE service consumer). |
| Service identifier | O | An identifier for the service associated with the requestor. |
| AIMLE client discovery criteria | M | Discovery criteria for finding suitable AIMLE clients for AI/ML operations as detailed in table 8.x.3.1-2. |
| Member UE filtering criteria | O | Filtering criteria from Table 4.15.13.2-1 of 3GPP TS 23.502 [a]. |

Table 8.x.3.1-2: AIMLE client discovery criteria

| Information element | Status | Description |
| --- | --- | --- |
| Requested ML models | O | Requested ML models |
| Requested AIML operations | M | Role for data collection, data preparation, training, and inferencing. |
| Target area | O | Target area for data collection purposes |
| Application layer AIMLE client capabilities | M | Application layer AIMLE client capability information (e.g., ML application type, client availability to support AIML operations at the UE). |
| Dataset requirements | O | Information about dataset |
| >Dataset availability | O | Dataset availability, including dataset identifiers, dataset size, age, list of dataset features. |
| >Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g. raw data), supported data processing capabilities (e.g. processed data) and supported exploratory data analysis functions. |
| Required service | O | Information about the service |
| >Required service ID | O | Required service ID that the client supports |
| >Required corresponding service permission level | O | Required corresponding service permission level (e.g., premium resource usage standard resource usage, limited resource usage) |

#### 8.x.3.2	AIMLE client discovery response

Table 8.x.3.2-1 shows the response sent by the AIMLE server to the AIMLE service consumer for the AIMLE client discovery/selection procedure.

Table 8.x.3.2-1: AIMLE client discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| List of AIMLE client IDs | M | A list of AIMLE client IDs that matches the AIML discovery criteria. |
| Number of discovered AIMLE clients | O | The number of AIMLE client profiles that was found to match the AIML discovery criteria. The number indicates the potential AIMLE clients that could participate in AIML operations. |

## 8.y	AIMLE client selection

### 8.y.1	General

The AIMLE client selection procedure has two modes of operation: static selection and dynamic selection. In static selection, a List of AIMLE client IDs is provided to form an AIMLE client set, which can be used for AI/ML operations (e.g. data management, training, inferencing). For dynamic selection, both AIMLE client discovery criteria and Number of the required AIMLE clients are provided for the AIMLE server to dynamically select AIMLE clients. A List of AI/ML policy IDs for the AIMLE client selection may also be provided to assist with obtaining candidates AIMLE clients, which can then be selected for AI/ML operations.

The following clauses specify procedures, information flows, and APIs for AIMLE client selection.

### 8.y.2	Procedure

#### 8.y.2.1	AIMLE client selection

Pre-conditions:

1.	AIMLE clients that support AI/ML operations have registered with the AIMLE server and included their AIMLE client profiles and optionally a list of supported services.

2.	The AIMLE server maintains a repository (e.g. ML repository) of AIMLE clients and associated AIMLE client profiles and supported services.

3.	An AIMLE service consumer may provision the required AI/ML policies and have the corresponding policy ID(s).

Figure 8.y.2.1-1: AIMLE client selection

1.	An AIMLE service consumer (e.g. VAL server) sends an AIMLE client selection request to an AIMLE server to select AIMLE clients available for participation in AI/ML operations (e.g. is available and have required data to train an ML model). The AIMLE client selection request includes information as described in Table 8.y.3.1-1.

2.	The AIMLE server performs authentication and authorization checks to determine if the requestor is able to select AIMLE clients.

3.	If the requestor is authorized, the AIMLE server performs either static selection or dynamic selection as described below.

For static selection, the AIMLE server receives List of AIMLE client IDs in the request and selects the AIMLE clients in the list.

For dynamic selection, the AIMLE server first determines a list of candidate AIMLE clients.

If Member UE filtering criteria is provided in the request, the AIMLE server utilizes the Member UE selection assistance procedure from 3GPP TS 23.502 [a], clause 4.15.13 to get assistance in selecting candidate UEs. The AIMLE server also obtains candidate UEs from the ML repository.

From the list of candidate UEs, the AIMLE server selects a list of UEs (i.e. AIMLE clients) that fulfils the discovery criteria based on AIMLE client profiles and the provisioned the list of AI/ML Policy IDs.

The AIMLE server may determine the QoS parameters for the AIML traffic session between the requestor and the selected AIMLE client(s) and configure the AI/ML traffic session(s) via SEALDD (Sdd_RegularTransmission API) or NEF services (AfSessionWithQoS API).

The AIMLE server assigns an AIMLE client set identifier (e.g., VAL group ID) for the selected AIMLE clients.

NOTE 1:	If the available AIMLE clients (as determined by the AIMLE server) that fulfil discovery criteria are less than the required minimum number of AIMLE clients for AI/ML operation (e.g. split AI/ML), then the AIMLE server may discover the remaining required AIMLE clients from other AIMLE servers over AIML-E reference point and include them in the response message.

NOTE 2: The AIMLE server can reuse SEAL group management for any necessary group management.

Editor’s Note: Whether and how the AIMLE Server determines the QoS parameters is FFS.

4.	The AIMLE server performs AIMLE client participation procedure with each AIMLE client as described in clause 8.z.

5.	The AIMLE server sends an AIMLE client selection response that includes information in Table 8.y.3.2-1.

### 8.y.3	Information flows

#### 8.y.3.1	AIMLE client selection request

Table 8.y.3.1-1 shows the request sent by an AIMLE service consumer to an AIMLE server for the AIMLE client selection procedure.

Table 8.y.3.1-1: AIMLE client selection request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g. AIMLE service consumer). |
| Service identifier | O | An identifier for the service associated with the requestor. |
| List of AIMLE client IDs | O (NOTE1) | A list of AIMLE client IDs that was previously discovered for inclusion into an AIMLE client set. |
| AIMLE client discovery criteria | O (NOTE2) (NOTE3) | Discovery criteria for finding suitable AIMLE clients for AI/ML operations as detailed in table 8.x.3.1-2. |
| Number of the required AIMLE clients | O (NOTE2) (NOTE3) | Indicates the requested number of AIMLE clients to be selected based on selection policies. |
| List of AI/ML policy IDs for the AIMLE client selection | O (NOTE2) (NOTE3) | Represents the list of policies that shall be enforced for the client selection procedure. |
| Member UE filtering criteria | O | Filtering criteria from Table 4.15.13.2-1 of 3GPP TS 23.502 [a]. |
| Consumer Endpoint | M | Represents the endpoint (pair Address-Port or URL) at the AIMLE service consumer for receiving ML model update from the selected AIMLE Client(s). |
| NOTE1: This information element is present for static AIMLE client selection. NOTE2: These information elements are present for dynamic AIMLE client selection. NOTE3: At least one of the information elements are present. |  |  |

#### 8.y.3.2	AIMLE client selection response

Table 8.y.3.2-1 shows the response sent by the AIMLE server to the AIMLE service consumer for the AIMLE client selection procedure.

Table 8.y.3.2-1: AIMLE client selection response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| AIMLE client set identifier | M | An identifier to associate with the set of AIMLE clients that the AIMLE service consumer has provided in the request. The AIMLE client set can be updated by using this identifier. |
| List of AIMLE client IDs | M (NOTE1) | A list of AIMLE client IDs that matches the AIML discovery criteria. |
| Number of discovered AIMLE clients | O (NOTE1) | The number of AIMLE client profiles that was found to match the AIML discovery criteria. The number indicates the potential AIMLE clients that could participate in AIML operations. |
| NOTE1: This information element is present for dynamic selection. |  |  |

## 8.z	AIMLE client participation

### 8.z.1	General

The following clauses specify procedures, information flows, and APIs for AIMLE client participation.

### 8.z.2	Procedure

#### 8.z.2.1	AIMLE Client Participation

Figure 8.z.2.1-1: AIMLE client participation

1.	An AIMLE service consumer (e.g. VAL server) sends a request to an AIMLE server to discover a list of AIMLE clients that are available to participate in AI/ML operations (e.g. is available and have required data to train an ML model) and/or to select a list of AIMLE clients that meet the requirements for AIML operations. The AIMLE client discovery/selection request includes information as described in Table 8.x.3.1-1.

2.	The AIMLE server validates the discovery/selection request and verifies the security credentials provided in the request. The AIMLE server further performs authentication and authorization checks to determine if the requestor is able to discover AIMLE clients or create an AIMLE client set.

### 8.z.3	Information flows

#### 8.z.3.1	AIMLE client participation request

Table 8.z.3.1-1 shows the request sent by the AIMLE server to each AIMLE client selected for AIMLE client participation procedure.

Table 8.z.3.1-1: AIMLE client participation request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| AIMLE client set identifier | M | An identifier for the AIMLE client set. |
| Client set operational indicator | M | Indicator for adding/removing the AIMLE client to/from the AIMLE client set. |
| AI/ML model | M | The AI/ML model to use for the AI/ML operation. |
| AI/ML operations | M | A list of AI/ML operations (e.g. data collection, data preparation, training, inferencing) required to be performed. |
| Operational schedule | O | Schedule for the AI/ML operations. |
| Dataset requirement | M | Dataset requirements for the AI/ML operations. Requirements includes dataset identifier, dataset size and age, and/or dataset features. |
| Service requirement | M | Information about the required Service ID and its service permission level (e.g., premium resource usage standard resource usage, limited resource usage) |

#### 8.z.3.2	AIMLE client participation response

Table 8.z.3.2-1 shows the response sent by the AIMLE clients to the AIMLE server for the AIMLE client participation procedure.

Table 8.z.3.2-1: AIMLE client participation response

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| Acknowledge | M | An indicator to acknowledge whether the AIMLE client accepts being added/removed from the AIMLE client set. |

* * * End of Change * * * *