3GPP TSG-SA WG6 Meeting #62	S6-243189

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0307 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add procedure of Geofencing UE(s) Information request/response |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT,ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the TR 23.700-72 v1.1.1, the KI#1 for Geofencing has been concluded as follows: Solution #2 and Solution#4 addressing the KI#1 can be considered in the normative work to realize the different functions. The related function and procedure will be specified in 3GPP TS 23.434 to support the two solutions, and the detailed APIs and information flows can be discussed in the normative phase. So it’s needed to specify the functions, procedures and information flows for Geofencing function specified in Sol#2 in TR 23.434 for the normative work. This CR is highlightd to add the peocedure of Geofencing UE(s) Information request/response included in the Sol#2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Geofencing function in current procedure of Obtaining UE(s) information at a location. - Add Geofencing related parameters in current information flow of Obtaining UE(s) information at a location. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Geofencing will not be realized for vertical application services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.10，9.3.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.10	Obtaining UE(s) information at a location

Figure 9.3.10-1 describes the procedure for obtaining UE(s) information at a location. This procedure also applies for the Geofencing service that the VAL server may request the UE(s) information when the UE entering the Geofencing location area.

Pre-condition:

-	The VAL server has a jurisdiction over a geographical area for which the location management server is configured to operate.

-	The UE(s) in the geographical area have provided its location information to the location management server.

-	The location management server has obtained the requested UE location information either from 3GPP CN or the target location management client/VAL UE.

Figure 9.3.10-1: Obtaining UE(s) information at a location

- The VAL server sends get UE information request to the location management server. The request contains the location information (e.g.  geographic area, VAL service area identified by the VAL service area ID, Geofencing location area represented by different types of shape, etc) and application defined proximity range as defined in table 9.3.2.9-1.

2.	The location management server authorizes the request and determines the UE(s) whose location are within the application defined proximity range of the location information or VAL service area identified by the VAL service area ID or Geofencing location area provided in step 1.

3.	The location management server sends get UE information response to the VAL server with a list of VAL UE(s) or VAL user(s) and its corresponding location information as determined in step 1.

* * * Next Change * * * *

9.3.2.9	Get UE(s) information request

Table 9.3.2.9-1 describes the information flow for a VAL server to get UE(s) information at the LM server.

Table 9.3.2.9-1: Get UE(s) information request

| Information element | Status | Description |
| --- | --- | --- |
| Location information | M | Location information around which the UE(s) information is requested (e.g. geographic area or VAL service area ID). The information is also used for Geofencing service. |
| > Shape type | O | It includes the Geofencing location area represented by different types of shape (e.g., polygon or circle). |
| > Area Coordinates | O | Geographical coordinates of shape. |
| Application defined proximity range information | M | Description of the range information over which the UE(s) information is required. |

* * * End of Changes * * * *