3GPP TSG-SA WG6 Meeting #62	S6-243643

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243251)

Source:	Huawei, Hisilicon

Title:	clarification on relationship with SA4 on data collection

Spec:	3GPP TS 23.482

Agenda item:	9.10

Document for:	Approval

Contact:	Yajie Hu (Huyajie1@huawei.com)

1. Introduction

This paper provide clarification on relationship with SA4 on data collection.

2. Reason for Change

SA4 defined data collection procedure and related functionality, however if AI/ML enabler layer is ehnahnced with data collection then the data collection functionality will be deplicated in SA4 and SA6.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.1.0

* * * First Change * * * *

Annex X(informative):
Data collection

3GPP TR 26.927 [11] defines AI and ML for Media. This study aims to identify relevant interoperability requirements and implementation constraints of AI/ML in 5G media services. This study includes media-based AI/ML use cases and architecture considerations related to media services.

3GPP TS 26.531 defines data collection which application client send the application layer data to data colleciton AF via direct data collection client, meanwhile application server can send application layer data to also to the data collection AF.

The AI/ML enabler server can obtain application layer data from data collection on data network side. Alternatively, the AI/ML enabler client could obtain data from Data Collection AF optionally via AI/ML enabler server. The DDCC can be implemented in AI/ML enabler client, and the DCAF can be implemented in AI/ML enabler server.

NOTE: The permissions (authorization) before sending / collecting application layer data is required

Figure X-1: Data collection