3GPP TSG-SA WG6 Meeting #62	S6-243646

Maastricht, Netherlands, 19th – 23th Aug 2024	(revision of S6-243060)

Source:	NTT DOCOMO

Title:	Resolve EN related to subclause 4.5.3 Usage of CAPIF by API provider

Spec:	3GPP TR 23.946 v0.5.0

Agenda item:	9.7

Document for:	Approval

Contact:	junpei.uoshima.mt@nttdocomo.com

1. Introduction

This pCR presents the resolve EN related to subclause 4.5.3 Usage of CAPIF by API provider.

2. Reason for Change

The procedures of CAPIF are extensive and could be difficult for developers to understand immediately. In particular, in order for an API invoker to invoke a service API, several steps must be taken, such as logging, charging, and monitoring and so on. Therefore, this pCR focuses on the API Provider, demonstrating an overview of the procedures for an API Provider to invoke service APIs. This will help developers understand the steps for service API invocation.

This pCR will be helpful for the developers in understanding these procedures.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.946.

* * * First Change * * * *

4.5.3	Usage of CAPIF by API provider

Figure 4.5.3-1 describes the usage of CAPIF by API provider for APIs which does not involve resource owner authorization.

Figure 4.5.3-1: Usage of CAPIF by API provider for APIs which does not involve resource owner

- The API publishing function sends service API publish request to the CAPIF core function, as specified in subclause 8.3 in 3GPP TS 23.222 [2].

2.	The API provider authenticates the API invoker prior to the service API invocation, utilizes the API exposing function, as specified in subclause 8.14 in 3GPP TS 23.222 [2].

3.	The API provider authenticates the API invoker upon invocation of the service APIs, utilizes the API exposing function, as specified in subclause 8.15 in 3GPP TS 23.222 [2].

4.	The API provider authorizes the API invoker to access, as specified in subclause 8.16 in 3GPP TS 23.222 [2].

5.	The API provider controls the access of the service API by the API invoker based on policy or usage limits, as specified in subclause 8.16 and 8.17 in 3GPP TS 23.222 [2].

6.	The API provider maintains the log of the API invocations at the CAPIF core function for services such as charging, invocation history, utilizes the API exposing function, as specified in subclause 8.19 in 3GPP TS 23.222 [2].

7.	The API provider facilitates charging of the API invocations, utilizes the API exposing function, as specified in subclause 8.20 in 3GPP TS 23.222 [2].

8.	The API provider facilitates monitoring such as API invoker's ID and IP address, utilizes the API management function, as specified in subclause 8.21 in 3GPP TS 23.222 [2].

9.	The API provider utilizes the API management function for auditing, as specified in subclause 8.22 in 3GPP TS 23.222 [2].

Figure 4.5.3-2 describes the usage of CAPIF by API provider for APIs which involves resource owner authorization.

Figure 4.5.3-2: Usage of CAPIF by API provider for APIs which involve resource owner

- The API publishing function sends service API publish request to the CAPIF core function, as specified in subclause 8.3 in 3GPP TS 23.222 [2].

2.	The API provider authenticates the API invoker prior to the service API invocation, utilizes the API exposing function, as specified in subclause 8.14 in 3GPP TS 23.222 [2].

3.	The API provider authenticates the API invoker upon invocation of the service APIs, utilizes the API exposing function, as specified in subclause 8.15 in 3GPP TS 23.222 [2].

4.	API authorization is based on the authorization information obtained from the resource owner function, as specified in subclause 8.31 in 3GPP TS 23.222 [2].

5.	The API provider authorizes the API invoker to access, as specified in subclause 8.16 in 3GPP TS 23.222 [2].

6.	The API provider controls the access of the service API by the API invoker based on policy or usage limits, as specified in subclause 8.16 and 8.17 in 3GPP TS 23.222 [2].

7.	The API provider maintains the log of the API invocations at the CAPIF core function for services such as charging, invocation history, utilizes the API exposing function, as specified in subclause 8.19 in 3GPP TS 23.222 [2].

8.	The API provider facilitates charging of the API invocations, utilizes the API exposing function, as specified in subclause 8.20 in 3GPP TS 23.222 [2].

9.	The API provider facilitates monitoring such as API invoker's ID and IP address, utilizes the API management function, as specified in subclause 8.21 in 3GPP TS 23.222 [2].

10.	The API provider utilizes the API management function for auditing, as specified in subclause 8.22 in 3GPP TS 23.222 [2].