3GPP TSG-SA WG6 Meeting #62	S6-243655

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243566)

Source:	Samsung

Title:	Pseudo-CR on solution for the open as part of key issue #1 related to analytics

Spec:	3GPP TR 23.700-01 V1.1.0

Agenda item:	8.6

Document for:	Approval

Contact:	Arunprasath R, arun.prasath@samsung.com

1. Introduction

This pCR proposes the solution for the open issues which are related to new analytics that need to be supported for satellite access inorder to provide better service experience.

2. Reason for Change

ADAES is a SEAL service that has been specified in 3GPP TS 23.436 and provides various features that support the derivation and exposure of application layer analytics to consumers in the application and enablement layers. Particularly, the analytics may include statistics and predictions. It would be beneficial for the application servers to get to know about the analytics related to UE’s RAT connectivity (terrestrial or non-terrestrial) so that it can plan the activities like data delivery, communication establishment, QoS adaption etc., for the UE’s based on the stats received from the ADAE server. Application server would be able to judge whether a particular UE if involved in the communication can meet the required KPIs or not. Also VAL server or application server can make necessary steps (e.g., extended data buffering, pending downlink data transferring) to minimize the service disruption. These analytics/stats can be used by the VAL server or application server when the prediction stats depicts that UE might latch on to NR satellite access but it is not limiting the VAL server to use the same for the terrestrial access network also. Also these stats/analytics can be used by ECS (Edge configuration server) to determine a suitable EES with a matching satellite ID to serve the UE with longest service time in service provisioning. Hence, it is desirable to study the analytics related to UE RAT connectivity.

This analytics is to predict on what satellite RAT type a particular or set of UE’s connect to given a particular time and particular location. The ADAE server can predict the RAT type which a particular or set of UEs can connect to by utilizing the UE mobility analytics from NWDAF and by fetching the location information of the UE whenever there is change in the satellite RAT type of the UE changes. ADAE server can subcribe to the location information of the UE from the LMS by setting the trigger criteria to report the location whenever UE satellite RAT type changes.

Application servers may use this analytics/prediction along with the Observed Service Experience related network data analytics from NWDAF

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01 v1.1.0.

* * * First Change * * * *

### 7.2.x	Solution #AEx: Analytics related to UE RAT connectivity

#### 7.2.x.1	Solution description

This solution addresses the open issues listed as part of the key issue 1 which are related to identifying the new analytics required to be supported for satellite access. It introduces a new analytics type which needs to be supported by the ADAE (Application Data Analytics Enablement) server. It proposes the procedure for supporting the UE RAT connectivity analytics by the ADAE server which can be consumed by the analytics consumers (e.g. VAL server) in-order to provide a better service experience over satellite access. This analytics helps to predict the type of satellite RAT that a UE will latch to on a particular location and/or particular time and helps the analytics consumer to decide on certain action. Some of the examples of the action could be as below:

- Extended data buffering

- Pending downlink data transferring

- Whether to schedule a communication with the UE

- QoS adaption

##### 7.2.x.1.1	Procedure on UE RAT connectivity analytics

Figure 7.2.x.1.1-1 illustrates the procedure support the UE RAT connectivity analytics.

Figure 7.2.x.1.1-1: Procedure of ADAE Server supporting UE RAT connectivity analytics

1.	The analytics consumer (e.g.VAL server) makes a subscription request to ADAE server for UE RAT connectivity prediction/stats, including an analytics event ID for UE RAT Connectivity analytics. The request may include also the target area, a target VAL service, or a VAL UE, or group of UEs of the VAL service, time validity. If the VAL UEs are provided by the VAL server, this request may also include the expected route or a set of waypoints for the UEs of the VAL application.

2.	The ADAE server sends a UE RAT connectivity analytics subscription response as an ACK to the VAL server.

3.	The ADAE server subscribes to the SEAL Location management server to get the location information of the VAL UE along with the RAT type. It can set the trigger criteria of receiving the location information of the UE whenever UE RAT type changes as specified in 3GPP TS 23.434 [7] clause 9.3.5.

4.	The ADAE server subscribes for NWDAF UE mobility analytics per VAL UE (for all the VAL UEs) and gets notification on the per UE location/mobility analytics based on 3GPP TS 23.288 [2] clause 6.7.2.

5. 	If the data is collected from multiple sources, the ADAES combines or correlates the data/analytics from steps 3-4 and stores the data into A-ADRF if needed.

6.	The ADAE server optionally requests UE RAT connectivity historical analytics /data from A-ADRF for the corresponding VAL UEs.

7.	Based on the request, the ADAE server receives UE RAT connectivity historical analytics /data from A-ADRF for the corresponding VAL UEs.

8.	The ADAE server abstracts or correlates the data/analytics from steps 5-6 and provides analytics on the UE RAT connectivity for the target VAL application.

9.	The ADAE server sends the location accuracy analytics notification to the consumer

#### 7.2.x.2	Architecture Impacts

This solution has no architecture impacts to ADAE or any other SEAL services.

#### 7.2.x.3	Corresponding APIs

#### 7.2.x.3.1	General

The following information flows are specified for UE RAT connectivity analytics.

#### 7.2.x.3.2	UE RAT Connectivity analytics subscription request

Table 7.2.x.3.2-1 describes information elements for the UE RAT connectivity analytics subscription request from the VAL server / Consumer to the ADAE server.

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server. |
| Analytics ID | M | The identifier of the UE RAT connectivity analytics event. |
| VAL UE ID(s) or Group ID | M | The identity of the VAL UE(s) or group of UEs for which the analytics subscription applies |
| VAL service ID | O | The identifier of the VAL service for which location accuracy analytics is requested. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Area of Interest | O | The geographical or service area for which the subscription request applies. |
| Time validity | O | The time validity of the subscription request. |
| UE mobility / route information | O | Information on the target UE or group UE mobility including the expected route/set of waypoints. |
| Reporting requirements | O | It describes the requirements for analytics reporting. This requirement may include e.g. the type and frequency of reporting (periodic or event triggered), the reporting periodicity in case of periodic, and reporting thresholds. |

#### 7.2.x.3.3	UE RAT Connectivity analytics subscription response

Table 7.2.x.3.3-1 describes information elements for the UE RAT connectivity analytics subscription response from the ADAE server to the VAL server /Analytics Consumer.

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the analytics subscription request (positive or negative acknowledgement) |

#### 7.2.x.3.4	UE RAT Connectivity data request

Table 7.2.x.3.4-1 describes information elements for the UE RAT connectivity data request from the ADAE server to the A-ADRF.

| Information element | Status | Description |
| --- | --- | --- |
| ADAE server ID | M | The identifier of the ADAE server |
| Analytics ID | M | The identifier of the analytics event i.e. UE RAT connectivity analytics |
| List of VAL UE IDs and addresses | M | The VAL UE(s) identifiers and IP address(es) for which the data/analytics apply |
| VAL service ID | O | The service ID, in case of requesting historical data for a particular VAL service. |
| Reporting configuration | O | The configuration for data reporting. This requirement may include e.g. the frequency of reporting (periodic), the reporting periodicity in case of periodic, and reporting thresholds, whether data abstraction is needed or not. |
| Data collection requirements | O | The requirements for data collection, including the format of data, frequency of reporting, level of abstraction of data, level of accuracy of data. |
| Area of Interest | O | The geographical or service area for which the subscription request applies |
| Time validity | O | The time validity of the request |

#### 7.2.x.3.5	UE RAT Connectivity data response

Table 7.2.x.3.5-1 describes information elements for the UE RAT connectivity data response from the A-ADRF to the ADAE server.

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. |
| List of VAL UE IDs and addresses | M | The VAL UE(s) identifiers and IP address(es) for which the analytics apply |
| VAL service ID | O | The service ID, in case of requesting historical data for a particular VAL service. |
| Analytics Output | M | The reported analytics for the UE RAT connectivity, which can be in form of offline stats/historical data for a specific VAL service or for particular UE(s) or group of UEs |

#### 7.2.x.3.6	UE RAT Connectivity analytics notification

Table 7.2.x.3.5-1 describes information elements for the UE RAT connectivity analytics notification from the A-ADRF to the ADAE server.

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event (UE RAT connectivity) |
| Analytics Output | M | The predictive or statistical parameter, which can be: - A predicted or expected RAT type changes for a particular UEs for a given waypoints - A predicted or expected RAT Type UE would latch on for a given time period or location |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |

#### 7.2.x.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

This solution is for KI#1 and addresses the open issues about the new analytics required to be supported for satellite access. This solution does not impact the architecture of ADAE or any other SEAL services.

The solution proposes and new analytics ID for the UE RAT connectivity. The analytics consumer can request the ADAE server for gathering the UE RAT connectivity analytics stats/predictions for a UE or set of UEs. The ADAE server interacts with the NWDAF and SEAL LMS for getting the mobility analytics and location of the UE whenever its RAT connectivity changes. It also fetches the historical analytics data from A-ADRF and derives at the UE RAT connectivity analytics stats/predictions. The solution is feasible.

* * * Next Change * * * *

### 10.4.1	Key issue #1:

Solution #AEX addresses the open issues for the key issue #1. It defines a new analytics for the UE RAT connectivity and proposes the procedures for the ADAE server. The solution proposed in this study will be added as part of 3GPP TS 23.436 [436].

* * * Next Change * * * *

### 11.3.2	Conclusions of key issue #1 and #2

The study concludes with following solution considerations for the normative work:

-	Solution #AE1

-	Solution #AE5

-	Solution #AE7 And

-	Solution #AEX

* * * Next Change * * * *