3GPP TSG-SA WG6 Meeting #62	S6-243666

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243297/3574)

Source:	Huawei, HiSilicon

Title:	pCR on solution of data channel application related capability exposure

Spec:	3GPP TR 23.700-92 v1.1.0

Agenda item:	8.2

Document for:	Approval

Contact:	gecuili@huawei.com

1. Introduction

This pCR proposes solution of data channel application related capability exposure which solves the Key Issue#7.

2. Reason for Change

Contribution with new solution solving KI#7.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-92 v1.1.0.

* * * First Change * * * *

## 8.x	Solution #x: data channel application related capability exposure

### 8.x.1	Description

This clause provides solution against Key Issue #7 Support of data channel application related capability exposure.

As specified in clause 6.2.10 of 3GPP TS 26.114[14] that the data channel application is created prior to the DCMTSI call.

In this procedure, the eMMTel Enabler Server provides service for the DC application provider to support DC application upload, upgrade, deletion etc, based on the capabilities of the IMS subsystem.

### 8.x.2	Procedures

#### 8.x.2.1	Data channel application upload

Figure 8.X.2.1-1 below illustrates how to upload a data channel application to DCAR via eMMTel Enabler Server.

Pre-conditions:

1)	The DC application has been uploaded to Controlling Application Server.

Figure 8.x.2.1-1: Upload a data channel application to DCAR via eMMTel Enabler Server

1.	The Controlling Application Server sends the DC application package configuration request to the eMMTel Enabler Server, the request message includes information elements as specified in Table 8.x.2.1-1.

Table 8.x.2.1-1: Information elements in the DC application package configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Application ID | M | Identifier of the Data Channel Application |
| Application version | M | Version of the Data Channel Application |
| DC Application Package | M | Code package of the Data Channel Application |

NOTE:	The DC application profile configuration procedure from Controlling Application Server to eMMTel Enabler Server has been captured in solution#4.

2.	The eMMTel Enabler Server uploads DC application package to DCAR, with the following two alternatives mechanisms:.

Option 1 SA2 defines the DCAR interface: The eMMTel Enabler Server uploads the DC application package to DCAR via NEF or DCSF.

Option 2 SA6 defines the DCAR interface: The eMMTel Enabler Server invokes the API of the DCAR to upload the DC application package.

Editor's note:	The mechanism about how to upload DC application package from DCAR depends on the feedback of SA2

3.	The eMMTel Enabler Server sends the DC application package configuration response to the Controlling Application Server, the response message includes information elements as specified in Table 8.x.2.1-2.

Table 8.x.2.1-2: Information elements in the upload DC application package response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of upload DC application package request. |
| Failure Cause (see NOTE) | O | The reason for failure |
| NOTE: This IE shall only be present when the upload DC application package result is Failure. |  |  |

#### 8.x.2.2	Data channel application upgrade

Figure 8.X.2.2-1 below illustrates how to upgrade the version of a data channel application via eMMTel Enabler Server.

Pre-conditions:

1)	The DC application has been uploaded to DCAR via eMMTel Enabler Server.

Figure 8.x.2.2-1:  Upgrade the data channel application version via eMMTel Enabler Server

- The Controlling Application Server sends the DC application package upgrade request to the eMMTel Enabler Server, the request message includes information elements as specified in Table 8.x.2.2-1.

Table 8.x.2.2-1: Information elements in the upgrade DC application package request

| Information element | Status | Description |
| --- | --- | --- |
| Application ID | M | Identifier of the Data Channel Application |
| Application version | M | Version of the Data Channel Application |
| Application related parameters | O | Other application information that may need to be updated, e.g. the elements in DC application profile. |
| DC Application Package | M | Code package of the Data Channel Application |

- The eMMTel Enabler Server uugrades the DC application upgrade package to DCAR,with the following two alternative mechanisms:.

Option 1 SA2 defines the DCAR interface: The eMMTel Enabler Server upgrades the DC application package to DCAR via NEF or DCSF.

Option 2 SA6 defines the DCAR interface: The eMMTel Enabler Server invokes the API of the DCAR to upgrade the DC application package.

Editor's note:	The mechanism about how to upgrade DC application package to DCAR depends on the feedback of SA2

- The eMMTel Enabler Server sends the DC application package upgrade response to the Controlling Application Server, the response message includes information elements as specified in Table 8.x.2.2-4.

Table 8.x.2.2-2: Information elements in the upgrade DC application package response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of upgrade DC application package request. |
| Failure Cause (see NOTE) | O | The reason for failure |
| NOTE: This IE shall only be present when the upgrade DC application package result is Failure. |  |  |

#### 8.x.2.3	Data channel application delete

Figure 8.X.2.3-1 below illustrates how to delete a data channel application via eMMTel Enabler Server.

Pre-conditions:

1)	The DC application has been uploaded to DCAR via eMMTel Enabler Server.

Figure 8.x.2.3-1:  Delete a data channel application via eMMTel Enabler Server

- The Controlling Application Server sends the DC application package delete request to the eMMTel Enabler Server, the request message includes information elements as specified in Table 8.x.2.3-1.

Table 8.x.2.3-1: Information elements in the delete DC application package request

| Information element | Status | Description |
| --- | --- | --- |
| Application ID | M | Identifier of the Data Channel Application |

- The eMMTel Enabler Server requests to delete DC application package delete from DCAR, with the following two alternative ways:

Option 1 SA2 defines the DCAR interface: The eMMTel Enabler Server delete the DC application package via NEF or DCSF.

Option 2 SA6 defines the DCAR interface: The eMMTel Enabler Server invokes the API of the DCAR to delete the DC application package.

Editor's note:	The mechanism about how to delete DC application package from DCAR depends on the feedback of SA2

- The eMMTel Enabler Server sends the DC application package delete response to the Controlling Application Server, the response message includes information elements as specified in Table 8.x.2.3-2.

Table 8.x.2.3-2: Information elements in the delete DC application package response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of delete DC application package request. |
| Failure Cause (see NOTE) | O | The reason for failure |
| NOTE: This IE shall only be present when the delete DC application package result is Failure. |  |  |

### 8.x.3	Impacts on Existing Nodes and Functionality

In this solution, the procedures of application upload, application upgrade and application deletion need the capabilities provided by IMS core.

* * * End of Changes * * * *