3GPP TSG-SA WG6 Meeting #62	S6-243692

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243193)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0311 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Exposure of value-added UE location information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the TR 23.700-72 v1.1.1, the KI#4 for enhancement to support location information exposure has been concluded as follows: Solution #5, #9, #11 and #12 addressing the KI#4 can be considered in the normative work to realize the different functions for SEAL-LM enabler and ADAE enabler seperately. The related function and procedure will be specified in 3GPP TS 23.434 [9] and 3GPP TS 23.436 [15] to support the solutions, the detailed APIs and information flows, and the relationship of the APIs exposed from SEAL-LM and ADAE to avoid duplicated APIs towards the VAL server can be discussed in the normative phase. So it’s needed to specify the functions, procedures and information flows for such KI in TR 23.434 for the normative work. This CR focuses on how to speficy the function for Sol#5 in TS 23.434. The Sol#5 proposes the LMS could statistic and calculate the received UE location data per the temporal/spatial granularity and expose the value-added UE location data with different granularity as well as additional information (e.g., velocity) to the VAL server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the function that LMS could statistic and calculate the received UE location data per the temporal/spatial granularity in the location information subscription and the location information notification procedure. And the related information flows are updated accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The value-added location requests from application server can not be met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.7,9.3.8, 9.3.2.1,9.3.2.5, 9.3.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.7	Location information subscription procedure

Figure 9.3.7-1 illustrates the high level procedure of location information subscription request. The same procedure can be applied for location management client and other entities that would like to subscribe to VAL user or VAL UE location information. This procedure is also used for initiating tracking a UE's location.

Figure 9.3.7-1: Location information subscription request procedure

1.	The VAL server sends a location information subscription request to the location management server to subscribe location information of one or more VAL users or VAL UEs. The request may include an indication for supplementary location information, the location QoS which contains the location accuracy, response Time and QoS class as defined in clause 4.1b of TS 23.273 [50], the velocity information of target UEs, and an indication whether the statistic of target UE location is needed per e.g., temporal/spatial granularity. If the indication whether the statistic of target UE location is needed, the corresponding time/location information should also be included.

2.	The location management server shall check if the VAL server is authorized to initiate the location information subscription request and may retrieve the available and proper location access type and positioning methods (e.g. as described in TS 23.273 [50] and TS 29.572 [51]) for the target VAL UE based on the received location QoS. Further, the location management server may initiate location reporting configuration optionally including the retrieved location access type and positioning methods with the location management client of the UE for immediate reporting.

3.	The location management server may optionally subscribe for UE location information from 3GPP core network for the UE. If the indication for supplementary location information is included in step 1, then UE location information is obtained from the 3GPP core network and/or the 3rd party location management server.

If the velocity information of target UE is requested in step 1, the LM server asks the LM client or 3GPP core network to report the target UE location data with the requested velocity information.

NOTE:	How the location management server obtains the UE location from the 3rd party location management server is out of scope of this specification.

4.	The location management server determines the UE location information of the UE as received in steps 2 and 3 and checks if it meets the location QoS requirements (if any) or not.

If the indication whether the statistic of target UE location is included in step 1, the LM server performs the following actions based on different granularity.

-	If the indication whether the statistic of target UE location is needed per temporal granularity is included in step 1, the LM server calculates the UE location data received in step 3 per the temporal granularity with the requested time condition.

-	If the indication whether the statistic of target UE location is needed per spatial granularity is included in step 1, the LM server calculates the UE location data received in step 3 per the spatial granularity with the requested location information.

5.	The location management server replies with a location information subscription response indicating the subscription status and if immediate reporting was requested, the location information of the VAL UE(s).

* * * Next Change * * * *

9.3.8	Event-trigger location information notification procedure

Figure 9.3.8-1 illustrates the high level procedure of event-trigger usage of location information. The same procedure can be applied for location management client and other entities that would like to subscribe to location information of VAL user or VAL UE. This procedure is also used for obtaining latest UE's location for tracking purpose.

Figure 9.3.8-1: Event-trigger usage of location information procedure

1.	The location management server receives the latest location information of the UE as per the location report procedure described in clause 9.3.3.3.

2.	The location management server may optionally receive the location information of the UE from 3GPP core and/or the 3rd party location management server network. If the indication for supplementary location information is included in the subscription, then UE location information is obtained from the 3GPP core network and/or the 3rd party location management server. If the velocity information of target UE is requested, the LM server asks the LM client or 3GPP core network to report the target UE location data with the requested velocity information.

3.	Based on the configurations, e.g., subscription, periodical location information timer, location management server is triggered to report the latest user location information to VAL server. The location management server determines the location information of UE as received in steps 1 and 2, including the supplementary location information (if indicated). The Location management server may report the location to the VAL server considering the location information received via non-3GPP positioning technologies (e.g. GNSS, Bluetooth), for instance, to improve the location accuracy.

If the indication whether the statistic of target UE location is included in the request from the VAL server/LM client, the LM server calculates the received UE location data per temporal/spatial granularity as requested and then expose them to the VAL server/LM client for the valued-added location information.

4.	The location management server sends the location information report including the latest location information of one or more VAL users or VAL UEs to the VAL server or to the location management client that has previously configured.

5.	VAL server may further share this location information to a group or to another VAL user or VAL UE.

NOTE:	For other entities, the step 5 can be skipped if not needed.

* * * Next Change * * * *

9.3.2.5	Location information subscription request

Table 9.3.2.5-1 describes the information flow from the VAL server or location management client to the location management server for location information subscription request or for updating an existing location information subscription.

Table 9.3.2.5-1: Location information subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL user or VAL UE (see NOTE 3) |
| Identities list | M | List of VAL users or VAL UEs whose location information is requested. |
| VAL service ID | O | Identity of the VAL service for which the location information is subscribed. |
| Time between consecutive reports | M | It indicates the interval time between consecutive reports |
| Supplementary location information indication | O | Indicates that supplementary location information is required. |
| Location QoS | O (NOTE 2). | Definition of the location Quality of Service for which the location information is requested (see NOTE 1). |
| Velocity information | O | Identifies the velocity of the target UE for which the location information is requested. |
| Indication for the location data statistic | O | It indicates whether the statistic/calculation of target UE location data is needed per e.g. time/location. |
| Requested time information | O (NOTE 4) | It indicates the time information when the target UE location data is calculated per time. |
| Requested location information | O (NOTE 4) | It indicates the location information when the target UE location data is calculated per location. |
| NOTE 1: The definition of location QoS has been defined in clause 4.1b of TS 23.273 [50] and the clause 6.1.6.2.13 of TS 29.572 [51]. NOTE 2: The element is only applicable for the information flow from the VAL server to the location management server. NOTE 3: This information element shall not be updated via location information subscription update procedure in clause 9.3.2.7a. NOTE 4: This information element only applies when the indication for the statistic of target UE location data is included. |  |  |

* * * Next Change * * * *

9.3.2.7	Location information notification

Table 9.3.2.7-1 describes the information flow from the location management server to the VAL server or the location management client.

Table 9.3.2.7-1: Location information notification

| Information element | Status | Description |
| --- | --- | --- |
| Identities list | M | List of the VAL users or VAL UEs whose location information needs to be notified |
| Identity (see NOTE 2) | O | Identity of the VAL user or VAL UE subscribed to location of another VAL user or VAL UE (see NOTE 1) |
| Subscription ID (see NOTE 2) | O | Subscription identity related to VAL server subscription with Location management server for location information notification. |
| Triggering event | M | Identity of the event that triggered the sending of the notification |
| Location Information | M | Location information |
| Velocity information | O | Indicates the velocity of the target UE for which the location information is requested. |
| Statistic of location data | O | Indicates the statistic result of target UE location data per temporal/spatial granularity as requested. |
| Timestamp | O | Timestamp of the location report |
| NOTE 1: This is only used for location management server sends location information notification to the VAL user or VAL UE who has subscribed the location. NOTE 2: Either Identity or Subscription ID shall be included. |  |  |

* * * Next Change * * * *

9.3.2.1	Location reporting configuration response

Table 9.3.2.1-1 describes the information flow from the location management server to the location management client for the location reporting configuration. This information flow may be sent individually addressed or group addressed on unicast or multicast.

Table 9.3.2.1-1: Location reporting configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or VAL group to which the location reporting configuration is targeted or identity of the VAL UE. |
| Requested location information | O (see NOTE 1) | Identifies what location information is requested |
| Requested location access type (see NOTE 2) | O (see NOTE 1) | Identifies the location access type for which the location information is requested, e.g. as described in TS 23.273 [50] and TS 29.572 [51]. |
| Requested positioning method | O (see NOTE 1) | Identifies the positioning method for which the location information is requested, e.g. as described in TS 23.273 [50] and TS 29.572 [51]. |
| Requested velocity information | O (see NOTE 1) | Identifies the velocity of the target UE for which the location information is requested. |
| Triggering criteria | O (see NOTE 1) | Identifies when the location management client will send the location report as specified in table 9.3.2.4-2. The triggering may be e.g., SAI changes, ECGI changes, RAT changes. |
| Minimum time between consecutive reports | O (see NOTE 1) | Defaults to 0 if absent otherwise indicates the time interval between consecutive reports |
| NOTE 1: If none of the information element is present, this represents a cancellation for location reporting. NOTE 2: The non-3GPP access as defined in 23.273[50] and TS 29.572 [51] is out of scope of the present specification. |  |  |

* * * End of Changes * * * *