3GPP TSG-SA WG6 Meeting #62	S6-243708

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243658, 3504, 3347)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0683 | rev | 3 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhance the EAS instantiation considering updated EAS instantiation time |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Samsung, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As for the EAS discovery feature, the case where the EAS instantiation in progress is missing. Whether to identify the EAS which EAS instantiation is in progress or not is not clear yet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Enhance the EAS discovery procedure, EAS determination should take real-time EAS instantiation time in to consideration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EAS discovery will remain uncomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.5.2.2	Request-response model

Pre-conditions:

1.	The EEC has received information (e.g. URI, IP address) related to the EES;

2.	The EEC has received appropriate security credentials authorizing it to communicate with the EES as specified in clause 8.11; and

3.	The EES is configured with ECSP's policy for EAS discovery.

NOTE 1:	Details of ECSP's policy are out of scope.

Figure 8.5.2.2-1: EAS Discovery procedure

1.	The EEC sends an EAS discovery request to the EES. The EAS discovery request includes the requestor identifier [EECID] along with the security credentials and may include EAS discovery filters, EEC service continuity support, and may also include UE location to retrieve information about particular EAS(s) or a category of EASs, e.g. gaming applications, or Edge Applications Server(s) available in certain service areas, e.g. available on a UE's predicted or expected route. The request may include an EAS selection request indicator.

2.	Upon receiving the request from the EEC, the EES checks if the EEC is authorized to discover the requested EAS(s). The authorization check may apply to an individual EAS, a category of EASs or to the EDN, i.e. to all the EASs. If UE's location information is not already available, the EES obtains the UE location by utilizing the capabilities of the 3GPP core network as specified in clause 8.10.3. If EAS discovery filters are provided by the EEC, but it does not contain Application group profile, the EES identifies the EAS(s) based on the provided EAS discovery filters and the UE location.

When the bundle EAS information is provided, then;

-	If bundle EAS information includes EAS bundle identifier, the EES identifies all or part of the EAS(s) associated with the same EAS bundle identifier.

-	If bundle EAS information includes a list of EASIDs, the EES identifies the EASs which are all or part of the EAS bundle.

If the EEC indicates that service continuity support is required, when identifying the EAS, the EES shall take the indication which ACR scenarios are supported by the AC, the EEC, the EES and the EAS and which of these are preferred by the AC into consideration. The EES may select one EAS and determine whether to perform application traffic influence for this AC based on AC's service KPI or EAS’s service KPI in desired response time, when the EAS does not perform traffic influence in advance.

If the Prediction expiration time is provided then the EES may determine whether to identify the instantiable but not instantiated EAS as T-EAS based on Prediction expiration time and the predicted EAS deployment time information obtained from ADAES as specified in clause 8.11 of TS 23.436 [28] or from local configured maximum EAS deployment time. The EES determines remaning EAS instantiation time or EAS instantiation completion time based on the timing receiving EAS instantiation is in progress from ECSP management system and the predicted EAS deployment time. Furthermore, if EES received the indication which the EAS instanitation is in progress, then the EES determines whether to identify the EAS which instantiation in progress as T-EAS based on Prediction expiration time and remaining EAS instantiation time information.

When EAS discovery filters are not provided, then:

-	if available, the EES identifies the EAS(s) based on the UE-specific service information at the EES and the UE location;

-	EES identifies the EAS(s) by applying the ECSP policy (e.g. based only on the UE location);

When EAS discovery filters contain Application group profile, the EES checks whether information about common EAS and related Application Group ID is available or not. If the common EAS information related to the Application Group ID is:

-	not available, then based on the policy if EES needs to select the common EAS, the EES identifies an EAS for the Application Group ID based on the provided EAS discovery filters such as KPIs, UE-specific service information or the ECSP policy. Furthermore, the EES stores the common EAS information and related Application Group ID.

-	available at the EES, then the EES provides information of that EAS as result for EAS discovery.

Furthermore, if the E2E response time is received:

    - when the UE in the overlapping area between the EDNs of common EAS and EAS registered to this EES (e.g. UE can connect either common EAS or EAS registered to this EES), the EES identifies either common EAS or EAS registered to this EES for the UE based on the common EAS KPI as specified in table 8.2.5 and EDN information, registered EAS KPI as specified in table 8.2.5 and EDN information, and inter-EAS communication performance of these EASs.

    - When the UE is not in the overlapping area between the EDNs of common EAS and EAS registered to this EES (e.g. UE is not in the common EAS service area), then the EES identifies candidate EAS(s) for the group from its registered EASs and checks if the candidate EAS(s) can satisfy the E2E response time considering the response time of the candidate EAS(s), the response time of available common EASs in other EDNs and the inter-EAS communication latency. If the E2E response time cannot be satisfied, the EES will respond the EEC in step 3 indicating such a rejection reason; otherwise, the EES selects a candidate EAS and interacts with the ECS-ER to store it as common EAS as described in clause 8.20.2.3

NOTE 2:	The EES can use ADAE analytics service in 3GPP TS 23.436 [28] or local configuration to obtain application performance (e.g. latency) for inter-EAS communication.

NOTE 3:	If the current EDN has overlapping area (where UE resides) with other EDN and both EDNs has available common EAS which can satisfy E2E response time, which EAS to select can take number of UEs in the group connected to available common EAS into consideration.

NOTE 4:	The EES may have previously determined and stored the common EAS for Application group ID, or the EES may have received the common EAS selection information for Application group ID during the common EAS announcement procedure.

When the ECS-ER is not available and the EES selects the common EAS, the selected common EAS shall be announced to other EES(s) as per procedure specified in clause 8.19.

When the ECS-ER is available, then

if E2E response time is received,

-	If the UE is in the overlapping area between the EDNs of common EAS and EAS registered to this EES (e.g. UE can connect either common EAS or EAS registered to this EES), the EES identifies either common EAS or EAS registered to this EES for the UE based on the common EAS KPI as specified in table 8.2.5 and EDN information, registered EAS KPI as specified in table 8.2.5 and EDN information, and inter-EAS communication performance of these EASs.

-	otherwise, the EES checks with the ECS-ER about all available common EASs and their corresponding EDNs by using procedure defined in clause 8.20.2.2. If no common EAS is available for the group, EES identifies one EAS for the group and interacts with the ECS-ER to store the common EAS information as described in clause 8.20.2.3. If current EDN already has available common EAS, the EES selects such common EAS considering the required E2E response time. If there are available common EASs in other EDNs, the EES identifies candidate EAS(s) for the group from its registered EASs and checks if the candidate EAS(s) can satisfy the E2E response time considering the response time of the candidate EAS(s), the response time of available common EASs in other EDNs and the inter-EAS communication latency. If the E2E response time cannot be satisfied, the EES will respond the EEC in step 3 indicating such a rejection reason; otherwise, the EES selects a candidate EAS and interacts with the ECS-ER to store it as common EAS as described in clause 8.20.2.3.

NOTE 5:	The EES can use ADAE analytics service in 3GPP TS 23.436 [28] or local configuration to obtain application performance (e.g. latency) for inter-EAS communication.

If E2E response time is not received and common EAS is not available, the EES identifies one EAS for the group and interacts with the ECS-ER to store the common EAS information as described in clause 8.20.2.3. If common EAS information is already available corresponding to the Application Group ID in the repository, then the ECS-ER returns the common EAS information to the EES as described in clause 8.20.2.3.

NOTE 6:	Details of the UE-specific service information and how it is available at the EES is out of scope.

NOTE 7:	Both steps are evaluated prior to sending a response.

Upon receiving the request from the EEC, the EES may also collect edge load analytics from ADAES (as specified in clause 8.8.2 of TS 23.436 [28]) or performance data from OAM to find whether the EAS(s) satisfies the Expected AC service KPIs or the Minimum required AC Service KPIs.

Upon receiving the request from the EEC, if the EEC does not indicate EAS Instantiation Triggering Suppress in the EAS Discovery request, the EES may trigger the ECSP management system to instantiate the EAS that matches with EAS discovery filter IEs (e.g. ACID) as in clause 8.12.

Otherwise, upon receiving the request from the EEC, if the EEC indicates EAS Instantiation Triggering Suppress in the EAS Discovery request and the EES supports such capability, the EES determines not triggering the ECSP management system to instantiate the EAS and may determine Instantiable EAS Information for EAS(s) that are instantiable but not yet instantiated and match the EAS discovery filter IEs. Instantiable EAS Information is provided in the EAS Discovery response and includes the EASID(s) and, for each EASID, the status indicating whether the EAS is instantiated or instantiable but not yet instantiated.

If the EEC provides in the EAS discovery request the EAS selection request indicator, the EES selects EAS satisfying the EAS discovery filter or based on other information (e.g. ECSP policy) as described above (if no EAS discovery filter received), and then provides the selected EAS information to the EEC in the discovered EAS list of EAS discovery response.

NOTE 8:	Without EAS selection request indication, the EES handling is as per R17 procedure.

3.	If the processing of the request was successful, the EES sends an EAS discovery response to the EEC, which includes information about the discovered EASs and Instantiable EAS Information. For discovered EASs, this includes endpoint information. Depending on the EAS discovery filters received in the EAS discovery request, the response may include additional information regarding matched capabilities, e.g. service permissions levels, KPIs, AC locations(s) that the EASs can support, ACR scenarios supported by the EAS, etc. The EAS discovery response may contain a list of EASs and Instantiable EAS Information with EAS instantiation completion time. This list may be based on EAS discovery filters containing a Geographical or Topological Service Area, e.g. a route, included in the EAS discovery request by the EEC. When the discovered EAS is for a certain application group, then the Application Group ID is also included in the response message. If the discovered EAS is registered to another EES, then the EES endpoint of the EES where the discovered EAS is registered is also included in the response message.

When the EES accepts the request and determines to trigger the EAS instantiation, then the response may indicate that the EAS instantiation is in progress so that the detailed EAS profile information will be available later. Then the EES determines the remaning EAS instantiation time or EAS instantiation completion time based on the timing receiving EAS instantiation in progress ECSP management system and the predicted EAS deployment time. When EEC receives the EAS instantiation in progress indication, the EEC may send EAS discovery subscription request message if not subscribed yet or send EAS discovery request message later to the EES for obtaining updated EAS information.

If the EES is unable to determine the EAS information using the inputs in the EAS discovery request, UE-specific service information at the EES or the ECSP policy, the EES shall reject the EAS discovery request and respond with an appropriate failure cause.

If the EEC is not registered with the EES, and ECSP policy requires the EEC to perform EEC registration prior to EAS discovery, the EES shall include an appropriate failure cause in the EAS discovery response indicating that EEC registration is required.

If the UE location and predicted/expected UE locations, provided in the EAS discovery request, are outside the Geographical or Topological Service Area of an EAS, then the EES shall not include that EAS in the discovery response. The discovery response may include EAS(s) that cannot serve the UE at its current location if a predicted/expected UE location was provided in the EAS discovery request.

Upon receiving the EAS discovery response, if the EEC selects an EAS which is instantiated (i.e., an EAS profile was provided), the EEC uses the endpoint information for routing of the outgoing application data traffic to EAS(s), as needed, and may provide necessary notifications to the AC(s). The EEC may use the border or overlap between EAS Geographical Service Areas for service continuity purposes. The EEC may cache the EAS information (e.g. EAS endpoint) for subsequent use and avoid the need to repeat step 1. If the Lifetime IE is included in the response, the EEC may cache the EAS information only for the duration specified by the Lifetime IE.

Upon receiving the EAS discovery response, if the EEC selects an EAS which is instantiable but not yet instantiated (i.e. an EAS profile is not provided), the EEC sends the EAS information provisioning request indicating the selected EASID as in clause 8.15. If the EAS discovery response message contains the EAS(s) information along with r EAS instantiation completion time which the EAS instantiation in progress, then the EEC determines whether to identify the EAS which instantiation in progress as T-EAS based on Prediction expiration time and remaining EAS instantiation time/EAS instantiation completion time.

NOTE 9:	Within the duration specified by the Lifetime IE, the cached EAS Profile can be updated (e.g. according to notifications from the EES for changes of EAS information due to EAS status change) or the cached EAS Profile can be invalidated due to new EAS information discovery (e.g. due to UE mobility). The EEC can update or invalidate the cached EAS information (e.g. on PDU Session Release or Modification Command).

NOTE 10:	The AC can cache the EAS information (e.g. EAS endpoint) for subsequent use. In the case of the cached information needing to be updated or invalidated, the mechanisms for the EEC to notify the AC is up to implementation and is not specified in the current release of the present document.

NOTE 11:	The EEC can use the EAS information provided by the discovery procedure to perform service continuity planning, for example when ultra-low latency ACR is required.

If the EAS discovery request fails, the EEC may resend the EAS discovery request, taking into account the received failure cause. If the failure cause indicated that EEC registration is required, the EEC shall perform an EEC registration before resending the EAS discovery request.

NOTE 12:	As long as a proper EAS (e.g. considering expected AC service KPIs included in EAS discovery request) is discovered and selected by the EES, EEC of a constraint UE can stop sending EAS discovery to rest candidate EES(s), and provide the selected EAS information to AC.

8.5.3.3	EAS discovery response

Table 8.5.3.3-1 describes information elements for the EAS discovery response from the EES to the EEC.

Table 8.5.3.3-1: EAS discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the EAS discovery request was successful. |
| > Discovered EAS list (NOTE 1, NOTE 3) | O | List of discovered EAS(s). Each element includes the information described below. |
| >> EAS profile | M | Profile of the EAS. Each element is described in clause 8.2.4 |
| >> Application Group ID list (NOTE 4) | O | List of Application Group IDs associated with EAS |
| >> EES Endpoint | O | The endpoint address (e.g. URI, IP address) of the EES. |
| >> Lifetime | O | Time interval or duration during which the information elements in the EAS profile is valid and supposed to be cached in the EEC (e.g. time-to-live value for an EAS Endpoint) |
| > Analytics information | O | Analytics data for each discovered application server. |
| >> Statistical data | O | Indicates the statistical analytics data (e.g. number of times the client received expected performance from the EAS). |
| >> Predictive data | O | Indicates predictive analytics data for EAS service status (e.g. EAS schedule, EAS status) change. |
| > Instantiable EAS Information (NOTE 1) | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not instantiated yet, instantiation in progress). |
| > EAS instantiation completion time | O | Indidates the time when the EAS instantiation is complete. |
| >> Instantiation criteria (NOTE 2) | O | The criteria upon which EAS can be instantiated (e.g. based on specific date and time). |
| Failure response | O | Indicates that the EAS discovery request failed. |
| > Cause | O | Indicates the cause of EAS discovery request failure. |
| NOTE 1: At least one of the IEs must be included in the Successful response. NOTE 2: "Instantiation criteria" IE shall be present only when the value of "Instantiable EAS information" IE is "instantiable but not be instantiated yet". NOTE 3: If EAS discovery is used for ENS scenario, discovered EAS list contains only those EASs which are allowed to be used by the subscribers of the serving MNO. NOTE 4: "Application Group ID list" IE shall be present when "Application Group profile" is included for "AC profile" in EAS discovery request as specified in clause 8.5.3.2 |  |  |

8.8.3.2	Discover T-EAS

Figure 8.8.3.2-1 illustrates the procedure for fetching T-EAS information. This procedure may be utilized by a S-EAS, which undertakes the transfer of application context information to a T-EAS directly, or can be invoked by the S-EES itself on deciding to execute ACR.

T-EAS discovery procedure also supports EAS retrieval which enables a S-EAS to obtain T-EAS(s) serving the application group so that the S-EAS can start communication with obtained EAS(s) for EAS content synchronization.

Pre-conditions:

1.	Information related to the EES is available with the S-EAS, if the procedure is triggered by the S-EAS.

Figure 8.8.3.2-1: Discover T-EAS

1a.	The S-EAS sends the EAS discovery request to the S-EES or the S-EES decides to execute the ACR. The EAS discovery request from the S-EAS includes the requestor identifier [EASID] along with the security credentials and includes EAS discovery filter matching its EAS profile. If target DNAI is available at the S-EAS via User Plane Path change event, the S-EAS provides the S-EES with the target DNAI. The S-EAS also includes an EAS service continuity support indicator indicating that the S-EAS decided ACR according to clause 8.8.2.4 is to be used for the ACR. The S-EAS includes the bundle ID and bundle type indicating the proxy bundle case to which the S-EAS belongs to. The request may include prediction expiration time.

The EAS may send EAS discovery request with EAS ID, Application Group ID and EAS content synchronization support, which indicates the request to obtain EAS(s) currently serving the Application Group ID with the requested EAS ID in order to perform EAS content synchronization.

The Application group ID is included in the EAS discovery request if the S-EAS wants to discover a common T-EAS for UE in the group due to UE's movement.

If the EAS discovery request is triggered by S-EAS due to overload or maintenance reason for the Application Group, the S-EAS also includes list of UE IDs in the EAS discovery request.

NOTE 1:	The trigger condition to invoke the Discover T-EAS API is up to application service logic, which is out of scope of this specification.

1b.	The S-EES either receive the target DNAI for T-EES discovery from the step 1a or by the user plane management event notification from the core network.

2.	If the request is received from the S-EAS, the S-EES checks whether the requesting EAS is authorized to perform the discovery operation.

If Application Group ID and EAS content synchronization support in EAS characteristics are received and the ECS-ER is available, the S-EES checks with the ECS-ER with the received EAS ID and Application Group ID and obtains a list of EAS(s) supporting EAS content synchronization and serving the application group for the desired application service identified by the EAS ID as described in clause 8.20. Step 2 to step 4 are skipped.

If the UE location is not known to the S-EES or provided by the S-EAS request, then the S-EES may interact with 3GPP core network to retrieve the UE location. If the S-EES decided to execute the ACR or when the requesting EAS is authorized, the S-EES checks if there exists a T-EAS information (registered or cached) that can satisfy the requesting EAS information, additional query filters and the Expected AC Service KPIs and the Minimum required AC Service KPIs if received from the EEC during the EAS discovery or from the S-EAS in step 1. In this case, the S-EES may collect Edge load performances from ADAES or OAM to find T-EAS(s) that satisfies the Expected AC service KPIs or the Minimum required AC Service KPIs. The S-EES may determine the use of statistics or prediction for evaluating KPIs based on the situation of the T-EAS discovery. The S-EES also considers resource consumption needs for the list of UEs (e.g. received in step 1a) when identifying T-EAS(s). If Application Group ID is not received and if the S-EES finds the T-EAS(s) in the cached or registered information, the flow either continues with step 5 for the S-EAS triggered discovery or stops for the S-EES decided ACR execution, else the S-EES retrieves the T-EES address from the ECS as specified in clause 8.8.3.3 and continues with step 3.

If UE ID list is available in the S-EES (e.g. received in step 1a or known by the S-EES via other means):

-	for UE(s) not in the overlapping EDN area, the S-EES tries to identify a T-EAS within locally registered EAS(s).

-	for UE(s) in the overlapping EDN area, the S-EES checks if there is available common EAS in each EDN which has overlapping EDN area with the current EDN by using procedure described in clause 8.20.2.2. If any common EAS is found, the S-EES decides whether to select an available common EAS or locally registered T-EAS as common EAS considering E2E response time and continues with step 5; otherwise, the S-EES tries to identify a T-EAS within locally registered EAS(s).

Updating common EAS with registered T-EAS in the ECS-ER is needed as described in clause 8.20.2.5 when locally registered T-EAS is selected by the S-EES. If no registered T-EAS can be found, the S-EES continues with step 5.

If Application group ID is available in the S-EES (e.g. received in step 1a or ACR management event subscription), the S-EES retrieves T-EES information and corresponding EDN connection information from the ECS as described in clause 8.8.3.3.

If Prediction expiration time is provided then the EES may determine whether to identify the instantiable but not instantiated EAS as T-EAS based on Prediction expiration time and the predicted EAS deployment time information obtained from ADAES as specified in clause 8.11 of TS 23.436 [28] or from local configured maximum EAS deployment time.

3.	The S-EES invokes the EAS discovery request on the T-EES retrieved from the ECS. The EAS discovery request includes the requestor identifier [EESID] along with the security credentials and includes EAS discovery filter. In the EAS discovery filter, the S-EES may include prediction expiration time, the Expected AC Service KPIs and the Minimum required AC Service KPIs if received from the EEC during the EAS discovery or from the S-EAS in step 1.

The S-EES also includes the EEC service continuity support indicator received from the EEC during EAS discovery. If in step 1 the S-EES received an EAS service continuity support indicator from the S-EAS, then the S-EES includes this EAS service continuity support indicator and its own EES service continuity support indicator indicating the ACR scenarios supported by the EES. If in step 1 the S-EES decided to execute the ACR, the S-EES includes the EAS service continuity support indicator received from the S-EAS during EAS registration and includes an EES service continuity support indicator indicating that the S-EES executed ACR according to clause 8.8.2.5 is to be used for the ACR.

Upon receiving the request, the T-EES may trigger the ECSP management system to instantiate the T-EAS that matches with EAS discovery filter IEs (e.g. ACID) as in clause 8.12.

4.	The T-EES discovers the T-EAS(s) and responds with the discovered T-EAS information to the S-EES. To filter T-EAS(s), the T-EES utilizes the discovery filters (e.g. Expected AC Service KPIs and the Minimum required AC Service KPIs) and the indications which ACR scenarios are supported by the AC, the EEC, the T-EES and the S-EAS. If T-EES gets the Expected AC service KPIs or the Minimum required AC Service KPIs, the T-EES may collect edge load analytics from ADAES (as specified in clause 8.8.2 of TS 23.436 [28]) or performance data from OAM to find T-EAS(s) that satisfies the Expected AC service KPIs or the Minimum required AC Service KPIs. The T-EES may determine the use of statistics or prediction for evaluating KPIs based on the situation of the T-EAS discovery. The S-EES may cache the T-EAS information.

When the T-EES accepts the request and determines to trigger the EAS instantiation, then the response may indicate that the EAS instantiation is in progress so that the detailed EAS profile information will be available later. When S-EES receives the EAS instantiation in progress indication, the S-EES may send EAS discovery subscription request message if not subscribed yet or send EAS discovery request message later to the T-EES for obtaining updated EAS information.

When the bundle EAS information (i.e. list of EASID) is provided and the bundle type indicating the direct bundle, and the S-EES received associated T-EES(s) along with part of EAS ID list in the step2 from the ECS, then the S-EES discover the target direct bundle EAS(s) which belongs to same EDN for all the associated S-EES(s). The request message contains direct bundle EAS(s) information (i.e. list of EASID and direct bundle type). Then the S-EES receives the direct bundle T-EAS(s) information from each associated T-EES(s).

NOTE 2:	T-EES(s) may belongs to same EDN.

NOTE 3:	The edge load analytics from ADAES can be either statistics or predictions on the T-EAS.

NOTE 4:	The statistical KPI value can be used for both normal ACR and service continuity planning.

When the ECS-ER is available and common EAS information corresponding to the Application Group ID (received in step 1a or ACR management event subscription) is not available, then the S-EES identifies one T-EAS for the application group and interacts with the ECS-ER to store the common EAS information as described in clause 8.20.2.3. If common EAS information is already available corresponding to the Application Group ID in the repository, then the ECS-ER returns the common EAS information to the S-EES as described in clause 8.20.2.3.

5.	If the request was received from the S-EAS, the S-EES responds to the S-EAS with the discovered T-EAS Information.

For responding to the S-EAS which is requesting EAS(s) serving the application group for EAS content synchronization purpose, only EAS endpoint and EAS ID are included in EAS profile of Discovered EAS list.

8.3.3.2.2	Request-response model

Figure 8.3.3.2.2-1 illustrates service provisioning procedure based on request/response model.

Pre-conditions:

1.	The EEC has been pre-configured or has discovered the address (e.g. URI) of the ECS;

2.	The EEC has been authorized to communicate with the ECS;

3.	The UE Identifier is either preconfigured or resulted from a successful authorization; and

4.	The ECS is configured with ECSP's policy for service provisioning.

NOTE 1:	Details of ECSP's policy are out of scope.

Figure 8.3.3.2.2-1: Service provisioning – Request/Response

1.	The EEC sends a service provisioning request to the ECS. The service provisioning request includes the security credentials of the EEC received during EEC authorization procedure and may include the UE identifier such as GPSI, connectivity information, UE location, EEC service continuity support and AC profile(s) information. EEC may provide its desired ECSP identifier(s) in the service provisioning request based on EEC preference.

2.	Upon receiving the request, the ECS performs an authorization check to verify whether the EEC has authorization to perform the operation. The ECS may utilize the capabilities (e.g. UE location) of the 3GPP core network as specified in clause 8.10.2. If the UE serving PLMN identifier is not provided by the EEC in the connectivity information of the service provisioning request, the ECS may invoke the NEF monitoring event API as described in 3GPP TS 23.502 [43] and 3GPP TS 23.682 [17] to obtain the UE roaming status and serving PLMN identifier. If the UE is roaming, the ECS may use the serving PLMN identifier to determine the roaming partner ECS (i.e. V-ECS) information to be provided to the EEC in the service provisioning response. If the  Prediction expiration time is provided then the ECS may determine whether to identify EES with the instantiable but not instantiated EAS based on the Prediction expiration time and predicted EAS deployment time information obtained from ADAES as specified in clause 8.11 of TS 23.436 [28] or from local configured maximum EAS deployment time. If AC profile(s) are provided by the EEC, and the Application group profile is not provided, the ECS identifies the EES(s) based on the provided AC profile(s) and the UE location.

When Application group profile is provided in the request, which applies to the common EAS case:

-	if the ECS-ER is not available, then

-	the ECS identifies EES(s) based on the information contained in the request (e.g.AC profile, Application group profile, UE location), specifically, the ECS identify the EES(s) based on the EASID and expected group geographical service area in the application group profile and the EAS ID supported by the EES and EES(s) service area in EES profile(s);

-	if the ECS-ER is available, and:

-	EES information is not available corresponding to the Application Group ID, then the ECS identifies EES(s) and stores the identified EES(s)'s information and related Application group ID into the ECS-ER; specifically, the ECS identify the EES(s) based on the EASID and expected group geographical service area in the application group profile and the EAS ID supported by the EES and EES(s) service area in EES profile(s); or

-	EES information is available corresponding to the Application Group ID, then the ECS retrieves the EES (s) information corresponding to the Application Group ID from the ECS-ER.

NOTE 2:	It is up to the ASP or the EES to determine validity of the application group.

The ECS may take Group Geographical Service Area information and KPI requirements of the AC to determine the EES(s) corresponding to the Application group ID.

When neither Application group profile nor AC profiles(s) are provided, then:

-	if available, the ECS identifies the EES(s) based on the UE-specific service information at the ECS and the UE location;

-	ECS identifies the EES(s) by applying the ECSP policy (e.g. based only on the UE location);

Furthermore, the ECS may identify the EES based on the EEC service continuity support information and EES service continuity support information.

The ECS may take the EAS(s) load information corresponding to the EASIDs registered with EES in the EDN(s) to determine the EES(s) corresponding to the provided AC profile(s).

Editor' Note:	The system impact of using EAS load info registered in ECS is FFS.

NOTE 3:	Details of the UE-specific service information and how it is available at the ECS is out of scope.

NOTE 4:	Both steps are evaluated prior to sending a response.

If desired ECSP identifier(s) is provided by the EEC, the ECS identifies the EES(s) to be sent in step 3 based on registered ECSP identifier in EES profile and the desired ECSP identifier(s).

NOTE 5:	For EEC desired ECSP identifier usage, it is assumed that the ECSP providing the EES and PLMN operator are the same organization and an ECSP providing the EES (desired by the EEC) registers its EES in ECS provided by another ECSP based on service agreement to provide services to EEC.

The ECS also determines other information that needs to be provisioned, e.g. identification of the EDN, EDN service area, EES endpoints.

For the roaming and federation case, if ECS does not identify any suitable EES(s) based on EDN configuration available at the ECS and UE's location, the ECS determines a partner ECS that may satisfy the requirements. Based on ECSP policy, the ECS may use preconfigured or OAM configured information about the partner ECSs or ECS discovery via ECS-ER as specified in clause 8.17.2.3 or both.

If required by the ECSP policies, the ECS may use service provisioning information retrieval procedure as specified in clause 8.17.2.4 to obtain service provisioning information from the partner ECS.

NOTE 6:	ECSP policies can restrict sharing partner ECSP's information with the EEC.

When the bundle EAS information is provided, which applies to the bundle EAS case then;

-	If bundle EAS information includes EAS bundle identifier, the ECS identifies all the EES(s) providing the same EAS bundle identifier.

-	If bundle EAS information includes a list of EASIDs, the ECS identifies the one or more EES which support all of the EASs within the same EDN based on the EDN information obtained in the EES profile.

If both Application group profile and bundle EAS information are provided in the request, then ECS considers them as common EAS bundle information and identifies all the EES(s) by utilizing the both Application group profile and bundle EAS information.

3.	If the processing of the request was successful, the ECS responds to the EEC's request with a service provisioning response. If the ECS has identified the relevant EES(s) information, the service provisioning response includes a list of EDN configuration information, e.g. identification of the EDN, EDN service area, and the required information (e.g. URI, IP address) for establishing a connection to the EES.

The ECS may provide associated EES(s) information (one or more EES information) in the service provisioning response along with the bundle EAS information.

If the alternative ECS(s) has been identified in step 2, the ECS sends a successful response including the Redirect information element containing the list of ECS(s) configuration information indicating that alternative ECS(s) is available for service provisioning request. The response may include information such as DNN and S-NSSAI for roaming UEs to establish a PDU session with the ECS as specified in 3GPP TS 23.548 [20].

If the ECS is not provisioned with any EDN configuration information or is unable to determine either the EES information or the partner ECS information using the inputs in service provisioning request, UE-specific service information at the ECS or the ECSP's policy, the ECS shall reject the service provisioning request and respond with an appropriate failure cause.

If the service provisioning response contains a list of ECS configuration information, the EEC may initiate service provisioning procedure with one or more ECS(s) provided in the response. If the UE is roaming to a V-PLMN and the ECS configuration information includes V-PLMN ID in the list of Supported PLMN ID(s), the EEC may establish a connection with the V-ECS based on parameters such as UE serving PLMN ID and supported PLMN ID(s) of the V-ECS received in the response message as specified in 3GPP TS 23.548 [20]. The connection with the V-ECS can be a HR-SBO PDU session or an LBO PDU session based on the information received from the ECS.

If the EDN configuration information includes an LADN DNN as an identifier for the EDN, the EEC considers the LADN as the EDN. Therefore, the service area of EDN is the LADN Service Area which can be discovered using the UE Registration Procedure.

The EEC may cache the service provisioning information (e.g. EES endpoint) for subsequent use and avoid the need to repeat step 1. If the Lifetime IE is included in the Service provisioning response, then the EEC may cache and reuse the Service provisioning information only for the duration specified by the Lifetime IE, without the need to repeat step 1.

If the ECS provided information regarding the service continuity support of individual EESs, the EEC may take this information into account when selecting an EES for EEC registration, EAS discovery or T-EAS discovery, respectively.

If for multiple EES(s), the instantiable EAS information IE for an EAS is not available or the instantiable EAS information IE is set to instantiated or instantiable, the EEC can select one or more such EES to perform EAS discovery. For EAS discovery to mitigate the waste of EDN resources EEC considers the instantiable EAS information and the associated instantiation criteria, the EEC selects one EES, if the EAS instantiation status corresponding to the EASID requested by AC/EEC is instantiable but not yet instantiated (i.e. no instantiated EAS).

NOTE 7:	If the service provisioning request fails, the EEC can resend the service provisioning request again, taking into account the received failure cause.

NOTE 8:	For example, using a particular DNAI to reach the data network containing the EES might be necessary to meet AC service KPIs.

NOTE 9:	If the EAS instantiation fails based on the selected EES, the EEC may retry the EAS discovery request to another EES (e.g. selecting another one EES based on the instantiable EAS information).

8.8.3.3	Retrieve T-EES procedure

Figure 8.8.3.3-1 illustrates the procedure for the S-EES to retrieve the T-EES information from the ECS.

Pre-condition:

1.	The S-EES has been pre-configured with the address of the ECS; and

2.	The AC at the UE already has on-going application traffic with the S-EAS.

Figure 8.8.3.3-1: Retrieve T-EES procedure

1.	The S-EES sends the Retrieve EES request (UE location information or UE identity, EASID of the S-EAS, bundle ID, bundle type (i.e. proxy bundle case), target DNAI and UE connectivity information) to the ECS in order to identify the T-EES which has an EAS available to serve the given AC in the UE. For obtaining announcement EES list, the Retrieve EES request includes application group id. The request message may also contain the AC, EEC service continuity support information.

2.	If the request contains the UE identity (e.g. GPSI) but the UE location is not known to the ECS, then the ECS interacts with 3GPP core network to retrieve the UE location. The ECS determines T-EES(s) as per the parameters (e.g. EASID, target DNAI) in the request and the UE location information. If the request message contains the AC, EEC service continuity support information, then the ECS may identify the T-EES taking the AC, EEC, T-EES service continuity support into consideration. The ECS may take the EAS(s) load information corresponding to the EASIDs registered with EES in the EDN(s) or use ADAES Edge load analytics as specified in clause 8.8.2 of TS 23.436 [27]) to take EDN load into consideration in identifying T-EES(s). When the bundle ID and is provided and bundle type indicating the proxy bundle case then the ECS can identify the T-EES based on the bundle ID in the EES profile and in the request message to ensure T-EAS is able to invoke the required proxy bundle EAS(s) as the S-EAS does.

Editor' Note:	Whether the ECS can obtain EDN load information via other mechanism (e.g., OAM) is FFS, and the system impact of using EAS load info registered in ECS is FFS.

If the Prediction expiration time is provided then the ECS may determine whether to identify T-EES with the instantiable but not instantiated EAS based on Prediction expiration time and predicted EAS deployment time information obtained from ADAES as specified in clause 8.11 of TS 23.436 [28] or from local configured maximum EAS deployment time.

If no ECS-ER is available and when the Retrieve EES request includes application group id to the ECS then the request EES list retrieval is for the announcement of common EAS, ECS determines the list of EESs serving the EASs (with same EASID) for the Group ID included in the Retrieve EES request.

If the ECS does not identify any suitable EES(s) based on EDN configuration available at the ECS and UE’s location, the ECS determines a partner ECS that may satisfy the requirements. Based on ECSP policy, the ECS may use preconfigured or OAM configured information about the partner ECSs or ECS discovery via ECS-ER as specified in clause 8.17.2.3 or both.

If required by the ECSP policies, the ECS may use service provisioning information retrieval procedure as specified in clause 8.17.2.4 to obtain service provisioning information from the partner ECS.

When the bundle EAS information is provided, then if bundle EAS information includes a list of EASIDs, then the ECS identifies the one or more T-EES associated with the same EDN which support all of the EASs within the same EDN based on the EES EDN information obtained in the EES profile.

NOTE 1:	T-EES(s) may belongs to same EDN.

3.	The ECS sends the Retrieve EES response. If the ECS has determined the EDN configuration information, the retrieve EES response includes the list of EDN configuration information to the S-EES. The list of EDN configuration information includes the EDN details with the endpoint information of T-EES(s) as described in table 8.3.3.3.3-2. If the ECS has determined suitable partner ECS(s) instead, the retrieve EES response includes a list of ECS configuration information.

The ECS may provide associated T-EES(s) information (one or more T-EES information) to the S-EES in the Retrieve T-EES response along with the bundle EAS information (i.e. list of EASIDs). When S-EES receives multiple associated T-EES(s), then each associated T-EES information is provided along with the part of the EAS ID list.

If the retrieve EES response contains a list of ECS configuration information, the S-EES may initiate retrieve T-EES procedure with one or more ECS(s) listed in the retrieve EES response.

NOTE 2:	The Retrieve EES request initiated by the S-EES can be restricted only to its registered ECS.

* * * END of Change * * * *