3GPP TSG-SA WG6 Meeting #62	S6-243727

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243049, S6-243593, S6-243662, S6-243700)

Source:	InterDigital Inc.

Title:	Pseudo-CR on AIML model distribution

Spec:	3GPP TS 23.482 v0.1.0

Agenda item:	9.10

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This CR brings normative text on AIML model distribution.

2. Reason for Change

The following agreement has been approved in TR 23.700-82 on transfer of AI/ML models.

The following principles will be considered in the normative phase for KI #5.

Transfer of AI/ML models:

-	Solution #24 will be considered for distribution of AIML models (request/response).

-	Solution #25 will be considered for distribution of AIML models (subscribe/notify).

NOTE 2: Possible alignment of the request/response and subscribe/notify models in Solution #24 and Solution #25 will be considered in normative phase.

Solution #24 and Solution #25 have a complementary relationship.

3. Conclusions

This pCR provides normative text for KI#5 Solution #24 and solution #25.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 v0.1.0.

* * * First Change * * * *

## 8.x	ML model Retrieval

### 8.x.1	General

ML model retrieval enable applications consuming services from the AI/ML application enablement layer to retrieve ML models available with the enablement layer.

ML model retrieval enables an AIMLE client or VAL server to obtain ML models of interest. The retrieval of ML models is based on matching ML model retrieval filters provided in the request.

ML model retrieval subscription enables an AIMLE client or VAL server to receive notifications about ML models of interest. The subscription is based on matching ML model retrieval filters provided in the subscription request.

### 8.x.2	Procedure

#### 8.x.2.1	General

Following procedures are supported for ML model retrieval:

-	ML model retrieval request-response procedure;

-	ML model subscribe-notify procedures for retrieval of ML models, comprising:

-	Subscription procedure;

-	Subscription update procedure; and

-	Unsubscribe procedure;

#### 8.x.2.2	ML model retrieval

Figure 8.x.2.2-1 illustrates the procedure for an AIMLE client or VAL server to discover and obtain ML models from the AIMLE server.

Pre-conditions:

- The AIMLE client or VAL server has  received information (e.g. URI, IP address) related to the AIMLE Server;

- The AIMLE client or VAL server has received security credentials authorizing it to communicate with the AIMLE server;

Figure 8.x.2.2-1: ML model retrieval

1.	The requestor (e.g., AIMLE client or VAL server) sends a ML model retrieval request to the AIMLE server. The request includes the requestor identifier along with security credentials and may include ML model retrieval filters.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request.

If the requestor is authorized, the AIMLE server may determine if the requested ML model is available with the AIMLE server based on ML model retrieval filters.

Editor’s Note: Details of AIMLE server interactions with the ML model repository are FFS.

3.	The AIMLE server sends a ML model retrieval response to the requestor. If the AIMLE server has determined ML models (e.g., locally, or from the ML repository), the response includes an indication of success and may include the ML models. Otherwise, the response includes an indication of failure and may include a reason for failure.

Upon receiving a response including ML models, the requestor can store the ML models and may provide the ML model to the VAL client(s) if the requestor is the AIMLE client.

#### 8.x.2.3	ML model retrieval subscription

##### 8.x.2.3.1	General

Clause 8.x.2.3.2 and clause 8.x.2.3.3 together illustrate the ML model retrieval subscribe/notify ML model.

Clause 8.x.2.3.4 illustrates the ML model retrieval subscription update procedure.

Clause 8.x.2.3.5 illustrates the ML model retrieval unsubscribe procedure.

##### 8.x.2.3.2	Subscribe

Figure 8.x.2.3.2-1 illustrates the procedure for an AIMLE client or VAL server to subscribe with the AIMLE server to be notified of retrieval of ML models.

Pre-conditions:

- The AIMLE client or VAL server has received information (e.g. URI, IP address) related to the AIMLE Server;

- The AIMLE client or VAL server has received security credentials authorizing it to communicate with the AIMLE server;

Figure 8.x.2.3.2-1: ML model retrieval subscription

1.	The requestor (e.g., AIMLE client or VAL server) sends a ML model retrieval subscribe request to the AIMLE server. The request includes the requestor identifier, security credentials and may include ML model retrieval filters and expiration time.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server creates the subscription and stores the subscription information.

3.	The AIMLE server sends a ML model retrieval subscribe response to the requestor. If the AIMLE server has created the subscription, the response includes an indication of success, the subscription identity and may include an expiration time; to maintain the subscription, the requestor shall send a subscription update request before the expiration time, otherwise the ML model retrieval subscription expires. If the AIMLE server has not created the subscription, the response includes an indication of failure and may include a reason for failure.

##### 8.x.2.3.3	Notify

Figure 8.x.2.3.3-1 illustrates the ML model retrieval  notify operation between the AIMLE server and an AIMLE client or VAL server.

Pre-conditions:

- The AIMLE client or VAL server has subscribed for ML model retrieval with the AIMLE Server;

Figure 8.x.2.3.3-1: ML model retrieval notification

1.	An event occurs at the AIMLE server that satisfies trigger conditions for notifying a subscriber (e.g., AIMLE client or VAL server). The event may be the detection of availability of new ML model(s) (e.g., new ML model or new revision of a ML model) that satisfies the ML model retrieval filters of the subscription.

Editor’s Note: Details of AIMLE server interactions with the ML model repository to detect trigger conditions are FFS.

2.	The AIMLE server sends a ML model retrieval  notification to the requestor indicating newly available ML models. The notification includes a subscription identity and newly available ML model(s).

##### 8.x.2.3.4	Subscription update

Figure 8.x.2.3.4-1 illustrates the procedure for an AIMLE client or VAL server to update a subscription with the AIMLE server.

Pre-conditions:

- The AIMLE client or VAL server has subscribed for ML model retrieval with the AIMLE Server;

Figure 8.x.2.3.4-1: ML model retrieval subscription update

1.	The requestor (e.g., AIMLE client or VAL server) sends a ML model retrieval update request to the AIMLE server. The request includes the requestor identifier, security credentials and the subscription identifier and may include ML model retrieval filters and expiration time.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server updates the subscription information.

3.	The AIMLE server sends a ML model retrieval subscription update response to the requestor. If the AIMLE server has updated the subscription, the response includes an indication of success and may include an expiration time. To maintain the subscription, the requestor shall send a subscription update request before the expiration time, otherwise the ML model retrieval subscription expires. If the AIMLE server has not updated the subscription, the response includes an indication of failure and may include a reason for failure.

##### 8.x.2.3.5	Unsubscribe

Figure 8.x.2.3.5-1 illustrates the procedure for an AIMLE client or VAL server to unsubscribe with the AIMLE server.

Pre-conditions:

- The AIMLE client or VAL server has subscribed for ML model retrieval with the AIMLE Server;

Figure 8.x.2.3.5-1: ML model retrieval unsubscribe

1.	The requestor (e.g., AIMLE client or VAL server) sends a ML model retrieval unsubscribe request to the AIMLE server. The request includes the requestor identifier, security credentials and the subscription identifier.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server cancels the subscription.

3.	The AIMLE server sends a ML model retrieval unsubscribe response to the requestor. If the AIMLE server has canceled the subscription, the response includes an indication of success. If the AIMLE server has not canceled the subscription, the response includes an indication of failure and may include a reason for failure.

### 8.x.3	Information flows

##### 8.x.3.1	ML model retrieval request

Table 8.x.3.1-1 describes information elements for the ML model retrieval request from the AIMLE client or VAL server to the AIMLE server.

Table 8.x.3.1-1: ML model retrieval request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., AIMLE client or VAL server). |
| Security credentials | M | The security credentials of the requestor. |
| Filtering criteria | O | Represents the filtering criteria, which can be any of the ML model information. |
| > ML model identifier | O | The identifier of the ML model. |
| > Analytics identifier | O | The identifier of the analytics. |

Editor’s Note: The alignment between ML model retrieval request and ML model information discovery request is FFS

##### 8.x.3.2	ML model retrieval response

Table 8.x.3.2-1 describes information elements for the ML model retrieval response from the AIMLE server.

Table 8.x.3.2-1: ML model retrieval response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the ML model retrieval request was successful. |
| List of ML models | O | List of retrieved ML models. |
| > ML model identifier | O | The ML model identifier. |
| > ML model retrieval endpoint (NOTE) | O | The endpoint (e.g., URL, URI, IP address) where the ML model file can be retrieved. |
| > ML model (NOTE) | O | The ML model containing a retrieved ML model. |
| Failure response | O | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: At least one of these elements is needed. |  |  |

Editor’s Note: The alignment between ML model retrieval response and ML model information discovery response is FFS

##### 8.x.3.3	ML model retrieval subscribe request

Table 8.x.3.3-1 describes information elements for the ML model retrieval subscription request from the AIMLE client or VAL server to the AIMLE server.

Table 8.x.3.3-1: ML model retrieval subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., AIMLE client or VAL server). |
| Security credentials | M | The security credentials of the requestor. |
| Notification Target Address | M | Notification target address (e.g. URL) where the notifications should be sent. |
| Proposed expiration time | O | Proposed expiration time for the subscription. |
| Filtering criteria | O | Represents the filtering criteria, which can be any of the ML model information. |
| > ML model identifier | O | The identifier of the ML model. |
| > Analytics identifier | O | The analytics identifier of the analytics. |

Editor’s Note: The alignment between ML model retrieval request and ML model information discovery request is FFS.

##### 8.x.3.4	ML model retrieval subscription response

Table 8.x.3.4-1 describes information elements for the ML model retrieval subscription response from the AIMLE server.

Table 8.x.3.4-1: ML model retrieval subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the ML model retrieval request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |

##### 8.x.3.5	ML model retrieval notification

Table 8.x.3.5-1 describes information elements for the ML model retrieval subscription response from the AIMLE server.

Table 8.x.3.5-1: ML model retrieval notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| List of ML models | O | List of retrieved ML models. |
| > ML model identifier | O | The ML model identifier. |
| > ML model retrieval endpoint (NOTE) | O | The endpoint (e.g., URL, URI, IP address) where the ML model file can be retrieved. |
| > ML model (NOTE) | O | The ML model containing a retrieved ML model. |
| NOTE: At least one of these elements is needed. |  |  |

Editor’s Note: The alignment between ML model retrieval notification and ML model information discovery response is FFS

##### 8.x.3.6	ML model retrieval subscription update request

Table 8.x.3.6-1 describes information elements for the ML model retrieval subscription update request from the AIMLE client or VAL server to the AIMLE server.

Table 8.x.3.6-1: ML model retrieval subscription update request

| Information element |  | Status |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| Requestor identifier |  | M |  | The identifier of the requestor (e.g., AIMLE client or VAL server). |  |
| Security credentials |  | M |  | The security credentials of the requestor. |  |
| Subscription ID |  | M |  | Subscription identifier corresponding to the subscription to be updated. |  |
| Notification Target Address |  | O |  | Notification target address (e.g. URL) where the notifications should be sent. |  |
| Proposed expiration time |  | O |  | Proposed expiration time for the subscription. |  |
| Filtering criteria |  | O |  | Represents the filtering criteria, which can be any of the ML model information. |  |
| > ML model identifier |  | O |  | The identifier of the ML model. |  |
| > Analytics identifier |  | O |  | The analytics identifier of the analytics. |  |

Editor’s Note: The alignment between ML model retrieval subscription update request and ML model information discovery request is FFS

##### 8.x.3.7	ML model retrieval subscription update response

Table 8.x.3.7-1 describes information elements for the ML model retrieval subscription update response from the AIMLE server.

Table 8.x.3.7-1: ML model retrieval subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the ML model retrieval subscription update request was successful. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |

##### 8.x.3.8	ML model retrieval unsubscribe request

Table 8.x.3.8-1 describes information elements for the ML model retrieval unsubscribe request from the AIMLE client or VAL server to the AIMLE server.

Table 8.x.3.8-1: ML model retrieval unsubscribe request

| Information element |  | Status |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| Requestor identifier |  | M |  | The identifier of the requestor (e.g., AIMLE client or VAL server). |  |
| Security credentials |  | M |  | The security credentials of the requestor. |  |
| Subscription ID |  | M |  | Subscription identifier corresponding to the subscription to be unsubscribed. |  |

##### 8.x.3.9	ML model retrieval unsubscribe response

Table 8.x.3.9-1 describes information elements for the ML model retrieval unsubscribe response from the AIMLE server.

Table 8.x.3.9-1: ML model retrieval unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the ML model retrieval unsubscribe request was successful. |
| Failure response | O | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |

* * * End of Changes * * * *