3GPP TSG-SA WG6 Meeting #63	S6-244051

Hyderabad, India, 14th – 18th October 2024	(revision of S6-243639)

Source:	Convida Wireless LLC

Title:	AIMLE data management

Spec:	3GPP TS 23.482 V0.2.0

Agenda item:	9.10

Document for:	Approval

Contact:	Ly.Quang@convidawireless.com, liu.lu@convidawireless.com

1. Introduction

The contribution introduces a procedure for AIMLE data management with necessary modifications for inclusion into TS 23.482.

2. Reason for Change

The conclusion for key issue #3 in TR 23.700-82 stated that solution #21 with modifications can be considered for normative work. The discussion paper S6-244050 describes what AIMLE data management is and why it is needed in AIML_App. It is well known that having quality data is crucial to the successful training of machine learning models. In addition, training data in AIML_App are application centric and therefore requires support for data management operations to ensure quality data are used to train ML models. Data management operations are futher required with Federated Learning due to data privacy concerns since data belongs to the UEs. AIMLE data management includes data collection, data preparation, and exploratory data analysis.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.2.0.

* * *  First Change  * * * *

## 8.x	AIMLE data management

### 8.x.1	General

AIMLE data management consists of data collection, data preparation, and exploratory data analysis. Data collection is the process of obtaining raw application data from a UE for use in AI/ML training or inferencing. Data preparation is the process of processing the raw application data into a format suitable for AI/ML training or inferencing as required by AI/ML models. Exploratory data analysis (EDA) is the process of analyzing data for general patterns and to obtain statistical properties of the data without having direct access to the data. Since Federated Learning requires data privacy, an AIMLE service consumer (e.g. VAL server) utilizes EDA requests to gain an understanding of the application data, e.g. to obtain data distributions and data ranges, to detect anomalies and outliers, and to determine correlations between/among dataset features and other statistical properties. Other types of learning, such as distributed, split AI/ML, and transfer learning, can also benenfit from performing EDA requests.

The following clauses specify procedures, information flows, and APIs for AIMLE data management.

### 8.x.2	Procedure

Figure 8.x.2-1: AIMLE data management

- An AIMLE service consumer (e.g., VAL server) makes a request for AIMLE data management. The request includes information as described in Table 8.x.3.1-1.

- The AIMLE server authenticates and authorizes the request. If authorized, the AMILE server assigns an identifier for the subscription.

- The AIMLE server sends an AIMLE data management subscription response to the AIMLE service consumer. The response includes information as described in Table 8.x.3.1-5.

- The AIMLE server sends AIMLE client data management subscription requests to AIMLE clients. The request can be for data collection, data preparatio, or exploratory data analysis. The request includes information as described in Table 8.x.3.1-6.

- The AIMLE client sends an AIMLE client data management subscription response with information as described in Table 8.x.3.2-1.

- The AIMLE client performs the data management operation necessary to trigger data management at the UE, e.g. from VAL clients. For application data collection, a VAL client on the UE performs data collection locally, which is out of scope of this specification. If data is to be collected from the UE, the AIMLE client performs data collection as described in procedures in 3GPP TS 23.436 or SA4 EVEX mechanism can be reused as described in 3GPP TS 26.531. For data preparation and exploratory data analysis, the VAL client performs the operation locally. If configured, the VAL client performs data collection, data preparation, and exploratory data analysis.

- After completing the data management operation, the AIMLE client sends a notification to the AIMLE server with information as described in Table 8.x.3.2-2.

Based on the received notifications and the data management requirements specified by the AIMLE service consumer, the AIMLE server determines if the data management operations need to be repeated and if the data management subscription request needs to be updated. For example, if the number of collected data samples is not enough, steps 4–7 is repeated to collect more data samples from the existing data producers or from new data producers, or the AIMLE server can update the data collection requirements in the client data management subscription request such as extending the data collection time period or updating the data collection schedule to increase the frequency of data reporting. If necessary, steps 4–7 is repeated to complete all the configured data management operations.

- The AIMLE server receives and aggregates data management notifications from the AIMLE clients and sends a notification to the AIMLE service consumer with information as described in Table 8.x.3.2-3.

### 8.x.3	Information flows

#### 8.x.3.1	AIMLE data management subscription

Table 8.x.3.1-1 shows the request sent by an AIMLE service consumer to an AIMLE server for the AIMLE data management subscription procedure.

Table 8.x.3.1-1: AIMLE data management subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Data management operations | M | An indicator showing what data management type is being requested: data collection, data preparation, exploratory data analysis. |
| Data management requirements | M | Requirements for the data management request: |
| >Data collection requirements | O (NOTE1) | Data collection requirements as detailed in Table 8.x.3.1-2. |
| >Data preparation requirements | O (NOTE1) | Data collection requirements as detailed in Table 8.x.3.1-3. |
| >Exploratory data analysis requirements | O (NOTE1) | Data collection requirements as detailed in Table 8.x.3.1-4. |
| AIMLE clients list | O (NOTE2) | A list of AIMLE clients for which data management should be performed. The list may be specified by AIMLE client set identifier. |
| AIMLE client selection criteria | O (NOTE2) | Selection criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2. |
| NOTE1: At least one of the information elements shall be provided. NOTE2: At least one of the information elements shall be provided. |  |  |

Table 8.x.3.1-2: Data collection requirements

| Information element | Status | Description |
| --- | --- | --- |
| Dataset identifier | M | An identifier for the dataset. |
| Dataset requirements | M | A list of dataset features to be collected. |
| Data collection time period | O | A time period for which the data is to be collected. |
| Data collection schedule | O | A schedule for data collection process, such as the frequency of collecting or reporting data samples. |
| Location for the data collection | O | A location or area for which the data is to be collected. |
| Number of required samples | M | The number of data samples required for the dataset. |

Table 8.x.3.1-3: Data preparation requirements

| Information element | Status | Description |
| --- | --- | --- |
| Dataset identifier | M | An identifier for the dataset. |
| Data preparation requirements | M | Requirements for data preparation. |
| > Dataset feature | M | Identifier or name of dataset feature to process. |
| > Data preparation function | M | Data preparation function to process the data. |
| > Missing data policy | O | A policy to specify what to do if data is missing from the dataset. |

Table 8.x.3.1-4: Exploratory data analysis requirements

| Information element | Status | Description |
| --- | --- | --- |
| Dataset identifier | M | An identifier for the dataset. |
| Dataset features list | M | A list of dataset features for which to perform exploratory data analysis. |
| > Dataset feature ID | M | Identifier or name of dataset feature. |
| > Statistical function | M | Function for which EDA is performed |
| Outlier and anomaly reporting | O | Indicator to report outliers and anomaly found in the dataset |
| Multivariate analysis | O | Requirement for multivariate analysis. |
| > Dataset features list | O | List of dataset features used in multivariate analysis. |
| > Analysis function | O | Analysis function use for multivariate analysis. |
| Output format | O | Output format of the EDA results. |

Table 8.x.3.1-5 shows the response sent by the AIMLE server to the AIMLE service consumer for the AIMLE data management subscription procedure.

Table 8.x.3.1-5: AIMLE data management subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the data management operation |
| Subscription identifier | M | An identifier for the subscription. |
| Expiration time | O | Expiration time for the subscription |

Table 8.x.3.1-6 shows the notification sent by the AIMLE server to the AIMLE service consumer for the AIMLE data management subscription procedure.

Table 8.x.3.1-6: AIMLE data management notify

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the data management operation |
| Data collection outputs | O (NOTE) | Provides outputs for data collection: dataset identifier, number of collected samples, timestamp and location of data collection. |
| Data preparation outputs | O (NOTE) | Provides outputs for data preparation: dataset identifier, data preparation report for each feature, and error list. |
| Exploratory data analysis outputs | O (NOTE) | Provides outputs for exploratory data analysis: dataset identifier, statistical outputs for each feature, outlier and anomaly detection report, and feature correlation information. |
| Timestamp | O | Timestamp of the data management operation |
| NOTE: At least one of the information elements shall be provided in the output. |  |  |

#### 8.x.3.2	Client data management subscription

Table 8.x.3.2-1 shows the request sent by the AIMLE server to AIMLE clients for the AIMLE client data management subscription procedure.

Table 8.x.3.2-1: Client data management subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Data management type | M | An indicator showing what data management type is being requested: data collection, data preparation, exploratory data analysis. |
| Data management requirements | M | Requirements for the data management request: |
| >Data collection requirements | O (NOTE) | Data collection requirements as detailed in Table 8.x.3.1-2. |
| >Data preparation requirements | O (NOTE) | Data collection requirements as detailed in Table 8.x.3.1-3. |
| >Exploratory data analysis requirements | O (NOTE) | Data collection requirements as detailed in Table 8.x.3.1-4. |
| Operational schedule | O | A schedule to perform the requested data management operation. |
| NOTE: At least one of the information elements shall be provided. |  |  |

Table 8.x.3.2-2 shows the response sent by AIMLE clients to the AIMLE server for the AIMLE client data management subscription procedure.

Table 8.x.3.2-2: Client data management subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the data management operation |
| Subscription identifier | M | An identifier for the subscription. |
| Expiration time | O | Expiration time for the subscription |

Table 8.x.3.2-3 shows the notification sent by AIMLE clients to the AIMLE server for the Client data management subscription procedure.

Table 8.x.3.2-3: Client data management notify

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the data management operation |
| Data collection outputs | O (NOTE) | Provides outputs for data collection: dataset identifier, number of collected samples, timestamp and location of data collection. |
| Data preparation outputs | O (NOTE) | Provides outputs for data preparation: dataset identifier, data preparation report for each feature, and error list. |
| Exploratory data analysis outputs | O (NOTE) | Provides outputs for exploratory data analysis: dataset identifier, statistical outputs for each feature, outlier and anomaly detection report, and feature correlation information. |
| Timestamp | O | Timestamp of the data management operation |
| NOTE: At least one of the information elements shall be provided in the output. |  |  |

* * * End of Changes * * * *