3GPP TSG-SA WG6 Meeting #63	S6-244072

Hyderabad, India, 14th – 18th October 2024

Source:	InterDigital Inc.

Title:	Pseudo-CR on Support for spatial mapping service exposure

Spec:	3GPP TS 23.437 v0.2.0

Agenda item:	9.11

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This pCR enhances spatial map discovery when a spatial map can be provided by a VAL server based on Solution #16.

2. Reason for Change

The following has been concluded in TR 23.700-21.

…

Solution #16 resolves open issue 1) and 2) by providing spatial map exposure and discovery service when a VAL server provides a spatial mapping service. Solution #16 is based on architecture option #1 and can be adapted to other proposed architectures options. The following aspects of Solution #16 can be considered for normative work:

1.	The VAL server can inform the MMES of the VAL server’s spatial map coverage (Solution #16, clause 7.16.1.2).

2.	The MMES can inform the consumer about the VAL server when the MMES does not have coverage for a requested area (e.g., solution #8).

3.	Reuse of CAPIF for the publish and discovery of the VAL server’s spatial map services will be considered in normative work.

…

Solution #8, Solution #16 and Solution #17 are complementary and can be combined in the normative phase.

NOTE: The aspects of Solution #16 considered for normative phase are described in clause 10.2.4.

…

Key issue #4 conclusions:

iv.	for Key issue #4 (Spatial mapping): Solution #8 , Solution #15, Solution #16 and Solution #17 can be considered for the normative work by defining a new SEAL server for spatial map management with necessary changes to adapt to the new SEAL server.

3. Conclusions

NA

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 v0.2.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.156: "Mobile Metaverse Services; Stage 1".

[3]	3GPP TR 22.856: "Feasibility Study on Localized Mobile Metaverse Services".

[4]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[5]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[r23222]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *

### 9.3.2	Discover spatial map

Figure 9.3.2-1 depicts the procedure for an authorized spatial map consumer (VAL server or SM client) to discover spatial map(s) or VAL server with SM capabilities.

Pre-conditions:

1.	The VAL server or SM client has received information (e.g. URI, IP address) related to the SM server;

2.	The VAL server or SM client has received security credentials authorizing it to communicate with the SM server.

Figure 9.3.2-1: Discover spatial map procedure

1.	The VAL server or SM client sends a SM discovery request message to the SM server to discover spatial map(s). The request includes a requestor ID, security credentials, three-dimensional area of interest or localization information to discover spatial maps in the area of interest and optionally discovery filters as described in clause 9.3.2.1.1.

NOTE 1:	How the consumer identifies which three-dimensional area of interest to get the spatial map is not in the scope of this document.

2.	The SM server authorizes the requestor and validates the request. If the request is authorized, the SM server then determines one or more spatial maps according to the requested area of interest and discovery filters. If the request is authorized and if the SM server does not determine any spatial map, the SM server can determine if any VAL server(s) registered with the SM server can offer spatial map(s) according to the requested area of interest and discovery filters.

3.	The SM server sends a SM discovery response message to the requestor. If the SM server has determined at least one spatial map or VAL server with SM capabilities registered with the SM server, the response includes an indication of success and may include spatial map ID(s) or information about VAL server(s) with SM capabilities. If the SM server cannot determine a spatial map or VAL server(s) with SM capabilities, the SM server sends a failure indication with a reason for failure.

NOTE 2:	A requestor can obtain spatial map(s) from a VAL server with SM capabilities.

NOTE 3:	A requestor can obtain VAL server APIs via CAPIF as specified in 3GPP TS 23.222 [r23222] if a CAPIF core function is deployed within the SM server to support publish and discovery of VAL server APIs.

#### 9.3.2.1	Information flows

##### 9.3.2.1.1	SM discovery request

Table 9.3.2.1.1-1 describes information elements of the SM discovery request sent from the VAL server or SM client to the SM server.

Table 9.3.2.1.1-1: SM discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., SM client, UE, VAL client, VAL server) |
| Requestor security credentials | M | The security credentials of the requestor |
| Area of interest | M | Three-dimensional area of interest of the requested spatial map. |
| Discovery filters | O | Characteristics of the requested spatial map. |
| > Location | O | The location of a spatial map (e.g., civic address, etc.) |
| > Update time | O | The time when the spatial map was last updated |
| > Update frequency | O | The frequency at which a spatial map is updated |

##### 9.3.2.1.2	SM discovery response

Table 9.3.2.1.2-1 describes information elements of the SM discovery response sent from the SM server to the VAL server or SM client.

Table 9.3.2.1.2-1: SM discovery response

| Information element | Status | Description |  |
| --- | --- | --- | --- |
| Successful response | O | Indicates that the request was successful |  |
| > Spatial map IDs (NOTE) | O | The identifiers if the discovered spatial map(s) |  |
| > VAL server profiles (NOTE) | O | The list of VAL server profiles that can offer a spatial map as described in Table 9.3.x.3.1-2. |  |
| Failure response | O | Indicates that the request failed |  |
| > Cause | O | Indicates the failure cause |  |
| NOTE: Only one IE is included. |  |  |  |

* * * Next Change * * * *

### 9.3.x	VAL server spatial map exposure

#### 9.3.x.1	General

A VAL server can complement the spatial mapping coverage of the SM server by providing spatial maps for areas of interest that are not available at the SM server. The SM server can inform a requestor discovering a spatial map about VAL servers registered with the SM server as described in clause 9.3.2.

A VAL server offering spatial maps can register a SM service with the SM server to provide information about the coverage area of the offered spatial maps. A VAL server can expose the SM service APIs via CAPIF as specified in 3GPP TS 23.222 [r23222] if a CAPIF core function is deployed within the SM server to support publish and discovery of VAL server APIs.

The following procedures are supported for VAL server spatial map exposure:

-	SM service registration request-response procedure;

-	SM service registration update request-response procedure;

-	SM service deregistration request-response procedure.

#### 9.3.x.2	Procedures

##### 9.3.x.2.1	SM service registration

Figure 9.3.x.2.1-1 depicts the procedure for an authorized VAL server with SM capabilities to register a SM service with the SM server.

Pre-conditions:

1.	The VAL server has received information (e.g. URI, IP address) related to the SM server;

2.	The VAL server has received security credentials authorizing it to communicate with the SM server.

Figure 9.3.x.2.1-1: SM service registration procedure

1.	The VAL server sends a SM service registration request to the SM server. The request includes a requestor ID, security credentials, and a SM service profile including SM service information (e.g., endpoint, availability period, availability location) and spatial map information (e.g., identifier, coverage area, civic address, latest update time).

2.	Upon receiving the request, the SM server validates if the requestor is authorized for the request. If the requestor is authorized, the SM server assigns a unique identifier for the registration and stores the SM service profile that was provided in the request.

3.	The SM server sends a SM service registration response to the VAL server. If the SM server successfully stored the SM service profile, the response includes an indication of success, the registration identifier and may include a proposed expiration time for the registration. Otherwise, the response includes an indication of failure and may include a reason for failure.

##### 9.3.x.2.2	SM service registration update

Figure 9.3.x.2.2-1 depicts the procedure for an authorized VAL server to update SM service information.

Pre-conditions:

1.	The VAL server has already registered the SM service with the SM server.

Figure 9.3.x.2.2-1: SM service registration update procedure

1.	The VAL server sends a SM service registration update request to the SM server. The request includes a requestor ID, security credentials, a registration ID, and may include an updated SM service profile and proposed expiration time for the updated registration.

2.	Upon receiving the request, the SM server validates if the requestor is authorized for the request. If the requestor is authorized, the SM server updates the SM service profile based on the request information.

3.	The SM server sends the SM service registration update response to the VAL server. If the SM server successfully updated the SM service profile, the response includes an indication of success and may include a proposed expiration time for the registration. Otherwise, the response includes an indication of failure and may include a reason for failure.

##### 9.3.x.2.3	SM Service deregistration

Figure 9.3.x.2.3-1 depicts the procedure for an authorized VAL server to deregister a SM service.

Pre-conditions:

1.	The VAL server has already registered the SM service with the SM server.

Figure 9.3.x.2.3-1: SM service deregistration procedure

1.	The VAL server sends a SM service deregistration request to the SM server. The request includes a requestor ID, security credentials, and a registration ID.

2.	Upon receiving the request, the SM server validates if the requestor is authorized for the request. If the requestor is authorized, the SM server ends the SM service registration and removes the stored SM service profile.

3.	The SM server sends the SM service deregistration response to the VAL server. If the SM server successfully deregistered the SM service, the response includes an indication of success. Otherwise, the response includes an indication of failure and may include a reason for failure.

#### 9.3.x.3	Information flows

##### 9.3.x.3.1	SM service registration request

Table 9.3.x.3.1-1 describes information elements in the SM service registration request from the VAL server to the SM server.

Table 9.3.x.3.1-1: SM service registration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the requestor (e.g., VAL server) |
| Security credentials | M | The security credentials of the requestor |
| Proposed expiration time | O | Proposed expiration time for the registration |
| SM service profile | O | The SM service profile as described in Table 9.3.x.3.1-2. |

Table 9.3.x.3.1-2 describes information elements in the SM service profile.

Table 9.3.x.3.1-2: SM Service Profile

| Information element | Status | Description |
| --- | --- | --- |
| SM service information | M | The information about the SM service; each element includes the information described below. |
| > Identifier | M | The identifier of the SM service. |
| > Endpoint | M | The endpoint information (e.g. URI, FQDN, IP address) used to communicate with the SM Service. |
| > Availability period | O | The time period when a service consumer can access the SM service. |
| > Availability location | O | The location where a service consumer can access the service from. |
| Spatial map information | M | The information about spatial map(s) available at the SM service; each spatial map of the list includes the information described below. |
| > Identifier | M | The identifier of a spatial map |
| > Coverage area | M | The spatial coverage area of a spatial map |
| > Location | O | The location of a spatial map (e.g., civic address, etc.) |
| > Update time | O | The time when the spatial map was last updated |
| > Update frequency | O | The frequency at which a spatial map is updated |

##### 9.3.x.3.2	SM service registration response

Table 9.3.x.3.2-1 describes information elements in the SM service registration response from the SM server to the VAL server.

Table 9.3.x.3.2-1: SM service registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the registration request was successful |
| > Registration ID | M | Identifier of the registration |
| > Expiration time | O | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the registration never expires. |
| Failure response | O | Indicates that the request failed |
| > Cause | O | Indicates the cause of the failure |

##### 9.3.x.3.3	SM service registration update request

Table 9.3.x.3.3-1 describes information elements in the SM service registration update request from the VAL server to the SM server.

Table 9.3.x.3.3-1: SM service registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the requestor (e.g., VAL server) |
| Security credentials | M | The security credentials of the requestor |
| Registration ID | M | Identifier of the registration |
| Proposed expiration time (NOTE) | O | Proposed expiration time for the registration |
| SM service profile (NOTE) | O | The SM service profile as described in Table 9.3.x.3.1-2. |
| NOTE: At least one of the IEs is included. |  |  |

##### 9.3.x.3.4	SM service registration update response

Table 9.3.x.3.4-1 describes information elements in the SM service registration update response from the SM server to the VAL server.

Table 9.3.x.3.4-1: SM service registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the registration update request was successful |
| > Expiration time | O | Indicates the expiration time of the updated registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the updated registration never expires. |
| Failure response | O | Indicates that the request failed |
| > Cause | O | Indicates the cause of the failure |

##### 9.3.x.3.5	SM service deregistration request

Table 9.3.x.3.5-1 describes information elements in the SM service deregistration request from the VAL server to the SM server.

Table 9.3.x.3.5-1: SM service deregistration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the requestor (e.g., VAL server) |
| Requestor security credentials | M | The security credentials of the requestor |
| Registration ID | M | Identifier of the registration |

##### 9.3.x.3.6	SM service deregistration response

Table 9.3.x.3.6-1 describes information elements in the SM service deregistration response sent from the SM server to the VAL server.

Table 9.3.x.3.6-1: SM service deregistration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the deregistration request was successful |
| Failure response | O | Indicates that the request failed |
| > Cause | O | Indicates the cause of the failure |

* * * End of Changes * * * *