3GPP TSG-SA WG6 Meeting #63	S6-244082

Hyderabad, India, 14th – 18th October 2024	(revision of S6-24xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0131 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MCVideo push enhancement via ECN marking for L4S |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR introduces a new procedure related to enhancing the MCvideo push service. Based on 3GPP TS 23.501, the ECN marking for L4S can be activated for both uplink and downlink to expose network congestion information. This CR focuses on the uplink, similar to the downlink, it can also face congestion, and this CR includes exposing network congestion information to the media sender (in the uplink case it is the MCVideo UE) and respectively update the data rate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. update the L4S ECN feedback report to include feedback to the uplink as well. 2. add a new procedure to enhance push service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unutilized 5GS capability in exposing network congestion information to the MCVideo UE sending media. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.2 and 7.7.3.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 7.7.2	Information flows

#### 7.7.2.1	L4S ECN feedback report

Table 7.7.2.1-1 defines the L4S ECN feedback report from MCVideo client to MCVideo server to provide DL L4S ECN feedback information, and from the MCVideo server to the MCVideo client to provide UL L4S feedback information.

Table 7.7.2.1-1: L4S ECN feedback report

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the MCVideo user |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| L4S ECN information | M | The L4S ECN information reported |

# * * * * Second change * * * *

#### 7.7.3.X	One-to-server- video push enhancement

Figure 7.7.3.X-1 describes a common signalling control plane procedure for MC video client initiating the establishment of video push where the push feature is enhanced via ECN marking for L4S capability.

Precondition:

-	The MCVideo server has already obtained the L4S handling capability from the MCVideo client 1.

Figure 7.7.3.X-1: One-to-server video push with ECN marking for L4S capability

1.-5.	Same as steps 1 to step 5 of 3GPP TS 23.281 [4] clause 7.4.2.4.2.

6.	The MCVideo server may determine to utilize the 5GS L4S capability to obtain the congestion information to improve the UL stream transmission control. For this purpose, the MCVideo server interacts with the 5GS to request the ECN marking for L4S as specified in 3GPP TS 23.502 [10].

7.	The 5GS starts to perform the ECN marking as specified in 3GPP TS 23.501 [7]. The 5GS receives the UL media stream with ECN marking in the IP header which reflects the UL network congestion status. The 5GS exposes the congestion information to the MCVideo server as specified in 3GPP TS 23.501 [7].

8.	Based on the UL ECN marking in the IP header, the MCVideo server generates and sends the L4S feedback report towards the MCVideo client as described in IETF RFC 9330 [24].

9.	Based on the L4S feedback report, the MCVideo performs UL stream control, e.g., adjust the UL media packet sending rate.

NOTE:	How the UL media packet sending rate is adapted is outside the scope of the present document.

10.-11.		Step 10 and step 11 are same as step 6 and step 7 f clause 7.4.7.4.2 of 3GPP TS 23.281 [4], respectively.

# * * * * End of changes * * * *