3GPP TSG-SA WG6 Meeting #63	S6-244122

Hyderabad, India, 14th – 18th October 2024	(revision of S6-243236)

Source:	Convida Wireless LLC

Title:	Digital assets management

Spec:	3GPP TS 23.438 V0.0.0

Agenda item:	9.11

Document for:	Approval

Contact:	Ly.Quang@convidawireless.com

1. Introduction

This contribution adds digital assets management procedures to TS 23.438.

2. Reason for Change

Digital assets play an important role in metaverse services as it is a digital representation that is shown to metaverse users. Key issue #3 in TR 23.700-21 identifies the need to support digital assets and the conclusion for KI #3 states that solution #6 can be considered for normative work. This contribution updates S6-243236 to focus on the management of digital assets individually as a first step.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.438 V0.0.0.

* * * First Change * * * *

## 8.X	Digital Assets Management

### 8.X.1	General

The following clauses specify procedures, information flows, and APIs for digital asset management.

### 8.X.2	Procedures

#### 8.X.2.1	Digital Asset Create

Figure 8.x.2.1-1: Digital Asset Create

- A requestor (e.g. A-DACM client) sends a digital asset create request to the A-DACM server. The request includes information listed in Table 8.x.3.1-1. A digital asset specific profile, e.g. a digital avatar profile, is provided in the request to provide further information about the digital asset.

2.	The A-DACM server authenticates and authorizes the request. If authorized, the A-DACM server creates the digital asset and stores the information about the digital asset in a local repository.

Based on the information received from the requestor and from local policies, the A-DACM optionally determines to send one or more requests to the underlying 3GPP network to determine information available in the 3GPP network for the digital asset (e.g. user identifier).

3.	The A-DACM server generates and sends a response to the requestor. The response includes information listed in Table 8.x.3.1-2.

#### 8.X.2.2	Digital Asset Retrieve

Figure 8.x.2.2-1: Digital Asset Retrieve

1.	A requestor (e.g. A-DACM client) sends a digital asset retrieve request to the A-DACM server. The request includes information listed in Table 8.x.3.2-1.

2.	The A-DACM server authenticates and authorizes the request. If authorized, the A-DACM server retrieves the digital asset from the local repository.

3.	The A-DACM server generates and sends a response to the requestor. The response includes information listed in Table 8.x.3.2-2.

#### 8.X.2.3	Digital Asset Update

Figure 8.x.2.3-1: Digital Asset Update

1.	A requestor (e.g. A-DACM client) sends a digital asset update request to the A-DACM server. The request includes information listed in Table 8.x.3.3-1.

2.	The A-DACM server authenticates and authorizes the request. If authorized, the A-DACM server updates information about the digital asset in the local repository.

3.	The A-DACM server generates and sends a response to the requestor. The response includes information listed in Table 8.x.3.3-2.

#### 8.X.2.4	Digital Asset Delete

Figure 8.x.2.4-1: Digital Asset Delete

1.	A requestor (e.g. A-DACM client) sends a digital asset delete request to the A-DACM server. The request includes information listed in Table 8.x.3.4-1.

2.	The A-DACM server authenticates and authorizes the request. If authorized, the A-DACM server deletes the digital asset from the local repository.

3.	The A-DACM server generates and sends a response to the requestor. The response includes information listed in Table 8.x.3.4-2.

#### 8.X.2.5	Digital Asset Subscribe

Figure 8.x.2.5-1: Digital Asset Subscribe

1.	A requestor (e.g. A-DACM client) sends a digital asset subscribe request to the A-DACM server. The request includes information listed in Table 8.x.3.5-1.

2.	The A-DACM server authenticates and authorizes the request. If authorized, the A-DACM server creates a subscription for the digital asset in the local repository and assigns an identifier for the subscription.

3.	The A-DACM server generates and sends a response to the requestor. The response includes information listed in Table 8.x.3.5-2.

#### 8.X.2.6	Digital Asset Unsubscribe

Figure 8.x.2.6-1: Digital Asset Unsubscribe

1.	A requestor (e.g. A-DACM client) sends a digital asset unsubscribe request to the A-DACM server. The request includes information listed in Table 8.x.3.6-1.

2.	The A-DACM server authenticates and authorizes the request. If authorized, the A-DACM server removes the subscription for the digital asset.

3.	The A-DACM server generates and sends a response to the requestor. The response includes information listed in Table 8.x.3.6-2.

#### 8.X.2.7	Digital Asset Notify

Figure 8.x.2.7-1: Digital Asset Notify

1.	An event occurs that is associated with a subscription event for a digital asset. The A-DACM server retrieves the information pertaining to the subscription event for the digital asset and generates a notification for the subscription.

2.	The A-DACM server sends a notification to the requestor. The notification includes information listed in Table 8.x.3.7-1.

3.	The requestor (e.g. A-DACM client) acknowledges the notification and include the information listed in Table 8.x.3.7-2.

### 8.X.3	Information flows

#### 8.X.3.1	Digital Asset Create

Table 8.x.3.1-1 describes the information elements for a digital asset create request sent from a requestor (e.g. A-DACM client) to a A-DACM server.

Table 8.x.3.1-1: Digital asset create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Digital asset owner identifier | M | An identifier for the digital asset owner. |
| > Subscriber/User identifier | M | Subscriber and/or user identifier to associate with the digital asset. |
| Digital asset identifier | M | An identifier for the digital asset. |
| > Digital asset type | M | The type of digital asset, e.g. avatar, wallet, etc. |
| > Asset type-specific profile | M | Profile information specific to the digital asset, e.g. an avatar profile. |
| > Digital asset object | M | Information about the digital asset that is used to render the digital asset, e.g. information to render an avatar. |
| >> Communication parameters | O | Communication parameters and requirements for sessions associated with the digital asset. The parameters are also used to create or derive the communication policies applicable to the asset which are persistent within metaverse. |
| >> Metaverse experience requirements | O | Information provided to create or derive associated metaverse experience parameters for each asset, e.g. information used to derive applicable rendering parameters |
| > List of Metaverse service provider identifiers | O | List of service providers for the digital asset. |

Editor’s Note: The association of identifiers for digital assets are to be determined.

Editor’s Note: The association of a subscriber/user identifier for the digital asset is pending completion of SA2 work.

Table 8.x.3.1-2 describes the information elements for the digital asset create response sent from a A-DACM server to a requestor (e.g. A-DACM client).

Table 8.x.3.1-2: Digital asset create response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | A status for the request: success, fail. If the status is fail, a failure code may be provided. |
| Digital asset identifier | O | The identifier for the digital asset. |

#### 8.X.3.2	Digital Asset Retrieve

Table 8.x.3.2-1 describes the information elements for the digital asset retrieve request sent from a requestor (e.g. A-DACM client) to a A-DACM server.

Table 8.x.3.2-1: Digital asset retrieve request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Digital asset identifier | M | An identifier for the digital asset. |

Table 8.x.3.2-2 describes the information elements for the digital asset retrieve response sent from a A-DACM server to a requestor (e.g. A-DACM client).

Table 8.x.3.2-2: Digital asset retrieve response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | A status for the request: success, fail. If the status is fail, a failure code may be provided. |
| Digital asset identifier | M | The identifier for the digital asset and all the sub information elements for the digital asset. |
| > Digital asset information |  | Information of the digital asset. |

#### 8.X.3.3	Digital Asset Update

Table 8.x.3.3-1 describes the information elements for the digital asset update request sent from a requestor (e.g. A-DACM client) to a A-DACM server.

Table 8.x.3.3-1: Digital asset update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Digital asset identifier | M | An identifier for the digital asset. |
| > Digital asset information | M | Information to update the digital asset. |

Table 8.x.3.3-2 describes the information elements for the digital asset update response sent from a A-DACM server to a requestor (e.g. A-DACM client).

Table 8.x.3.3-2: Digital asset update response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | A status for the request: success, fail. If the status is fail, a failure code may be provided. |

#### 8.X.3.4	Digital Asset Delete

Table 8.x.3.4-1 describes the information elements for the digital asset delete request sent from a requestor (e.g. A-DACM client) to a A-DACM server.

Table 8.x.3.4-1: Digital asset delete request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Digital asset identifier | M | An identifier for the digital asset. |

Table 8.x.3.4-2 describes the information elements for the digital asset delete response sent from a A-DACM server to a requestor (e.g. A-DACM client).

Table 8.x.3.4-2: Digital asset delete response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | A status for the request: success, fail. If the status is fail, a failure code may be provided. |

#### 8.X.3.5	Digital Asset Subscribe

Table 8.x.3.5-1 describes the information elements for the digital asset subscribe request sent from a requestor (e.g. A-DACM client) to a A-DACM server.

Table 8.x.3.5-1: Digital asset subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Digital asset subscription criteria | M | Criteria for the subscription of the digital asset. |

Table 8.x.3.5-2 describes the information elements for the digital asset subscribe response sent from a A-DACM server to a requestor (e.g. A-DACM client).

Table 8.x.3.5-2: Digital asset subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | A status for the request: success, fail. If the status is fail, a failure code may be provided. |
| Subscription identifier | M | An identifier for the subscription. |

#### 8.X.3.6	Digital Asset Unsubscribe

Table 8.x.3.6-1 describes the information elements for the digital asset unsubscribe request sent from a requestor (e.g. A-DACM client) to a A-DACM server.

Table 8.x.3.6-1: Digital asset unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Subscription identifier | M | An identifier for the digital asset subscription. |

Table 8.x.3.6-2 describes the information elements for the digital asset unsubscribe response sent from a A-DACM server to a requestor (e.g. A-DACM client).

Table 8.x.3.6-2: Digital asset unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | A status for the request: success, fail. If the status is fail, a failure code may be provided. |

#### 8.X.3.7	Digital Asset Notify

Table 8.x.3.7-1 describes the information elements for the digital asset notify sent from a A-DACM server to a requestor (e.g. A-DACM client).

Table 8.x.3.7-1: Digital asset notify

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Subscription identifier | M | An identifier for the digital asset subscription. |
| Digital asset identifier | M | An identifier for the digital asset. |
| > Digital asset information | M | Information about the digital asset that trigger the notification. |

Table 8.x.3.7-2 describes the information elements for the digital asset notify acknowledge sent from the requestor (e.g. A-DACM client) to the A-DACM server.

Table 8.x.3.7-2: Digital asset notify acknowledge

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | A status for the request: success, fail. If the status is fail, a failure code may be provided. |

### 8.X.4	APIs

* * * End of Change * * * *