3GPP TSG-SA WG6 Meeting #63	S6-244208

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on DC application and profile deletion

Spec:	3GPP TS 23.392 v0.1.0

Agenda item:	9.12

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

This pCR proposes to add the DC application and profile delete procedure.

2. Reason for Change

With the conclusion of FS_eMMTelAPP for KI#7 and KI#2, the MMTel Enabler layer is responsible for the DC application management, the corresponding normative work need to be implementd.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.392 v0.1.0.

* * * First Change * * * *

### 8.2.3	DC application and profile deletion

#### 8.2.3.1	General

DC application and profile deletion enables a Controlling Application Server to delete DC application and profile with the MMTel Enabler Server.

#### 8.2.3.2	Procedure

Figure 8.2.3.2-1 illustrates the DC application and profile deletion procedure.

Pre-conditions:

1.	The Controlling Application Server has connected to the serving network successfully.

2.	Both the Controlling Application Server and MMTel Enabler Server have been configured with the necessary credentials to enable authenticating one another.

Figure 8.2.3.2-1: DC application and profile deletion

1.	The Controlling Application Server sends a DC Application and Profile delete request to the MMTel Enabler Server. The request message includes information elements as specified in Clause 8.2.3.3.1.

2.	Upon receiving the request, the MMTel Enabler Server initiates authentication procedures with the Requester Identity and authorizes the Controlling Application Server if needed. If successful, the MMTel Enabler Server requests the DCAR to delete the DC Application and Profile. The northbound interfaces for DC Application management on DCAR is implementation-based.

3.	The MMTel Enabler Server sends a DC Application and Profile delete response to the Controlling Application Server. The response message includes information elements as specified in Clause 8.2.3.3.2.

#### 8.2.3.3	Information flows

##### 8.2.3.3.1	DC Application and profile delete request

Table 8.2.3.3.1-1 describes information elements for the DC Application and profile delete request from the Controlling Application Server to the MMTel Enabler server.

Table 8.2.3.3.1-1: Information elements in DC Application and Profile delete request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| Security credentials | O | Security information required by the MMTel Enabler Server. |
| List of Application ID | M | List of identifiers of the DC application to be deleted. |

##### 8.2.3.3.2	DC Application and profile delete response

Table 8.2.3.3.2-1 describes information elements for the DC Application and profile delete response from the MMTel Enabler Server.

Table 8.2.3.3.2-1: Information elements for DC Application and Profile delete response

| Information element | Status | Description |
| --- | --- | --- |
| DC Application and profile delete response list | M | A list of Application and profile delete response. The detailed information is listed in Table 8.2.3.3.2-2. |

Table 8.2.3.3.2-2: Information elements in DC Application and profile delete response list

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates the deletion of the DC Application and Profile was successful |
| Failure response | O | Indicates the deletion of the DC Application and Profile was failed |
| > Cause | O | Indicates the cause of failure |

* * * End of Changes * * * *